3GPP TSG-RAN WG4 Meeting # 113							R4-2419366

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.21.2.2

Source: 	OPPO

Title: 	on co-existence evaluations

Document for:	Approval

- Introduction

In RAN4#112-bis meeting, the co-existence evaluations have been fully carried out. Many of the scenarios have been discussed. In this paper we give further discussion based on the simulation results.

- Discussion

For the D2T2 scenario, simulation for case 5-1 and 5-2 have been carried out. In the agreed WF [1], it has captured as below table with the simulation conclusion.

Table 1 Simulation results and conclusion

| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |
|  |  |  | g | Reader->NR UL | TBA |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | TBA |
|  |  |  | g | Reader->NR UL | FFS: NR UL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range Interference coordination/mitigation solution can be considered. |
|  |  |  | h | NR UL->device | TBA |

Hence for deployment 5-1, case e has been concluded. For case f and h, the outage percentage needs to be further discussed while for case g still companies simulation results are not converged.

Based on the previous meeting agreement, rapporteur will provide low, mid and high SNR values for D2R and R2D to derive the outage percentage. As captured below table 2.

Table 2 SNR for device 1 and 2a for coherent and non-coherent

|  | D2T2 D2R（coherent） | D2T2 D2R（non-coherent） | R2D SNR for device 1 | R2D SNR for device 2a |
| --- | --- | --- | --- | --- |
| min | -7.6 | -9 | -14.8 | -14.1 |
| max | -5 | 2.1 | 9.3 | 5.1 |
| average | -6 | -3.2 | -6.5 | -7.3 |

As summarized above, for case f and h, the outage percentage needs to be further discussed. Below figure 1 and figure 2 are CDF from the simulation results.

Figure 1 CDF of case 5-1-f for reader

Figure 2 CDF of case 5-1-h for device

The outage percentage is listed below in table 2:

Table 2 Outage percentage for case f

|  | D2T2 D2R（coherent）SNR | D2T2 D2R（coherent）CDF | D2T2 D2R（non-coherent）SNR | D2T2 D2R（non-coherent）CDF | R2D SNR for device 1 | R2D CDF for device 1 |
| --- | --- | --- | --- | --- | --- | --- |
| min | -7.6 | 0.1 | -9 | 0.1 | -14.8 | 0 |
| max | -5 | 0.1 | 2.1 | 2.5 | 9.3 | 2 |
| average | -6 | 0.1 | -3.2 | 0.1 | -6.5 | 0 |

It can be observed from table 2 that for average SNR for D2R both outage percentage is only 0.1% and for R2D the percentage is 0.

In such case we believe the outage percentage is small enough to conclude the 5-1-g and 5-1-h with no interference.

Observation 1: For average SNR for D2R both outage percentage is only 0.1% for coherent and non-coherent.

Observation 2: For average SNR for R2D the outage percentage is 0.

For deployment 5-2, case e also has been concluded. For case f and h as all legacy UE indoor, there should be large interference for the UE UL transmission to the device and reader RX. Hence some further interference coordination and mitigation mechanism should be needed.

3	Conclusions

In this paper, we give further discussion on A-IoT co-existence study, the observations and proposals are as below:

Observation 1: For average SNR for D2R both outage percentage is only 0.1% for coherent and non-coherent.

Observation 2: For average SNR for R2D the outage percentage is 0.

4 References

[1] R4-2417196	WF on Co-existence study for A-IoT and NR/LTE, CMCC