3GPP TSG-RAN WG4 Meeting # 113	R4-2419375

# Orlando, Florida, USA, 18th – 22nd November, 2024

Source:			T-Mobile USA, US Cellular, Verizon, AT&T and Deutsche Telekom

Title:			NTN S-Band UE-UE coexistence

Agenda item:		6.17.2

Document for:		Discussion

# Introduction

At the RAN4#112bis, a Way Forward was approved for the NTN Work Items. The Way Forward included the following:

#### Issue 1-1-2: General Methodology for UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL

  - Way Forward:

    - Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence with a more relaxed value compared to -50dBm/MHz, that allows the NTN service to work effectively, across the full n252 band, whilst still protecting the TN UE at least in the majority of cases

      - Further discuss the actual value and methodology for the requirement (e.g. corresponding separation assumption)

      - Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations

      - Further discuss how to capture the solutions in the specification

This contribution discusses the views of T-Mobile USA, US Cellular, Verizon, AT&T and Deutsche Telekom on TN-NTN UE-to-UE Coexistence.

# Discussion

## Background

At RAN4#112, a contribution from T-Mobile USA discussed NTN S-Band coexistence, and how A-MPR is likely unacceptable for a satellite network due to link budget concerns, and propose uplink configuration restrictions as an alternative to NS signalling and A-MPR to ensure coexistence [4].

At RAN4#112bis, a contribution from T-Mobile USA, AT&T and US Cellular discussed NTN S-Band coexistence, and proposed that in realistic deployment scenarios, UEs will likely be closer together than random drops in a given geographic area would suggest [5].

Also at RAN4#112bis, a contribution from Apple presented A-MPR needed to meet -50 dBm/MHz below 1995 MHz for 5 MHz and 20 MHz n252 channel bandwidths [6].

## A-MPR for -50 dBm/MHz below 1995 MHz

The A-MPR study in [6] showed that with 20 MHz uplink channel bandwidth, even for very small numbers of PRBs, significant levels of A-MPR are required to meet -50 dBm/MHz below 1995 MHz, both for RBstart below 30 and above 80.

Observation 1: Significant amounts of A-MPR would be needed even for small numbers of RBs in order to meet -50 dBm/MHz below 1995 MHz.

The A-MPR study in [6] also showed that with 5 MHz uplink channel bandwidth, for small numbers of PRBs, not much A-MPR is required to meet -50 dBm/MHz regardless of the position of the 5 MH channel in the n252 uplink.

Observation 2: With 5 MHz channel bandwidths, most small RB allocations would require no A-MPR in order to meet -50 dBm/MHz below 1995 MHz, regardless of the position of the 5 MHz carrier in the n252 uplink.

## SEM protection below 1995 MHz

If we look at the Spectrum Emission Mask we can see how much protection is provided by the SEM alone. For a 20 MHz channel bandwidth in band n252, the SEM would only guarantee -13 dBm/MHz from 1980-1995 MHz.

Observation 3: For a 20 MHz channel bandwidth in band n252, the SEM would only guarantee -13 dBm/MHz from 1980-1995 MHz.

For a 10 MHz channel BW at the low edge of band n252, the SEM would only guarantee -13 dBm/MHz from 1990-1995 MHz.

Observation 4: For a 10 MHz channel BW at the low edge of band n252, the SEM would only guarantee -13 dBm/MHz from 1990-1995 MHz.

For a 5 MHz channel BW at the low edge of band n252, the SEM would guarantee -25 dBm/MHz from below 1994 MHz.

Observation 5: For a 5 MHz channel BW at the low edge of band n252, the SEM would guarantee -25 dBm/MHz from below 1994 MHz.

Observation 6 : Limiting the uplink channel bandwidth to 5 MHz guarantees 12 dB better protection for 1980-1994 MHz compared to a 20 MHz uplink channel bandwidth.

## New NS Signalling and A-MPR

Since one of the proposals in the way forward was

- Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations

We would like to see proposals from vendors on what level of protection is reasonable with what levels of A-MPR.

## Proposals

Because we are sceptical if NS signalling would ever be used in an NTN network, even if new NS signalling and A-MPR is defined we would like to propose the following to provide a reasonable level of protection for services in Band2/n2/B25/n25 without requiring excessive A-MPR on the n252 uplink:

Proposal 1: NR band n252 should be defined with only a 5 MHz uplink channel BW, and 5, 10, 15 and 20 MHz channel bandwidth on the downlink.

Proposal 2: NR band n252 should be defined with an asymmetric channel bandwidth set of 5 MHz uplink channel BW paired with 5, 10, 15 and 20 MHz channel bandwidth on the downlink.

# Summary

Observation 1: Significant amounts of A-MPR would be needed even for small numbers of RBs in order to meet -50 dBm/MHz below 1995 MHz.

Observation 2: With 5 MHz channel bandwidths, most small RB allocations would require no A-MPR in order to meet -50 dBm/MHz below 1995 MHz, regardless of the position of the 5 MHz carrier in the n252 uplink.

Observation 3: For a 20 MHz channel bandwidth in band n252, the SEM would only guarantee -13 dBm/MHz from 1980-1995 MHz.

Observation 4: For a 10 MHz channel BW at the low edge of band n252, the SEM would only guarantee -13 dBm/MHz from 1990-1995 MHz.

Observation 5: For a 5 MHz channel BW at the low edge of band n252, the SEM would guarantee -25 dBm/MHz from below 1994 MHz.

Proposal 1: NR band n252 should be defined with only a 5 MHz uplink channel BW, and 5, 10, 15 and 20 MHz channel bandwidth on the downlink.

Proposal 2: NR band n252 should be defined with an asymmetric channel bandwidth set of 5 MHz uplink channel BW paired with 5, 10, 15 and 20 MHz channel bandwidth on the downlink.

# References

- R4-2414287, “WF on Rel-19 FR1 NTN bands,” Moderator (Inmarsat)

- R4-080710, “TS36.101: TP for UE Spurious emission limits,” Motorola

- TS 25.942 v1.0.0, “RF System Scenarios”

- R4-2413305, “NTN S-Band coexistence,” T-Mobile USA

- R4-2416507, “NTN S-Band UE-UE coexistence with TN,” T-Mobile USA, AT&T, US Cellular

- R4-2415314, Initial view on the UE power back-off while operating in 2000-2020 MHz UL range, Apple