3GPP TSG-RAN WG4 Meeting #113	R4-2419041

# Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on 8Rx performance requirements with MMSE-IRC

Source: 	Huawei, HiSilicon

Agenda item:	7.16.2

Document for:	Discussion

Background

In last meeting, WF R4-2416555 [1] was approved, one objective is related to 8Rx requirements under interference scenario, this contribution provides our views.

- Discussions

General

In last meeting, it was agreed to cover baseline 8Rx MMSE-IRC receiver and simplified 8Rx MMSE-IRC receiver in Rel-19 8Rx performance tests. However, how to define the requirements for such two receiver types are FFS:

8Rx UE MMSE-IRC receiver coverage Agreement: - Cover both Baseline 8Rx UE MMSE-IRC receiver and simplified 8Rx UE MMSE-IRC Receiver for the initial simulation analysis. - TBD: Criteria used to decide if one set of requirements can be introduced covering both receiver types. - Note: Test coverage shall cover both receivers

It is common understanding that number of sets of requirements to be defined depends on performance difference between such two receivers. According to our results shown in Appendix, the performance difference under inter cell interference is about 2.3dB for rank1, 3.3dB~3.7dB for Rank2 and 6.6dB for Rank4, the performance difference under MU-MIMO scenario is about 2.0dB~3.4dB. It can be observed that such significant difference makes it unreasonable to apply one requirement to both receiver with additional margin. Hence, we propose to define two sets of requirements for both receivers.

Observation 1: The gap between baseline receiver and simplified receiver is significant for all cases.

Proposal 1: RAN4 to define two sets of requirements for baseline 8Rx receiver and simplified 8Rx receiver.

PDSCH requirements with inter cell interference

Scenarios

In last meeting, following agreements were made:

Interference profile and power levels Agreement: - Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx. - TBD: Further discuss applicability rules to potentially reduce UE effort to only comply to either 1 or 2 interference cells.

My understanding is that the agreement means that interference profiles will be reused if corresponding scenario is agreed. (i.e. HomNet or Hetnet) but still FFS for scenarios to be defined. Firstly, we should discuss the scenario(s). Based on our understanding, HomNet scenario is worse case with higher interference power level compared to HetNet scenario. So we don’t think that if a UE which has passed the severe test still needs to pass the optimistic test since UE baseband behaviour will not be changed.

Proposal 2: RAN4 to only consider HomNet scenario.

One key issue is Rank and MCS, in last meeting, we had following candidates for evaluation:

MCS of serving cell Agreement: Evaluate the following configurations for initial analysis: - Rank 1: MCS17 - Rank 2: MCS17 - Rank 4: MCS13 In case one of the above configurations is found not feasible consider alternative MCS. Interrested companies can provide simulations results with: - Rank 1: MCS19 - Rank 2: MCS13 / MCS19

Rank

Rank highly impacts tested SNR points and performance difference between baseline receiver and simplified receiver. Rank1,2 and 4 are candidate options. It should be mentioned that R17 2Rx requirements for inter-cell interference with HomNet scenario is 15.4dB, which could be a reference when judging the reasonability of 8Rx test SNR points.

Table 5.2.2.1.15-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | 15.4 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 12.5 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

To make target SNR points reasonable, with lower operating SNR points for 8Rx, higher rank should be selected. Based on our simulation results, given rank4 with MCS13, target SNR is 8.6dB for baseline receiver and 14.2dB for simplified receiver, which is suitable for cell edge scenarios. Some companies question about the feasibility of rank4+MCS13, it is mentioned that such configurations have been used for R15 PDSCH requirements definition (Table 5.2.3.1.1-6 in 38.101-4). Hence, we propose to choose Rank4+MCS13.  Furthermore, interference estimation processing with Rank 4 is more complicate than rank2. For Rank4, number of DMRS CDM groups without data is 2, i.e. two CDM groups are allocated with DMRS, which means two CDM groups can be used for interference estimation. However, for rank1/2, the number of CDM groups is 1, which means only one CDM group can be used for interference estimation. Therefore, introducing rank4 can comprehensively verify the interference estimation algorithm by utilizing all potential CDM groups.

Observation 2: Legacy PDSCH tests with ICI only have one number of DMRS CDM group without data configured, which cause the limitation that only one CDM group can be used for interference estimation. However, Rank4 allows UE to estimate interference utilizing two CDM groups.

Proposal 3: RAN4 to choose Rank4 with MCS13.

Propagation conditions:

Considering we have proposed to choose Rank4, we prefer to use TDLA30-10, which is more suitable for high rank.

Proposal 4: RAN4 to use TDLA30-10.

CQI requirements with inter cell interference

Rank

One key issue is rank for serving cell. Rank1,2 and 4 are candidate options:

Rank for the serving cell Proposals - Option 1: Rank 1 - Option 2: Rank 2 - Option 3: Rank 4 - Option 4: Cover the following for initial study and possible down select based on simulation results: - Rank 1, 2 & 4 - Other options not precluded.

Firstly, we prefer to preclude Rank1, which is not typical configuration for 8Rx as denoted before. We did some evaluation for both Rank 2 and Rank 4 with baseline receiver. It can be observed that for Rank 4, performance gain of interference model over white noise model is quite limited at low SINR points, for Rank2, the gain is sufficient for all SINR points. Hence, we think Rank2 is more reasonable for 8Rx CQI requirements.

Figure 2-1: Simulation results for CQI tests with comparison of rank2 and rank4.

Observation 3: For Rank4, at low SINR, the gamma value (Performance gain of interference model over white noise model) is quite limited.

Proposal 5: RAN4 to consider Rank2 for CQI requirements with inter cell interference.

CQI calculation algorithm

Same as demodulation processing, there could be two different CQI calculation assumptions:

- Baseline CQI calculation: UE perform CQI calculation with 8Rx joint processing

- Simplified CQI calculation: UE perform CQI calculation with separate 4Rx processing

Based on our understanding, CQI calculation algorithm should be consistent with demodulation algorithm, otherwise, performance degradation may occur. For example, if demodulation is based on 8Rx joint processing but CSI calculation is based on two separate 4Rx joint processing, CQI derived by CSI-RS estimation may be underestimated and absolute throughput may be lower than it should be. Therefore, CQI calculation algorithm is expected to match to demodulation algorithm. We did some simulations for both two receiver types: Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing. It’s observed that in most SNR points γfor baseline receiver is higher than simplified receiver. The reason is that baseline receiver has higher interference suppression capability, given the same total power of interference and white noise, baseline receiver can cancel more interference. Therefore, we propose to define CSI requirements for such two receiver types.

Figure 2-2: CQI test results for baseline receiver and simplified receiver.

Proposal 6: RAN4 to define CQI requirements for two receiver types:

Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing.

Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing.

CSI-IM and NZP CSI-RS configurations

One issue is CSI-IM and NZP CSI-RS configuration:

Proposals - Option 1: Reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation: - CSI-IM on serving cell overlapping with PDSCH from interference cell - NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell - Other options not precluded

Option 1 is reusing test setups from R17 2/4Rx inter-cell interference suppressing requirements for simulation.  However, we find some issues for 2/4Rx test setups: For FDD, CSI-IM on serving cell is overlapped with ZP-CSI-RS from interference cell:

Table 2-1: CSI-RS configuration for FDD for R17 2Rx/4Rx test (Table 6.2.2.1.2.3-1 in 38.101-4)

| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
| --- | --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 |
|  | CSI-RS periodicity and offset | slot | 5/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 | 1 |
|  | CDM Type |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | Same as serving cell |

The CSI-RS position for FDD is shown in figure 2-3:

Figure 2-3: CSI-RS configurations for both serving cell and interference cell for FDD

For TDD, the configurations are different with FDD but complies with the common understanding that CSI-IM on serving cell overlapping with PDSCH from interference cell, NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell:

Table 2-2: CSI-RS configuration for TDD for R17 2Rx/4Rx test

| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
| --- | --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 2 | 1 |
|  | CDM Type |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3(8) | Row 2(8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 |
|  | CSI-RS periodicity and offset | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 | 1 |
|  | CDM Type |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 | Same as serving cell |

The CSI-RS position for TDD is shown in figure 2-4:

Figure 2-4: CSI-RS configurations for both serving cell and interference cell for TDD

- Observation 4：For R17 FDD 2Rx/4Rx CQI test with interference, CSI-IM on serving cell is overlapped with ZP-CSI-RS on interference cell, which will cause performance degradation. And TDD has the correct CSI-RS configuration which is different from FDD.

Hence, it’s proposed to re-discuss CSI-RS configuration for 8Rx test.

Proposal 7: RAN4 shall not reuse R17 test setup of 2Rx/4Rx FDD CQI test with interference and discuss the new test setup for 8Rx.

PDSCH requirements under MU-MIMO scenario

The left issues for PDSCH requirements under MU-MIMO scenario are rank, MCS and propagation conditions. In last meeting, RAN4 agreed some initial cases for simulation:

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation Agreement: For initial simulation analysis: - Rank 1+1 for 2T8R ULA Low - Rank 2+2 for 4T8R ULA Low - Rank 4+4 for 8T8R ULA Low - Note: Will require DMRS len=2, hence optional feature, which means if introduced requirements would also be needed for one mandatory configuration (i.e. either 1+1 or 2+2) - Note: TBD: Down-selection of configurations. MCS Agreement: Evaluate: - Rank 1+1: MCS13 - Rank 2+2: MCS13 - Rank 4+4: MCS13 as starting point. Interrested companies can provide results for - Rank 1+1: MCS17 - Rank 2+2: MCS19

Rank and MCS

Following candidate options for evaluation were made in last meeting:

MCS of serving cell Agreement: Evaluate the following configurations for initial analysis: - Rank 1: MCS17 - Rank 2: MCS17 - Rank 4: MCS13 In case one of the above configurations is found not feasible consider alternative MCS. Interrested companies can provide simulations results with: - Rank 1: MCS19 - Rank 2: MCS13 / MCS19

My preference is only considering Rank 2+2.  For Rank4+4, 70% of max throughput is unachievable for simplified receiver based on our simulation results in clause 4, which can’t accommodate the agreements that test coverage should cover simplified receiver. Furthermore, we don’t think Rank1+1 is enough to verify the interference cancellation capability of 8Rx UE,  which support up to 4 or 8 layers, it is a reasonable configuration for 2Rx UE, but too optimistic for 8Rx UE and the target SNR is quite low. Hence we think Rank 2+2 is a good choice for both receiver and usually occur in the real deployment. If UE passes the test with higher interference level, i.e. Rank2+2, UE doesn’t need to pass the test with lower interference level, i.e. Rank 1+1.

Observation 5: It has been agreed that test coverage should cover simplified receiver, however, for case with Rank 4+4, 70% of max throughput for simplified receiver is unachievable.

Proposal 8: RAN4 to only consider Rank 2+2.

We did simulations for both MCS13 and MCS19, it can be observed that for MCS19, 70% of max throughput is unachievable for MMSE-MRC for both baseline receiver and simplified receiver which means MMSE-IRC can be better verified compared to MCS13. Furthermore, MCS13 has been selected for 2Rx/4Rx requirements definition in Rel-17 MMSE-IRC WI, higher MCS is more reasonable for 8Rx considering 8Rx UE has higher baseband processing capability. Hence MCS19 should be selected.

Proposal 9: RAN4 to choose MCS19 for PDSCH requirements under MU-MIMO scenario.

Applicability rule and release independent

Test applicability

We have following options for test applicability rules:

Proposals - Option 1: The new requirements for 8Rx MMSE-IRC receiver shall be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined. - Option 4: - The new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs - If agreed to define two set of requirements (baseline and simplified receiver), introduce new applicability rules based on Table 5.1.1.3-1 of TS38.101-4. - Option 5: Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed regardless the concrete test setup. - Other options not precluded.

Table 5.1.1.3-1 (Requirements applicability for optional UE features) should be updated to capture the applicability rules that which requirement is applicable for which 8 Rx receiver. The other aspect is how to handle 2Rx/4Rx tests if 8Rx UE has passed 8Rx tests. We prefer that if 8Rx UE passes 8Rx test, 2Rx/4Rx test can be skipped regardless the test configurations, since Ruu calculation is main step of MMSE-IRC receiver and 8x8 Ruu calculation is much complicate than 4x4 or 2x2, UE doesn’t need to pass test cases corresponding to simper implementation if has passed cases corresponding to complicate implementation.

Proposal 10: The new applicability rules are needed to specify that which requirement is applicable for which 8 Rx receiver, meanwhile, introduce the applicability rules that if UE passes 8Rx tests, 2Rx/4Rx tests shall be skipped.

Release independency

We have following options for release independency:

Release independency Proposals - Option 1: The new requirements for 8Rx should be release independent from Rel-18 - Option 2: The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced. - Option 3: Postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics. - Option 4: Discuss release independence requirements at a later stage in the WI. - Other options not precluded.

Noted that supporting 8Rx is introduced in Rel-18 but release independent from Rel-17. Considering there is short period between Rel-18 and Rel-19 and MMSE-IRC receiver is basic feature which is always be assumed as baseline receiver for demodulation requirements. It’s reasonable to define the new requirements release independent from Rel-18. To be conservative, the requirements could be optional for Rel-17.

Proposal 11:  The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Conclusion

- In this paper, we provide our views on 8Rx requirements with interference. The proposals are:

General:

Observation 1: The gap between baseline receiver and simplified receiver is significant for all cases.

Proposal 1: RAN4 to define two sets of requirements for baseline 8Rx receiver and simplified 8Rx receiver.

PDSCH requirements with inter cell interference:

Proposal 2: RAN4 to only consider HomNet scenario.

Observation 2: Legacy PDSCH tests with ICI only have one number of DMRS CDM group without data configured, which cause the limitation that only one CDM group can be used for interference estimation. However, Rank4 allows UE to estimate interference utilizing two CDM groups.

Proposal 3: RAN4 to choose Rank4 with MCS13.

Proposal 4: RAN4 to use TDLA30-10.

CQI requirements with inter cell interference

Observation 3: For Rank4, at low SINR, the gama value (Performance gain of interference model over white noise model) is quite limited.

Proposal 5: RAN4 to consider Rank2 for CQI requirements with inter cell interference.

Proposal 6: RAN4 to define CQI requirements for two receiver types:

Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing.

Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing.

- Observation 4：For R17 FDD 2Rx/4Rx CQI test with interference, CSI-IM on serving cell is overlapped with ZP-CSI-RS on interference cell, which will cause performance degradation. And FDD has different CSI-RS configuration with TDD.

Proposal 7: RAN4 shall not reuse R17 test setup of 2Rx/4Rx CQI test with interference and discuss the new test setup for 8Rx.

PDSCH requirements with MU-MIMO

Observation 5: It has been agreed that test coverage should cover simplified receiver, however, for case with Rank 4+4, 70% of max throughput for simplified receiver is unachievable.

Proposal 8: RAN4 to only consider Rank 2+2.

Proposal 9: RAN4 to choose MCS19 for PDSCH requirements under MU-MIMO scenario.

Applicability rules:

Observation 6: Both 8Rx baseline SU-MIMO receiver and 8Rx simplified SU-MIMO receiver are optional features which means the new applicability rules are needed when applying MMSE-IRC receiver requirements to such both receivers.

Proposal 10: The new applicability rules are needed to specify that which requirement is applicable for which optional 8 Rx receiver, meanwhile, introduce the applicability rules that if UE passes 8Rx tests, 2Rx/4Rx tests shall be skipped.

Proposal 11:  The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Appendix

Table 4-1 and Figure 4-1 provides simulation results for 8Rx with inter-cell interference.

- Table 4-1: Summary of simulation results for 8Rx with inter-cell interference (HomoNet)

| Propagation conditions | Rank | MCS |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance difference (Baseline receiver vs Simplified receiver) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| TDLC300-100 | Rank1 | MCS17 |  | 5.5 | 9.8 | 4.3 | 7.8 | 11.3 | 3.5 | 2.3 |
|  |  | MCS19 |  | 7.2 | 11.4 | 4.2 | 9.5 | 12.7 | 3.2 | 2.3 |
| TDLA30-10 | Rank2 |  | MCS13 | 4.2 | 9.3 | 5.1 | 7.5 | 11.0 | 3.5 | 3.3 |
|  |  |  | MCS17 | 7.3 | 12.1 | 4.8 | 10.8 | 14.2 | 3.4 | 3.5 |
|  |  |  | MCS19 | 8.9 | 13.8 | 4.9 | 12.6 | 15.9 | 3.3 | 3.7 |
|  | Rank4 |  | MCS13 | 8.6 | 13.7 | 5.1 | 15.2 | 17.2 | 2.0 | 6.6 |

- Table 4-1: Summary of simulation results for 8Rx with inter-cell interference (HetNet)

| Propagation conditions | Rank | MCS |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance difference (Baseline receiver vs Simplified receiver) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| TDLA30-10 | Rank1 | MCS17 |  | 3.6 | 7.1 | 3.5 | 5.3 | 8.5 | 3.2 | 1.7 |
|  |  | MCS19 |  | 5.2 | 8.6 | 3.4 | 6.7 | 10.0 | 3.3 | 1.5 |
|  | Rank2 |  | MCS13 | 3.4 | 8.0 | 4.6 | 5.6 | 9.7 | 4.1 | 2.2 |
|  |  |  | MCS17 | 6.4 | 10.7 | 4.3 | 8.9 | 12.8 | 3.9 | 2.5 |
|  |  |  | MCS19 | 8.2 | 12.3 | 4.1 | 10.6 | 14.5 | 3.9 | 2.4 |
|  | Rank4 |  | MCS13 | 7.8 | 11.9 | 4.1 | 13.4 | 15.9 | 2.5 | 5.6 |

Table 4-2 and Figure 4-2 provides simulation results for 8Rx with MU-MIMO.

- Table 4-2: Summary of simulation results for 8Rx with MU-MIMO

|  |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance loss |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| Rank1+1 | MCS13 | 4.5 | 9.0 | 4.5 | 5.5 | 10.3 | 4.8 | 1.0 |
|  | MCS17 | 7.4 | 21.5 | 14.1 | 8.7 | 23.7 | 15.0 | 1.3 |
| Rank2+2 | MCS13 | 6.0 | 11.7 | 5.7 | 8.9 | 18.4 | 6.5 | 2.9 |
|  | MCS19 | 10.8 | Inf | Inf | 15.2 | Inf | Inf | 4.4 |
| Rank4+4 | MCS13 | 12.4 | Inf | Inf | Inf | Inf | Inf | Inf |

Figure 4-2: Simulation results for PDSCH requirements with MU-MIMO

- Reference

[1]  R4-2416555 WF on NR demodulation performance: Phase 5.