3GPP TSG-RAN WG4 Meeting #113															R4-2419415

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.2.4.1

Source:	Samsung

Title:	Co-existence simulation results for 14800 – 15350 MHz

Document for:	Discussion

1 Introduction

Under the Study Item to study the IMT parameters relevant for sharing and compatibility for the frequency ranges based on the LS from ITU-R WP 5D [1, 2], for 15 GHz frequency range, RAN4 agreed to conduct coexistence studies to derive interference levels on adjacent channels.

Although it is a new frequency range to study and still has some multiple options for future discussion, a significant progress was made for coexistence study trying for a single simulation package as captured in TR 38.922 [3]. In this contribution, we would like to provide additional simulation results based on the updated simulation assumption focusing on the first priority parameters in the view of ITU-R sharing study impact.

2 Discussion

As mentioned earlier, simulation assumptions for coexistence study have been updated and captured in [3]. Table 1 shows a summary of the updated simulation scenarios for 14800 - 15350 MHz, which are basic assumptions considered for the coexistence simulation during the SI.

Table 1: Summary of initial simulation scenarios for 14800 - 15350 MHz [3]

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100/200 MHz | Same as aggressor | DL to DL | 15 GHz | Indoor hotspot | Second |
| 2 | eMBB | NR, 100/200 MHz | Same as aggressor | DL to DL | 15 GHz | Urban macro | First |
| 3 | eMBB | NR, 100/200 MHz | Same as aggressor | DL to DL | 15 GHz | Dense urban | Third |
| 4 | eMBB | NR, 100/200 MHz | Same as aggressor | UL to UL | 15 GHz | Indoor hotspot | Second |
| 5 | eMBB | NR, 100/200 MHz | Same as aggressor | UL to UL | 15 GHz | Urban macro | First |
| 6 | eMBB | NR, 100/200 MHz | Same as aggressor | UL to UL | 15 GHz | Dense urban | Third |

On top of the simulation scenarios, other important assumptions were also confirmed to avoid any unnecessary workload with reduced scenarios and assumptions as much as possible. Table 2 below tries to summarize changed assumptions from the last meeting to compare the simulation results with what we have done for the range of 14800 - 15350MHz.

Table 2: Summary of parameter changes

|  | For RAN4#112-bis | For RAN4#113 |
| --- | --- | --- |
| Bandwidth | 100/200/400 | [100]/200 |
| ISD | 450 m (first priority) 350 m (second priority) | 450 m |
| UE Ant | FR1-like w/o beamforming FR2-like w/ beamforming | FR1-like w/o beamforming |
| BS Ant | Uma: 16 x 24 x 4 / 8 x 32 x 8 InH: 8 x 8 / 4 x 4 | Uma: 16 x 24 x 4 InH: 4 x 4 |
| UE power | 23dBm (first priority) 26dBm (second priority) | 23dBm |
| UE noise figure | 11dB | 13dB |

In accordance with the update assumption above, in this contribution, we have further conducted our simulation work for both urban macro and indoor scenarios, and summarized the results for each network layout model and channel bandwidth as below.

2.1	Urban macro

2.1.1	100MHz

Table 5: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  | Un-coordinated (100% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 4% | 2% | 1% | 0% | 0% | 0% | 4% | 3% | 2% | 1% | 0% | 0% |
| 5%-tile | 30% | 17% | 10% | 5% | 4% | 2% | 34% | 21% | 12% | 8% | 4% | 3% |

| <0% grid shift> | <100% grid shift> |
| --- | --- |

Figure 1: Simulation results for DL throughput loss (%)

2.1.3	200MHz

Table 7: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  | Un-coordinated (100% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 4% | 2% | 1% | 1% | 0% | 0% | 4% | 3% | 2% | 1% | 0% | 0% |
| 5%-tile | 28% | 16% | 9% | 5% | 3% | 2% | 34% | 20% | 12% | 7% | 3% | 2% |

| <0% grid shift> | <100% grid shift> |
| --- | --- |

Figure 3: Simulation results for DL throughput loss (%)

Based on the tables and figures for the simulation results of urban macro scenarios above, it is observed that the throughput loss is comparable enough for both average and edge cases as expected. It is our understanding that such performance would come from the massive antenna pattern assumptions that should be enough at 15 GHz frequency range. Therefore, corresponding ACIR requirements should be determined for urban macro scenarios.

Observation 1:	It is observed that the throughput loss is comparable enough for both average and edge cases as expected.

Proposal 1:		Corresponding ACIR requirements should be determined for urban macro scenarios.

2.2	Indoor hotspot

2.2.2	100MHz

Table 10: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 20% | 5% | 0% | 0% | 0% | 0% |
| 5%-tile | 27% | 10% | 0% | 0% | 0% | 0% |

Figure 6: Simulation results for DL throughput loss (%)

2.2.4	200MHz

Table 12: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 12% | 0% | 0% | 0% | 0% | 0% |
| 5%-tile | 16% | 0% | 0% | 0% | 0% | 0% |

Figure 1: Simulation results for DL throughput loss (%)

3	Conclusion

This contribution provides our simulation results of DL-to-DL co-existence studies for 15 GHz frequency range based on the assumptions in [3]. Following observations and proposals should be considered by this meeting.

Observation 1:	It is observed that the throughput loss is comparable enough for both average and edge cases as expected.

Proposal 1:		Corresponding ACIR requirements should be determined for urban macro scenarios.

Reference

[1] R4-2400333	Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency
 				bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

[2]	RP-241326	Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

[3]	3GPP TR 38.922 V0.2.0	(2024-08)