3GPP TSG-RAN WG4 Meeting #113															R4-2419416

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.2.4.2

Source:	Samsung

Title:	Views on BS and UE parameters for 14800 - 15350 MHz

Document for:	Discussion

1 Introduction

Based on the LS from ITU-R WP 5D [1], a Study Item was approved in RAN#103 to task RAN4 to study the IMT parameters relevant for sharing and compatibility for the frequency ranges, and to send LS reply to ITU-R WP 5D directly (cc to RAN) [2].

In spite of the challenging schedule of the SI to complete a set of IMT parameters for each of three different frequency ranges as below, RAN4 successfully sent two reply LSs on time based on companies’ understanding of the importance [3, 4]:

- 4 400-4 800 MHz	Estimated date for completion: May 2024 (RAN WG4#111)

- 7 125-8 400 MHz	Estimated date for completion: August 2024 (RAN WG4#112)

- 14.8-15.35 GHz	Estimated date for completion: November 2024 (RAN WG4#113)

In RAN4#113, thanks to [3, 4], RAN4 is able to focus on the completion of 15 GHz parameters based on the options and agreements until the last meeting. In this contribution, we would like to provide our additional views on overall IMT parameters for both BS and UE based on our initial study and the updated options from the last meeting to finalize parameters of the new frequency range of 14800 to 15350 MHz.

2 Discussion

As a background, at its earlier stage of the 15 GHz discussion, it was agreed to conduct coexistence studies in the new frequency range to derive appropriate IMT parameters based on the study outcome. In addition, on top of the parameters for simulation purpose, RAN4 has discussed them for the LS purpose separately, which both achieved big progress as captured in the chairman note and two WFs, i.e., 15 GHz BS and UE parameters [5, 6]. Based on the two WFs, this contribution would start with BS and system part, followed by UE part in this clause.

2.1 BS and System parameters

For BS and system parameters, some remaining issues should be resolved in this meeting, e.g., channel bandwidth, emission levels, which are extracted from the WF as below [5].

Regarding channel bandwidth, as captured in the chairman note below, RAN4 tentatively agreed to have 200 MHz as typical bandwidth for 15 GHz among the options of 100/200/400 MHz for the LS purpose.

Issue 4-5: Typical channel bandwidth Agreement: - [200MHz] as typical bandwidth, document in TR other bandwidths not precluded - The note should capture the association of channel bandwidth with maximum transmission power /PSD - If the practical issues are identified or additional work required for replying LS for 200MHz, RAN4 will consider 100MHz as typical bandwidth

In our view, given the legacy maximum channel bandwidth of FR1 and FR2, 200 MHz could be adopted for the frequency range in-between FR1 and FR2, which also enables to enjoy a larger contiguous bandwidth on the higher frequency range than FR1. At the same time, considering the legacy services around the range, e.g., fixed and passive satellite services, military uses, etc., available channel bandwidth for single carrier would be limited considering larger channel bandwidth than 200 MHz compared to FR2. In this context, we prefer to confirm 200 MHz as a nominal channel bandwidth for the frequency range around 15 GHz. We could see no other issue/limitation such as PSD for output powers given the existing requirement concept at this moment.

Observation 1:	As maximum channel bandwidth for 15 GHz, 200 MHz could enable to enjoy a larger contiguous bandwidth on the higher frequency range than FR1.

Observation 2: At the same time, larger than 200 MHz channel bandwidth would be limited considering the legacy services around the frequency range of 15 GHz.

Proposal 1:		RAN4 should confirm 200 MHz as typical channel bandwidth for 15 GHz.

Meanwhile, other remining issues than channel bandwidth would be to considered as deciding whether to capture the category B for both emission mask and spurious emission levels as below:

Issue 4-10: Emissions mask Agreement: For the reply, refer to the following table as basis - [category B] and category A - Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category A Frequency offset of measurement filter 3dB point, Df; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz £ Df < 50 MHz; 0.05 MHz £ f_offset < 50.05 MHz; -7dBm-750foffsetMHz-0.05; 100 kHz. 50 MHz f < min(100 MHz, Dfmax); 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax); -14 dBm; 100 kHz. 100 MHz £ Df £ Dfmax; 100.5 MHz £ f_offset < f_offsetmax; -13 dBm; 1MHz. - [Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category B Frequency offset of measurement filter 3dB point, Df; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz £ Df < 50 MHz; 0.05 MHz £ f_offset < 50.05 MHz; -7dBm-750foffsetMHz-0.05; 100 kHz. 50 MHz f < min(100 MHz, Dfmax); 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax); -14 dBm; 100 kHz. 100 MHz £ Df £ Dfmax; 100.5 MHz £ f_offset < f_offsetmax; -15 dBm; 1MHz. ] Issue 4-14: Spurious emission Agreement: For the reply, refer to the following table as basis - [category B] and category A - Table 2.2.2.3-1: BS spurious emission limits for 14800 to 15350 MHz for Category A Frequency range; Limit; Measurement Bandwidth; Note. 30 MHz – 1 GHz; -13 dBm; 100 kHz; Note 1. 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band; 1 MHz; Note 1, Note 2. NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1.. - Table 2.2.2.3-2: BS spurious emission limits for 14800 to 15350 MHz for Category B Spurious frequency range; Limit; Measurement bandwidth. 30 MHz – 1 GHz; -36 dBm; 100 kHz. 1 GHz – 18 GHz; -30 dBm; 1 MHz. 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band; -20 dBm; 10 MHz.

In our understanding, such emission requirements in 3GPP specs, e.g., TS 38.104, are based on the regulatory requirements to protect adjacent bands or services from the frequency range. The different categories defined in BS RF specs also have the same principle, which are category A and B. Category A applies in general and specifies relaxed limits compared to category B, which normally applies for Europe. However, for the study of new frequency range during SI phase, we do not see any background regulation, or information from ITU-R for their study to have different categories as RAN4 did before. Before WRC-27 where to confirm the frequency ranges with their requirements, category B for both emission mask and spurious emission for the new frequency range would be meaningless, in our view, and would make confusion in our future discussion as RAN4 has been doing some revision work after WRC-19/23. It is preferred to remove category B at this stage unless otherwise requested from ITU-R.

Observation 3:	Category B for both emission mask and spurious emission for the new frequency range would be meaningless before WRC-27 where to confirm the frequency ranges with their requirements.

Proposal 2:		It is preferred to remove category B unless otherwise requested from ITU-R.

2.2 UE parameters and antenna characteristics

For UE parameters, the remaining issues in the WF can be considered to decide whether to reuse legacy requirements as much as possible, e.g., emission mask, spurious emission, blocking and spurious response as below [6]:

Emissions mask - Option 1: UE SEM given in Section 7.1.2, TR 38.921. - Option 2: Other options. Spurious emission - FFS if RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1 Blocking and spurious response - FFS if RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

Since the parameters above are for the reply LS, and also RAN4 has agreed to FR1-like UE type at the last meeting already, we believe that there is no outstanding issue even if the existing requirements in TS 38.101-1 or TR 38.921 are taken for the 15 GHz range. Given the timeline of the reply and SI, it would be good to remove FFS for those UE parameters above.

Observation 4:	Since RAN4 has agreed to FR1-like UE type already, there is no outstanding issue even if the existing requirements in TS 38.101-1 or TR 38.921 are taken for the 15 GHz range.

Proposal 3:		It would be good to remove FFS for UE parameters of emission mask, spurious emission, blocking and spurious response.

Regarding UE antenna type discussion, as discussed in the last meeting, FR1-like antenna was agreed for the reply LS as below:

UE-type discussion - LS - Only one UE antenna model to be captured. - 1Tx omnidirectional antenna UE, understood as the worst case (highest ACIR values) assumptions for sharing studies and not on deployment, coverage, implementation, or requirement aspects. - TR - Additional UE antenna models could be captured.

As we also have provided our understanding of FR2-like UE type for 15 GHz in past meetings, it would not make sense for handheld UE to expect the similar uplink and downlink performances when compared with FR2, considering the estimated antenna array size and limited number of antenna modules/panels. Therefore, it is good for RAN4 to have 1Tx omnidirectional as only one UE antenna model for the LS from the perspective of both feasibility and ITU-R sharing studies.

Observation 5:	As agreed in the last meeting, it is good for RAN4 to have 1Tx omnidirectional as only one UE antenna model for the LS from the perspective of both feasibility and ITU-R sharing studies.

Moreover, even for FR1-like UE type with omnidirectional antennas, not a few FR1-like antenna model has been discussed to derive better antenna gain/performance for the simulation and reference for future discussions. One of the ideas was to assume multi-Tx based antenna to achieve better performances in terms of antenna gains, e.g., 5dBi, at the higher frequency range than FR1.

Theoretically, at the 15 GHz band, the higher number of antennas than currently used for FR1 could be implemented taking into account the smaller antenna size or correlation/isolation. However, it should be noted that increasing the number of antennas does not always guarantee the better performance given the more loss at higher frequency ranges. For example, additional antennas will result in the increased coupling among antennas, leading to a degradation in MIMO performance. Also, it would be not easy to optimize all the antenna feeding lines to ensure the better performance due to the higher insertion loss per unit length than FR1. In that sense, the idea of beam steering with two discrete single element patch antennas might not also be so easy in terms of its feasibility due to the higher insertion loss between two separate patches for spherical coverage.

So, in order to figure out the impact with the FR1-like UE antennas mentioned earlier, we have conducted a simulation to estimate anticipated antenna gain with two general metal frame Tx antennas without beam steering assumptions. The antenna placement for the evaluation is captured in Figure 1.

Figure 1: Exemplary placement of three metal frame antennas within a mobile device.

With those assumptions, which are now general FR1-like UE antennas on the market, we confirmed that the multi-Tx approach is not always better than single-Tx in terms of antenna gains as expected. Table 1 and Figure 2 show peak antenna gains of each Tx scenario for cases of single-beam and combined-beam, which are not about the best beam based on the beam steering assumption.

Table 1: Estimated antenna gains for one-Tx and two-Tx

|  | Ant1 | Ant2 | Ant3 |
| --- | --- | --- | --- |
| Gain | 3.9 | 4.8 | 2.7 |
|  | Ant1 + Ant2 | Ant1 + Ant 3 | Ant2 + Ant3 |
| Combined gain | 3.8 | 2.6 | 3.9 |
| Gain drop | -1.0 | -1.3 | -0.9 |

Figure 2: Estimated antenna gains for one-Tx and two-Tx

As we can see from Table 1 and Figure 2, from the antenna gain perspective at least, the peak gain of combined two Tx antennas does not show better performance, but rather drops compared to the single beam case. In our view, it can be explained by two different radiation patterns as depicted in Figure 3.

Figure 3: Simulated radiation patterns for one-Tx and two-Tx cases

Based on simulation results with normal FR1-like antennas above, it should be noted that the benefit from multi-Tx is not necessarily guaranteed at 15GHz. It would be meaningless for RAN4 to discuss the number of necessary Tx antennas for the better antenna gain at this frequency range.

Observation 6:	From the antenna gain perspective, the peak gain of combined two Tx antennas does not show better performance, but rather drops compared to the single beam case with normal FR1-like antennas.

Proposal 4:		It would be meaningless for RAN4 to discuss the number of necessary Tx antennas for the better antenna gain at this frequency range.

2.3 Proposed parameters

As presented above, Table 2 summarizes our preference for IMT parameters at 15 GHz frequency range for both BS and UE based on the options and agreements so far. Some parameters are marked FFS which might need coexistence results. For antenna array parameters, the LS can have the same table with the simulation assumptions in the TR.

Table 2: IMT technology related parameters in 14800 – 15350 MHz

|  |  | IMT |  |  |
| --- | --- | --- | --- | --- |
| No. | Parameter | Base station (non-AAS) (Note 1) | Base station (AAS) | Mobile station |
| 1 | Duplex Method | TDD |  |  |
| 2 | Channel bandwidth (MHz) | 200 MHz typical |  |  |
| 3 | Signal bandwidth (MHz) | Signal bandwidth = NRB × SCS × 12 |  |  |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB |  | 56 dB typical |
| 4.2 | Spectral mask (dB) | Table 6.1.2-1 in TR 38.921 |  | FFS |
| 4.3 | ACLR | FFS |  | FFS |
| 4.4 | Spurious emissions | Table 6.1.4-4 in TR 38.921 |  | FFS |
| 4.5 | Maximum output power | For wide area BS, 46dBm for TRP for dual polarization; For dense urban BS, [36dBm] for TRP for dual polarization; For Indoor: 23dBm per polarization |  | 23 dBm |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | 8 dB (Wide Area BS) 13 dB (Medium Range BS) 16 dB (Local Area BS) |  | 11 dB |
| 5.2 | Sensitivity (dBm) | To be specified |  | To be specified |
| 5.3 | Blocking response | FFS |  | FFS |
| 5.4 | ACS | FFS |  | FFS |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |  |

Proposal 5:		For antenna array parameters, the LS can have the same table with the simulation assumptions in the TR.

3	Conclusion

This contribution provides following observations and proposals to be considered by this meeting. Table 2 shall also be taken into account for the final LS to ITU-R WP 5D.

<BS and system parameters>

Observation 1:	As maximum channel bandwidth for 15 GHz, 200 MHz could enable to enjoy a larger contiguous bandwidth on the higher frequency range than FR1.

Observation 2: At the same time, larger than 200 MHz channel bandwidth would be limited considering the legacy services around the frequency range of 15 GHz.

Proposal 1:		RAN4 should confirm 200 MHz as typical channel bandwidth for 15 GHz.

Observation 3:	Category B for both emission mask and spurious emission for the new frequency range would be meaningless before WRC-27 where to confirm the frequency ranges with their requirements.

Proposal 2:		It is preferred to remove category B unless otherwise requested from ITU-R.

<UE antenna parameters>

Observation 4:	Since RAN4 has agreed to FR1-like UE type already, there is no outstanding issue even if the existing requirements in TS 38.101-1 or TR 38.921 are taken for the 15 GHz range.

Proposal 3:		It would be good to remove FFS for UE parameters of emission mask, spurious emission, blocking and spurious response.

Observation 5:	As agreed in the last meeting, it is good for RAN4 to have 1Tx omnidirectional as only one UE antenna model for the LS from the perspective of both feasibility and ITU-R sharing studies.

Observation 6:	From the antenna gain perspective, the peak gain of combined two Tx antennas does not show better performance, but rather drops compared to the single beam case with normal FR1-like antennas.

Proposal 4:		It would be meaningless for RAN4 to discuss the number of necessary Tx antennas for the better antenna gain at this frequency range.

<Proposed parameters>

Table 2: IMT technology related parameters in 14800 – 15350 MHz

|  |  | IMT |  |  |
| --- | --- | --- | --- | --- |
| No. | Parameter | Base station (non-AAS) (Note 1) | Base station (AAS) | Mobile station |
| 1 | Duplex Method | TDD |  |  |
| 2 | Channel bandwidth (MHz) | 200 MHz typical |  |  |
| 3 | Signal bandwidth (MHz) | Signal bandwidth = NRB × SCS × 12 |  |  |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB |  | 56 dB typical |
| 4.2 | Spectral mask (dB) | Table 6.1.2-1 in TR 38.921 |  | FFS |
| 4.3 | ACLR | FFS |  | FFS |
| 4.4 | Spurious emissions | Table 6.1.4-4 in TR 38.921 |  | FFS |
| 4.5 | Maximum output power | For wide area BS, 46dBm for TRP for dual polarization; For dense urban BS, [36dBm] for TRP for dual polarization; For Indoor: 23dBm per polarization |  | 23 dBm |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | 8 dB (Wide Area BS) 13 dB (Medium Range BS) 16 dB (Local Area BS) |  | 11 dB |
| 5.2 | Sensitivity (dBm) | To be specified |  | To be specified |
| 5.3 | Blocking response | FFS |  | FFS |
| 5.4 | ACS | FFS |  | FFS |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |  |

Proposal 5:		For antenna array parameters, the LS can have the same table with the simulation assumptions in the TR.

Reference

[1] R4-2400333	Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency
 				bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

[2]	RP-241326	Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

[3]	R4-2410576	LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and 
 				compatibility studies in preparation for WRC-27

[4]	R4-2414449	LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and
 				compatibility studies in preparation for WRC-27

[5]	R4-2417107	WF on 15GHz BS parameters

[6]	R4-2417108	WF on 15GHz UE parameters