3GPP TSG RAN WG4 Meeting #113	R4-2419432

Orlando, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	5.23.1

Source: 	Ericsson, Qualcomm Incorporated.

Title:	RRM requirements for L1/L2 based inter-cell mobility

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the some of the open issues on L1/L2 triggered inter-cell mobility (LTM).

# Discussion

In last meeting it was agreed to support LTM without L1-RSRP report. In this contribution we discuss requirements to support the LTM without L1-RSRP report in the subsequent sections.

## TCI state activation requirements for LTM candidate cell

As per RAN2 design, one MAC-CE activates TCI states of a single candidate cell. If UE supports multiple TCI states activation, NW may activate the TCI states of different cells at the same time. For multiple candidate cells TCI state activation, NW may send MAC CE activation command at the same time in the same MAC PDU or in a different MAC PDU at different time instances. In this scenario, TCI state activation of the UE depends on different cases. The TCI state activation requirements depend on different aspects such as whether SSB are overlapping among the candidate cells or whether the candidate cell is intra-frequency candidate cell or inter-frequency candidate cell or whether the cell operates on FR1 or FR2.

For intra-frequency FR1 cells: Irrespective of whether SSB of the candidate cells overlap or not, if the candidate cell is an intra-frequency cell, UE can perform the T/F tracking in parallel (i.e., at the same measurement occasion).

For inter-frequency FR1 or intra-frequency/inter-frequency FR2 cells: If the SSB are overlapping, UE need to perform T/F tracking in sequential order as UE measures one frequency layer at a time in FR1 and one cell at a time in FR2. If the SSB are not overlapping, UE can perform T/F tracking in parallel (that means on non-overlapping resources).

Further in previous meeting, RAN4 agreed to define the TCI state activation requirements for known and unknown TCI state activation in FR1.  In the rest of this section, we discuss the requirements for FR1 known and unknown TCI state activation and FR2 known TCI state activation requirements.

While defining the TCI state activation requirements we think we can borrow the principles used in legacy TCI state activation.

- Principle 1: If the TCI state is known, and if the TCI state is not in the active TCI state list, to activate the TCI state in FR1 or FR2, UE needs one SSB sample to obtain T/F tracking.

- Principle 2: If the TCI state is unknown, in FR1, UE need one SSB sample for T/F tracking.

- Principle 3: If the TCI state is unknown in a FR2 cell, UE needs to perform L1-RSRP measurement and additional T/F tracking to complete the TCI state activation.

In this section, we first look at the single cell TCI state activation requirements and then define the scaling rules when more than one cell is to be activated. If a TCI state activation command is received at slot n, UE shall be able to finish the TCI state activation within slot n+ THARQ +3Nslotsubframe,µ+TTF + TSSB-proc. Where TTF for single cell in different scenario is mentioned below.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 ..., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 ..., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length |

When a UE supports more than one candidate cell TCI states activation, NW may activate the TCI states from multiple cells. Since one cell TCI states are activated using one MAC CE, NW may send the different MAC CE to activate more than one cell in same slot or different slot. Following scenario may occur for activating the more than one cells TCI states.

- Scenario 1: MAC CE for TCI state activation for different candidate cells are received within same slot.

- Scenario 2: MAC CE for different candidate cells are not received within same slot but received before the TCI state activation completion of another cell.

Since supporting more than one candidate cells TCI state activation is up to UE capability, if UE supports and reports this capability, it is important for the NW to know the UE behaviour for the TCI state activation. We further discuss the expected UE behaviour for the scenario 1 and 2.

Scenario 1: UE activates both cells either in parallel or sequential depending on the intra or inter-frequency cases.

Scenario 2: this case may be bit complicated as we may need to define a window to define a UE measurement behaviour.

To simply the specification work, we can leave this scenario to UE implementation. That means, if NW would like to schedule multiple candidate cells TCI state activation, it can either do using scenario 1 or after the completion of the other ongoing TCI state activation.

In the next table we discuss the requirements for scenario 1.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |

Based on the above analysis we make following proposal.

- When one or more of TCI state activation commands are received at slot n, UE shall be able to finish the TCI state activation within slot n+ THARQ +3Nslotsubframe,µ+TTF + TSSB-proc. where TTF  is mentioned in the below table.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |

Maximum time gap between TCI state activation and cell switch command:

Other open issue is what is the maximum time gap between completion of TCI activation and cell switch command to skip the T/F tracking. In last meeting following WF is agreed.

  - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

    - the time gap between completion of TCI activation and cell switch command is larger than 480ms

    - Note: This is not to extent 160ms to 480ms directly. Longer L3 measurement delay may be expected in the 480ms duration.

  - Discuss the wording in CR.

In the CR, following is captured.

The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and

-	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8. 25.3, and not more than TCI state activation delay stated in section 8. 25.3 + [480 ms], or

[Note: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.]

-	The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].

We think the tentative time gap of 480ms agreed in last meeting can be quite helpful for the on the NW implementation in inter-CU scenarios. For the case of inter-CU LTM, when UE sends PRACH to target cell, we expect the following sequence of operations before the cell switch command is received at the UE.

- Target DU estimate the TA

- Target DU sends TA command to target CU.

- Target CU sends TA command to source CU

- Source CU sends TA command to source DU

- Source DU prepares cell switch command

- Source DU send cell switch command to the UE

- Potential retransmissions of the cell switch command due to weak channel conditions in the HO region

Since different CU can be involved, and the CU can belong to different vendors, we may not guarantee to perform all these operations within 160ms. We also understand the concern raised during last meeting about the L3 measurements may be impacted due to additional T/F tracking UE may need to perform on the activated TCI states. We are fine to have this statement on the L3 measurements.

- In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

    - the time gap between completion of TCI activation and cell switch command is larger than 480ms.

    - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

## LTM Cell switch delay requirements

In this section we discuss the following open issues.

### TCI state indication and activation in cell switch command

Last meeting following agreement is made. However same is not captured in spec. We suggest capturing this using proposal 3.

- Confirm current cell switch delay requirements are applicable to the case that TRS is configured as a QCL source in the indicated TCI state in cell switch command.

- FFS whether Tfirst-ssb in cell switch delay requirements need to be updated, e.g. based on TRS.

- We suggest modifying the TCI known condition to following

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

-	The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or

-	The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or

## LTM UE features

### Capability related to cell switch delay

RAN4 agreed to introduce LTM fast RRC processing capability with following two components

•	maxNumberStoredConfigCells: indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and SCells(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations.

•	maxNumberConfigs-r18: represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133.

RAN2 sent a LS asking for the feedback on following questions.

Currently, only the number of LTM candidate configurations is visible in LTM-config, while the number of SpCell/SCell(s) in LTM candidate configurations is not visible until the UE decodes LTM candidate configurations. RAN2 would like to know the answers for the following three questions: - As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations? - Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration? Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

If the early DL (TCI state activation on candidate cell) or UL sync (PDCCH order to transmit RACH to candidate cell) is not triggered on a LTM candidate cell, UE is not required to decode that particular LTM candidate config as the UE does not know if the NW will perform the cell switch to that cell. If the early DL sync or early UL sync procedures are not triggered, even if the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations less than maxNumberStoredConfigCells, UE is not required to perform fast RRC processing on those cells.

Even when early UL and DL sync are triggered following scenarios can occur. To further analyse, we further consider following examples. UE capability for max number of stored cells is let’s say 16.

When the serving cells (activated or deactivated) cells are less than 16, let’s say 6 serving cells are configured across MCC and SCC (including SpCell and activated and deactivated SCell), upon early DL or UL sync execution, UE decodes the LTM candidate configuration and checks how many cells are configured in the LTM candidate configuration. If the number of total cells (SpCell and SCells configured in LTM configuration) in that particular LTM candidate configuration, is less than 10, UE stores decoded LTM candidate configuration. If the number of cells in the LTM candidate configuration is more than 10, UE is not required to store the LTM candidate configuration of SpCell also (as UE is expected to store full configuration).

For further analysis, we consider following example.

Total number of cells for fast RRC processing UE can store is 16, i.e. ltm-FastProcessingConfig-r18 = 16.

- # of the current active/deactivated serving cells:6

- # of the LTM candidate configurations: 4

  - LTM candidate configuration 1: total cells configured including SpCell and SCells are 6

  - LTM candidate configuration 2: total cells configured including SpCell and SCells are 4

  - LTM candidate configuration 3: total cells configured including SpCell and SCells are 5

  - LTM candidate configuration 4: total cells configured including SpCell and SCells are 6

- At time t1, NW performs the early sync to candidate cell 1.

  - Total cells: 16;

  - Utilised so far: 6:

  - Number of cells in current LTM candidate: 6;

  - UE action: Stores the LTM candidate config 1;

  - remaining cells after storing current LTM candidate: 4,

- At time t2 (>t1), NW performs the early sync to candidate cell 2.

  - Total cells: 16;

  - Utilised so far: 12:

  - Number of cells in current LTM candidate: 4;

  - UE action: Stores the LTM candidate config 2;

  - remaining cells after storing current LTM candidate: 0.

- At time t3 (>t2), NW performs the early sync to candidate cell 3.

  - Total cells: 16;

  - Utilised so far: 16:

  - Number of cells in current LTM candidate: 5;

  - UE action: Stores the LTM candidate config 3 and replaces one of the LTM config 1;

  - remaining cells after storing current LTM candidate: 1.

- At time t4 (>t3), NW performs the early sync to candidate cell 4.

  - Total cells: 16;

  - Utilised so far: 15:

  - Number of cells in current LTM candidate: 6;

  - UE action: Stores the LTM candidate config 4 and may remove LTM candidate configuration 2 and 3;

  - remaining cells after storing current LTM candidate: 4.

RAN4 has been discussing a specific rule about how to down-select a set of LTM candidate configurations for fast RRC processing. As shown above, the down-selection rule aims to ensure that the UE and NW are always in sync regarding which configurations will be selected for early processing and whether the UE can store all of them before the LTM cell switch command reception/execution, regardless of whether the number of cells configured/to-be-stored. Therefore, we do not think any additional information is absolutely necessary.

A problem with providing the number of cells configured within a LTM candidate cell is that UE may need to report dynamically the capability on whether it is capable of performing fast processing or not. First, this is not in line with the concept of capability reporting itself, since the UE capabilities are static and network pull them from the UE very seldomly. Second, this solution will generate a huge signalling overhead for both the UE and the network as the capability container (which is quite large and most of the time needs to be segmented) would need to be reported very often.

Our understanding of the RAN2 discussion was some companies was of the understanding that first network indicates total cells and number of LTM candidate cells, and then UE reports the capability. We think this involves lot of overhead as whenever NW changes the LTM candidate UE is required to report if it can support the fast RRC processing or not. This can make the capability practically useless. We think the capability should be exchanged only once and it is NW responsibility to configure as per UE reported capability if fast RRC processing capability to be utilised by the NW. Hence, we think providing the number of cells in the LTM candidate config introduces more issue than it solves.

- RAN4 to confirm to RAN2 that NW do not need to provide the total cells information in the LTM candidate configuration and inform RAN2 about the RAN4 agreements on how the fast processing is expected to work.

Further RAN4 was discussing following scenario in last meeting.

- If the total number of cells to be fast RRC decoded does not exceed UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not exceed maxNumberConfigs-r18

  - TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH,

- Otherwise,

  - TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) [and/or] PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

    - FFS: This is at least applicable to the case that NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

    - FFS: Whether to support and further optimize the case that NW triggers TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

Based on the above analysis on how the UE capability should work, NW is not expected to configure in excess of UE capability reported. We think RAN4 is spending time on the configurations that do not exist in field. We suggest not to support the cases when the configuration exceeds UE capability. With this we suggest remove the [] in last meeting agreement and no need to further discuss the FFS parts. We also suggest some rewording of the proposal to remove the conception of NW configures in excess of UE capability.

- Till the total number of cells to be fast RRC decoded does not reach UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not reach maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH. Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) till the UE capability reach maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

According to the current UE capability structure of the feature, the UE can separately indicate its capabilities for the maximum number of candidate cells for early PRACH transmission and the maximum number of candidate cells for LTM RRM configuration. To maximize the benefit of LTM feature support, UE will need to report the same value for both capabilities. Otherwise, LTM candidate cells would need to be reconfigured via RRC to add or remove the early PRACH-related field from each LTM candidate cell based on the reported L1/L3 measurement results, which is undesirable due to significant RRC signalling overhead and latency.

Considering the limited UE capability in terms of memory and other resources to keep track of the time and frequency of all candidate cells to which early PRACH transmission was conducted, we propose a new UE capability for RACH-less LTM cell switch support. This capability would effectively limit the number of target cells for LTM cell switch based on the reverse chronological order of early PRACH transmission.

- RAN4 to define the following UE capability:

- RACH-less LTM cell switch can be conducted to one of ‘N’ cells to which the UE most recently transmitted the ‘PDCCH-order PRACH’ except for the cell configured as SCell.

  - N = {[1], 2, …, 7}, if not reported, N=8.

  - Granularity: Per UE

# Summary and Conclusion

In this contribution we have analysed RAN4 aspects for L1/L2 based inter-cell mobility and made following proposals.

- When one or more of TCI state activation commands are received at slot n, UE shall be able to finish the TCI state activation within slot n+ THARQ +3Nslotsubframe,µ+TTF + TSSB-proc. where TTF  is mentioned in the below table.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |

- In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

    - the time gap between completion of TCI activation and cell switch command is larger than 480ms.

    - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

- We suggest modifying the TCI known condition to following

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

-	The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or

-	The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or

- RAN4 to confirm to RAN2 that NW do not need to provide the total cells information in the LTM candidate configuration and inform RAN2 about the RAN4 agreements on how the fast processing is expected to work.

- Till the total number of cells to be fast RRC decoded does not reach UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not reach maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH. Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) till the UE capability reach maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

- RAN4 to define the following UE capability:

- RACH-less LTM cell switch can be conducted to one of ‘N’ cells to which the UE most recently transmitted the ‘PDCCH-order PRACH’ except for the cell configured as SCell.

  - N = {[1], 2, …, 7}, if not reported, N=8.

  - Granularity: Per UE

# References

- R4-2410302 WF on mobility enhancement’s part 1, Mediatek

- RP-221799 “WID on Mobility enhancements”, MediaTek