3GPP TSG RAN WG4 Meeting #113		 R4-2419434

# Orlando, Florida, US, November, 18 – 22, 2024

Source:	EchoStar, Boost Mobile Network (former DISH Network), Terrestar

Title:	On n252 UE Coexistence Relaxation

Agenda item:			6.17.2

Document for:	Approval

# 1. Introduction

In the last RAN4#112bis, the following way forward was agreed for n252 UE to UE coexistence [1]:

- Agreement: - Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz, that allows the NTN service to work effectively, across the full n252 band, whilst still protecting the TN UE at least in the majority of cases - Further discuss the actual value and methodology for the requirement (e.g. corresponding separation assumption) - Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations - Further discuss how to capture the solutions in the specification

In this paper, we consider aspects from recent standard agreement and deployment perspective to propose a value for n252 UE spurious emission and to specify as A-MPR with separate NS values for n2 and n25.

# 2. Discussion

## 2.1 UE co-existence spurious emission limit for n252

In the last RAN4#112bis meeting, RAN4 could not reach agreement on adopting probabilistic approach to estimating the UE-to-UE separation distance in order to specify the UE co-existence spurious emission limit for protecting bands n2 and n25.

As an alternative to the probabilistic approach (that was not agreed), in order to define a sufficient protection value for n2 and n25 while still allowing the NTN service to work effectively across the full n252 band, the following aspects are considered to develop the proposal for the UE coexistence spurious emission limit for n252 to protect terrestrial bands n2 and n25:

- Review of past precedence in terms of adopting relaxed limit (other than -50 dBm/MHz) for UE spurious emission in the current specification

We reviewed the UE coexistence spurious emission limit specified for various bands in TS 38.101-1 and observed that 3GPP has specified value more relaxed compared to -50 dBm/MHz for several bands. (See Annex). More recently RAN4 agreed a limit of -30 dBm/MHz for n106 (UL 896 MHz – 901 MHz, DL 935 MHz – 940 MHz) to protect E-UTRA band 5 and band 26, in which the n106 UL and band 5/26 DL separation is only 4 MHz (Only 3 MHz channel bandwidth between 898 – 901 MHz has been specified). There are several TDD bands (e.g., n41/n53, n77/n48) where TDD synchronization is not mandated by regulators between operators. As a result, there are no guarantees that adjacent spectrum licensees would adopt the same TDD pattern to ensure there is no UE-UE coexistence issue.

Observation 1: 	     Several bands have relaxed UE coexistence spurious emission.

Observation 2: 	Several US TDD bands do not have UE-UE coexistence limit of -50 dBm/MHz even when TDD synchronization is not guaranteed by regulatory rules.

Observation 3: 	Relaxed emission limit of -30 dBm/MHz value was agreed for n106 to protect nearby bands n5 and n26.

- Implications of domestic roaming

T-Mobile has raised the scenario where, as band 70 and 71 coverage ends[ref], an NTN UE would connect to n252 while T-Mobile’s network remains available in the area due to its more extensive coverage. This may result in NTN UEs and TN UEs operating in close proximity to each other, with a potential risk of NTN uplink (UL) interfering with the terrestrial network (TN) downlink (DL).

3GPP has developed NTN specifications assuming that UE will connect to the NTN only when there is no TN network on any band. With domestic roaming agreement in place for Boost Mobile Network while it matures its terrestrial network, n252 UEs are expected to connect only if there is no TN coverage from its own network or from its domestic partners’ TN coverage. In the coverage area of n2 and n25, it will automatically connect to T-Mobile or AT&T networks nationwide when Boost Mobile Network’s coverage is unavailable. Under these conditions, the likelihood of n252 UE causing interference to n2 and n25 TN UEs is expected to be minimal.

Observation 4: 	With the domestic roaming agreement in place, the likelihood of n252 NTN UE becoming an aggressor to TN UE in the coverage area of n2 and n25 is very minimal.

- DL TN n66 impact to service availability of NTN DL UE in n252

In the Rel-17 TR 38.863, it was concluded that that NTN UE DL performance significantly degrade when dropped near or inside the TN coverage area, due to adjacent channel interference from high powered TN base stations. Given the maturity of buildout of terrestrial networks in bands n2, n25 and n66, there is a very high probability of overlapping coverage from n2, n25 and n66. It is therefore likely that the availability of NTN service in the coverage area of n2 and n25 bands will be impacted due to n66 DL. As a result, this will further reduce the likelihood of n252 NTN UE impacting the DL of n2 and n25 TN UEs.

Observation 5: In the case where n2, n25 and n66 TN coverages are overlapping, service availability for n252 in the coverage area of n2 and n25 bands will be impacted due to n66 DL; as a result the probability of n252 NTN UE becoming the aggressor to n2 and n25 TN UEs will be even lower.

From the above observations, it is clear that the risk of interference from n252 UE to n2 and n25 UEs is minimal. However, to further reduce the risk of interference from n252 UE to n2 and n25 UEs, it is proposed that the emission limit of -30dBm/MHz be adopted (as was done for recently specified North American TN band 106).  It is understood that that this may require power back-off for certain allocation for wider channels while still allow full utilization of the authorized spectrum for NTN service.

Proposal 1: 	Adopt -30dBm/MHz as the UE spurious emission limit for n252 to protect n2 and n25 UEs.

## 2.2 How to specify the UE spurious emission

Once we agreed with the value, we propose specifying -30dBm/MHz UE spurious emission for n252 in the A-MPR table. The A-MPR approach allows the operator to utilize the whole authorized spectrum and at the same time providing guidance for the operator to carefully place the channel bandwidth and allocate the resource.

In the last RAN#112bis, there was a proposal to introduce RB restriction [2]. However, restricting a certain RB allocation in the specification will lead to inefficient utilization of the authorized spectrum and unnecessary restriction in deployment flexibility. By using A-MPR specification, the power reduction in the RB region that is otherwise restricted can be mitigated by e.g., satellite implementing better transmit antenna/power, UE implementation, allocation in the certain area that may not have n2/n25 neighbors, etc. thereby supporting more efficient deployment. Also, considering the possibility of different deployment condition of n2 and n25, we also propose that separate NS values are defined.

Proposal 2: 	To define A-MPR for -30 dBm/MHz UE spurious requirement for all n252 channel bandwidths defined in the WID

Proposal 3: 			To define two separate NS value for n2 and for n25

# 3. Conclusion

Aspects from recent standard agreement and deployment perspective were considered to derive emission value for UE-UE coexistence between n252 and n2/n25. The following were observed:

Observation 1: 	   Several bands have relaxed UE coexistence spurious emission.

Observation 2: 	Several US TDD bands do not have UE-UE coexistence limit of -50 dBm/MHz even when TDD synchronization is not guaranteed by regulatory rules.

Observation 3: 	Relaxed emission limit of -30 dBm/MHz value was agreed for n106 to protect nearby bands n5 and n26.

Observation 4: 	With the domestic roaming agreement in place, the likelihood of n252 NTN UE becoming an aggressor to TN UE in the coverage area of n2 and n25 is small.

Observation 5:	In the case where n2, n25 and n66 TN coverages are overlapping, service availability for n252 in the coverage area of n2 and n25 bands will be impacted due to n66 DL; as a result the probability of n252 NTN UE becoming the aggressor to n2 and n25 TN UEs will be even lower.

To define a sufficient protection value for n2 and n25 while still allowing the NTN service to work effectively across the full n252 band, the following are proposed:

Proposal 1: 	Adopt -30dBm/MHz as the UE spurious emission limit for n252 to protect n2 and n25 UEs.

Proposal 2: 	To define A-MPR for -30 dBm/MHz UE spurious requirement for all n252 channel bandwidths defined in the WID

Proposal 3: 			To define two separate NS value for n2 and for n25

# Reference

- R4-2417206, “WF on NTN and IoT-NTN bands, Moderator (Inmarsat), RAN4#112bis, Hefei, China, 14 – 18 October, 2024

- R4-2416507, “NTN S-Band UE-UE coexistence with TN”, T-Mobile USA Inc., AT&T, UScellular, RAN4#112bis, Hefei, China, 14 – 18 October, 2024

# ANNEX: List of some bands with relaxed UE spurious emission limit

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
| n7 | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| n18 | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
| n26 | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
| n28, n83 | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
| n38 | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| n39, n98 | Frequency range | 1805 | - | 1855 | -40 | 1 | 33 |
| n41 | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 2530 | - | 2535 | -25 | 1 | 49 |
|  | Frequency range | 2505 | - | 2530 | -30 | 1 | 49 |
| n71 | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
| n74 | Frequency range | 1475 | - | 1488 | -28 | 1 | 15, 42 |
|  | Frequency range | 1475.9 | - | 1510.9 | -35 | 1 | 15, 46 |
| n106 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -30 | 1 |  |
| n109 | Frequency range | 758 | - | 773 | -32 | 1 | 15 |