3GPP TSG-RAN WG4 Meeting #113	R4-2419454

Orlando, US, 18 – 22 November, 2024

Source:	Nokia

Title:	Discussion on Fragmented Carrier reduction of Rx RF chains

Agenda item:	7.6.2

Document for:	Discussion

Introduction

Agreements at previous meetings

The latest WF for NR FR1 DL fragmented carriers study [1] presents one of the open issues as found below:

#### Issue 2-4: Discussion on semi-statically switch hardware resources

Discussion

In this document we further discuss the RF architecture and how the hardware is used in relation to the operation of fragmented carriers.

Semi-statically switched hardware resources

In the fully shared RF chain architecture, using fewer Rx RF chains is the end goal of the study item. From the presented architectures and compared to the partially shared RF chain architecture, it is found that the UE loses performance in receiver selectivity and sensitivity when it receives using only one analogue baseband comprising an analogue channel filter for receiving two non-contiguous intra-band carriers.

The UE loses performance in receiver selectivity and sensitivity when it receives using only one analogue baseband comprising an analogue channel filter for receiving two non-contiguous intra-band carriers.

The implementation of the receiver in the UE design may present different challenges to conform with the type of reception of this study item. The identification of the Rx signal properties and receiver conditions in a configured state of fragmented carrier operation must be an aspect of the study while deriving Rx requirements, leaving it to the UE implementation to comply with the requirements.

The identification of the Rx signal properties and receiver conditions in a configured state of fragmented carrier operation must be an aspect of the study while deriving Rx requirements, leaving it to the UE implementation to comply with the requirements.

In operating bands with 100 kHz channel raster, the CCs can have different subcarrier grids at 15 kHz SCS. A UE receiving carriers on a 100 kHz channel raster shall support these cases of different subcarrier grids even if it needs more than one FFT. This is in our view independent of using fewer Rx RF chains in relation to the reception of fragmented carriers. Hence, we do not see that using fewer Rx RF Chains, even to the fully shared RF chain, relates to any relaxation or limitation in FFT use at the UE.

A UE receiving carriers on a 100 kHz channel raster shall support the case of different subcarrier grids, even if it needs more than one FFT, independent of using fewer Rx RF chain in relation to the reception of fragmented carriers.

Given this study item is focused in finding impact on RF requirements and fragmented carrier operation it is expected to become a feature which is to be declared supported or not by a UE it should not be the UE's digital baseband resources which limits or dictates RF requirements.

Any need to use separate FFTs for the CCs shall not limit the benefit from DL fragmented carrier operation for increasing the total number of DL CCs that the UE can support.

The UE is likewise not obligated to indicate the achieved additional carrier support from receiving with the fully shared RF chain configuration. It is for the network to determine if the desired end goal is achieved from the UE capabilities of supported band combinations. Only the band combination list of the UE reflects the additional carrier combinations supported in higher order CA combinations from applying the fragmented carrier operation in one or more inter-band combinations with fragmented carriers in at least one of the bands.

Only the band combination list of the UE reflects the additional carrier combinations supported in higher order CA combinations from applying the fragmented carrier operation in one or more inter-band combinations.

From the agreed terminology and reference architecture, the number of antennas activated is not considered in the SID. It follows that discussions related to the actual operation of antennas at the UE where the number of active antennas impacts the UE receiver performance is relevant for the study. Of the example bands included from the agreements in RAN4#112-bis, only bands n7 and n41 have mandatory operation at 4Rx, while it is up to UE implementation to declare 2Rx or 4Rx for any single, or multi-carrier configuration of bands n2/n25, n3, n26 or n66. Other bands may be added to the study later, provided they support non-contiguous intra-band carrier aggregation.

The number of active antennas impacts the UE receiver performance and is relevant for the study.

It is up to UE implementation to declare 2Rx or 4Rx for any single, or multi-carrier configuration of bands n2/n25, n3, n26 or n66.

Since the UE may support the bands n2/n25, n3, n26 and n66 with more than the mandated number of antennas in the downlink, we propose that the study must consider the aspect whether a UE may trade off the number of Rx antennas as a means for using fewer Rx RF chains when receiving fragmented carriers. The following Fig.  1shows a UE using four Rx RF chains for receiving fragmented carriers using the wider analogue filter.

Fig.  1: Single filter fragmented carrier scenario

Considering the UE is for some bands only mandated to receive using a minimum of 2Rx antennas, a UE may divide the use of the Rx RF chains among the antennas in a way possible to support the fragmented carrier operation using the same number of Rx RF chains. This allows the UE to improve the Rx performance by applying the analogue channel filter matching the respective channel bandwidth. An example of this is shown in Fig.  2, where 2Rx is configured for each fragmented carrier with a channel filter matching the carrier bandwidth.

Fig.  2: Selective filter fragmented carrier scenario

The study must consider the aspect whether a UE may trade off the number of Rx antennas as a means for using fewer Rx RF chains when receiving fragmented carriers.

If a UE receives at different downlink antenna configurations, it would also be relevant to discuss if the case of the aggregation of fragmented carriers in the bands n7 and n41 could relieve the UE, only when configured in fragmented carrier operation in these bands, of the requirement to support 4Rx.

Discuss if the case of the fragmented carrier operation in the bands n7 and n41 could relieve the UE, only when configured in fragmented carrier operation in these bands, of the requirement to support 4Rx.

There are several UE implementation aspects involved in the configurations within this study. In this section we have brought attention to these. Since there is a variety of ways to support fewer Rx RF chains, we want to encourage RAN4 to identify the mechanisms that surround the UE implementation, i.e. when fragmented carrier operation can be supported. These mechanisms can be thresholds, measurements, network managed transitions or UE events that alter the UE configuration to comply with the non-contiguous intra-band CA receiver requirements, when entering, operating or exiting the fragmented carriers operational state.

RAN4 to identify the mechanisms that surround the UE implementation, i.e. when fragmented carrier operation can be supported.

Prioritization of fragmented carriers

Currently the SID is limited to the case of 2 CC in total. Additionally, the SID states that: “Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.”). From that it can be understood that one of the fragments has to be considered as a PCELL, while the other can be considered as a SCELL. On the other hand, the study should be future-proof for the higher order inter/intra-band combinations [2]. Meaning that if the intention is to use fragmented carrier operation only in combination with another CC, then the scenario would be that both of the fragments can be defined as SCELLs.

RAN4 to clarify whether either of the carriers in a fragmented carrier pair shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s.

Additionally, if RAN4 decides that there shall be a PCELL as a part of the fragmented carrier configuration, RAN4 needs to consider whether there should be prioritization on this CC meaning that if any of the mechanisms as discussed in section 2.1 is triggered and the UE can not sustain fragmented carrier operation, it can fallback to the PCELL only to avoid link failure.

RAN4 to consider fallback behaviour with priority to a PCELL, if defined.

Syntax for defining fragmented carriers

We find it necessary to clearly distinguish the radio configuration of the non-contiguous intra-band carrier aggregation between the modes of operation related to using the partially shared RF architecture and the fully shared RF chain at the UE side. It must be agreed that configuring the UE using the fully shared RF chain includes the operation at the UE as identified in this study including a risk of performance degradation.

The currently undefined NR CA bandwidth class (BCS) ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

TS 38.101-1: Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34, 4 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |  |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |  |
| F | BWChannel ≤ 100 MHz | 1, NOTE 5 | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |  |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 5: For fragmented carrier operation with a single contiguous frequency range configured for the UE with multiple fragmented carriers as indicated by IE [FragmentedCarrier]. |  |  |  |

In the proposal above, where the newly added parts are highlighted with yellow, F is inserted with a corresponding number of contiguous CCs of 1 as the intention is to treat the multiple fragmented carriers as a single in the perspective of Rx RF chains. The added Note 5 clarifies that this single contiguous CC can consist of multiple fragmented carriers. The new BCS, F, is then added with fallback group 4, which is added correspondingly for the single CC BCS, A, as the fallback from fragmented carrier operation would be single CC operation.

The currently undefined NR CA bandwidth class ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

By adding the new NR CA bandwidth class this can be used to identify fragmented carrier operation and the number of fragments. Examples using band n25 with two and four (should the concept evolve beyond the 2CC in scope) fragments are shown in the figures below.

Fig.  3: Example of CA_n25(2F)

Fig.  4: Example of CA_n25(4F)

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: The UE loses performance in receiver selectivity and sensitivity when it receives using only one analogue baseband comprising an analogue channel filter for receiving two non-contiguous intra-band carriers.

Proposal 1: The identification of the Rx signal properties and receiver conditions in a configured state of fragmented carrier operation must be an aspect of the study while deriving Rx requirements, leaving it to the UE implementation to comply with the requirements.

Proposal 2: A UE receiving carriers on a 100 kHz channel raster shall support the case of different subcarrier grids, even if it needs more than one FFT, independent of using fewer Rx RF chain in relation to the reception of fragmented carriers.

Proposal 3: Any need to use separate FFTs for the CCs shall not limit the benefit from DL fragmented carrier operation for increasing the total number of DL CCs that the UE can support.

Observation 2: Only the band combination list of the UE reflects the additional carrier combinations supported in higher order CA combinations from applying the fragmented carrier operation in one or more inter-band combinations.

Observation 3: The number of active antennas impacts the UE receiver performance and is relevant for the study.

Proposal 4: It is up to UE implementation to declare 2Rx or 4Rx for any single, or multi-carrier configuration of bands n2/n25, n3, n26 or n66.

Proposal 5: The study must consider the aspect whether a UE may trade off the number of Rx antennas as a means for using fewer Rx RF chains when receiving fragmented carriers.

Proposal 6: Discuss if the case of the fragmented carrier operation in the bands n7 and n41 could relieve the UE, only when configured in fragmented carrier operation in these bands, of the requirement to support 4Rx.

Proposal 7: RAN4 to identify the mechanisms that surround the UE implementation, i.e. when fragmented carrier operation can be supported.

Proposal 8: RAN4 to clarify whether either of the carriers in a fragmented carrier pair shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s.

Proposal 9: RAN4 to consider fallback behaviour with priority to a PCELL, if defined.

Proposal 10: The currently undefined NR CA bandwidth class ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

- References

- R4-2417201, WF on NR FR1 DL fragmented carriers study, MediaTek Inc.

- R4-2414457, WF on fragmented DL carriers study, MediaTek Inc.