3GPP TSG-RAN WG4 Meeting #113						R4-2419474

Orlando, US, November 18 – 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | draftCR | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2 with Annex N Expected EIRP calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is draft CR to TS 38.141-2 with new Annex N that provides EEIRP mask calculations according to work split agreed during RAN4#112 meeting in R4-2413509. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New annex N added with EIRP calculation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will not include details on EEIRP calculations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex N |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of draft CR>

# Annex N (normative):

# Expected EIRP calculation

# N.1	General

The key requirements of ITU Resolution Resolution 220 [25] are:

- compliance with values of Expected EIRP above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to Resolution 220 [25].

The Expected EIRP calculation is based on two averaging processes:

- Averaging of beamforming directions

- Averaging over horizontal and vertical angles

These two averaging processes are inter-related and should not be viewed as the order in which the calculation is done.

    - N.1.1 Averaging over beamforming directions

The first averaging process involves averaging over different beamforming directions. For a given vertical angle θ and horizontal angle φ with a sampling of K beamforming directions, the equation can be written as:

EIRPθ,φ=k=1KwkEIRPk(θ,φ)                        (1)

where wk refers to the weight for the kth beamforming direction, i.e., the fraction of the steering range represented by the kth beamforming direction.

For the case where there are K uniform equispaced beams assumed in the azimuth and elevation, respectively, and where each beam covers an equal range of angles, the equation can be written as:

EIRPθ,φ=1Kk=1KwkEIRPk(θ,φ)                              (2)

The power measured is the sum of the EIRP in both polarisations.

The EIRP of each individual AAS beam denoted by EIRPk(θ,φ) is an average EIRP calculated from the second averaging process (i.e. in the next section, N.1.2).

    - N.1.2 Averaging over horizontal and vertical angles

The second averaging process is over a spherical strip. This average EIRP for a spherical strip is therefore:

EIRPθHLi , θHHi=12πsinθHHi-sinθHLiθHLiθHHi-ππEIRPθ,φ cosθ dφ dθ                        (3)

With the help of solid angles and spherical cap formula, we can approximate this double integral in discrete form. For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth ϕ (over the range of [-π to π]), and with N and M points respectively, this average EIRP can be approximated to

EIRPθHLi , θHHi=12πsinθHHi-sinθHLiθLθH-ππEIRPθ,φ cosθ ∆φ ∆θ    ,                     (4)

where

dφ≈∆φ=2πM

dθ≈∆θ=π2N

Therefore, the overall equation can be written as

EIRPθHLi , θHHi=π2MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,                       (5)

where

θn	is the midpoint between the elevation intervals (midpoint is the middle point between the intervals, e.g. for the interval 5°-10°, the midpoint is 7.5°).

φm	is the midpoint between the azimuth intervals

nL	is the lowest elevation point within the θHLi , θHHi range

nH	is the highest elevation point within the θHLi , θHHi range

m	is the azimuth point within the -π , π range

Alternatively, equation (5) can be written as

EIRPθHLi , θHHi=(θHHi-θHLi)MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,                       (6)

by substituting the alternative ∆θ in equation (4) with

∆θ=θHHi-θHLiN

<End of draft CR>