3GPP TSG-RAN WG4 Meeting # 113						R4-2419480

Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			On CW cancellation capability

Agenda item:		7.21.3.4

Document for:		Approval

# Introduction

In this paper, we present our views on CW cancellation for A-IoT RAN4 study.

# Discussion

In this paper, we continue to discuss the CW cancellation related issue.

## CW cancelation

For CW cancellation, there are passive or active cancellation schemes existing for RFID reader front-end. In both schemes, a copy of the CW leakage signal is created with same amplitude but opposite phase. The cancellation scheme is illustrated in Figure 1. For the passive cancellation scheme, the cancellation path needs to be calibrated and for active cancellation scheme, the cancellation path can be tuned to adapt to the leakage CW signal. However, the cancellation is limited by either the hardware imperfection or detecting signal amplitude/phase accuracy.

However, when the CW and A-IoT signal are transmitted from different hardware, there is no way to create a local copy of leakage CW signal in A-IoT BS/UE as reader. Therefore, the passive CW cancelation scheme is not possible. Alternatively, the active CW cancellation scheme should be used in this case. This applies to the case where the CW node is co-located with A-IoT BS for D1T1-A2 or D2T2-A2. As reader needs to determine the correct amplitude and phase for the cancellation signal , it may take time at reader to perform CW cancellation , which means that additional CW cancellation training time would be needed before D2R reception. As such, the time needed to prepare the CW cancellation circuitry and enable the CW cancellation in D2R needs to be considered.

CW cancellation preparation time needs to be considered before D2R reception

The interference cancellation ratio is simulated with different SNR of the leakage CW signal. It can be observed from Figure 2 that the SNR cancellation performance relates to the SNR of the CW leakage signal and also depending on the phase and amplitude error between cancellation signal and leakage signal.

Reader will need some time to train the algorithm to converge on the designed performance target before starting the D2R reception. Figure 3 illustrates the different training time needed to reach different cancellation performance. It can be observed that the cancellation training time is implementation specific and also relates to the targeted cancellation performance.

Figure 1: CW cancellation scheme

Figure 2: ICR (Interference cancellation ratio) for different SNR level (SNR=50dB, 30dB and 10dB for picture at left, middle and right)

Figure 3: cancellation training time vs the performance [1]

To enable the CW cancellation function, reader needs some time to train the circuit to generate the reverse CW leakage with the same amplitude and the opposite phase. Considering this time and also the transient time to transmit the CW signal, the timing relation between the R2D, CW and D2R can be illustrated in Figure 4.

Figure 4:Transient time for CW and R2D transmission and CW cancellation training time before D2R reception

The transient time between R2D and CW can be specified and tested but the CW cancellation training time must be accommodated with protocol design.  The most relevant RAN1 agreement is below:

RAN1#117:

Agreement Scheduling information of PDRCH transmission is provided by a corresponding PRDCH. Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ³ TR2D_min - FFS the maximum value(s) for TR2D

As it is not clear how RAN1 designs the timing relation between R2D and D2R considering the CW cancellation, we think it is critical to consider the CW cancellation training time, because if reader is not ready with training the hardware to reach enough cancellation performance, it may fail the D2R reception. As such we think it is appropriate to send an LS to RAN1 and notify RAN1 there is additional consideration on D2R transmission time from CW cancellation perspective.

The LS wording is below:

RAN4 identify that when CW cancellation is enabled at reader, there is a need to consider the CW cancellation training time before D2R receiving after CW transmission. Reader may fail the D2R reception for early D2R transmission because CW cancellation needs a minimum training time to reach the full cancellation performance.

Additionally, RAN4 identify that there is a transient time between R2D and CW transmission. For device using the CW for energy harvesting, such transient time may impact the device power hold-up time.

RAN4 kindly asks RAN1 to consider above information in protocol design.

- TP starts here:

### 6.7.5	  Feasibility study

### 6.7.5	CW interference cancelation study

#### 6.7.5.1	General

The CW cancellation can be divided into 3 stages sequentially: (1) Spatial isolation for [separate TX/RX antenna configuration]; (2) Analog cancellation from antenna to ADC and (3) Digital cancellation at baseband.

Spatial isolation for [separate TX/RX antenna configuration]

For bistatic antenna configuration, the spatial isolation can be achieved by antenna isolation, or coupling loss between CW transmitter and D2R receiver.

For example, if the CW transmission source is separately deployed from the D2R receiver, the spatial isolation relies on the distance between the CW node and A-IOT node, it also depends on the directivity of the two antennas.

Analog cancellation

Interference cancellation in the RF front-end can be applied to reduce the desensitization of the receiver. In principle, it can be implemented by reconstructing a local CW with the same amplitude but opposite phase of a CW interferer received from antenna at the input of the LNA, and combine this cancellation signal with the received signal. The cancellation performance depends on the phase and amplitude error between the reconstructed and actual received CW interference.

Additionally, in the case of integrated carrier-wave transmitter and D2R receiver in sharing antenna case, the interference suppression could be provided by a circulator/directional coupler.

Digital cancellation

In the digital baseband, suppression can be achieved by using a narrow-band filter centered at the backscattered signal frequency. Digital interference cancellation can also be achieved by using  a low complexity self-interference active cancellation technique as shown in the figure 6.7.5.1-1.

Figure 6.7.5.1-1. Full-duplex system model [2]

#### 6.7.5.2	Interference cancelation analysis

##### 6.7.5.2.1	Ericsson

The min received power could be -74 dBm or lower, the signal with such low level received power should be decoded in the presence of the CW transmission which is either 33 dBm or 23 dBm for monostatic antenna and reference at antenna. The CW transmission can leak to the receiver chain via either reflection from environment or the imperfection of the isolation between transmitter and receiver when CW is transmitted by A-IoT BS or UE as reader. As the receiver has limited dynamic range and linearity, this CW leakage needs to be attenuated/isolated to mitigate the desensitization of the receiver.  The CW signal frequency is used to modulate the backscatter signal and the frequency separation of the CW signal and modulated signal is around 100kHz for example as illustrated in Figure 6.7.5.2.1-1 according to the current D2R waveform design. Using an RF filter before the LNA to attenuate the CW signal is not possible. As such, a high-level CW leakage signal will be present together with a low wanted backscattered signal before LNA.

Figure 6.7.5.2.1-1: D2R waveform in frequency domain

To analysis the impact of the receiver desensitization, the backscattered signal is modelled as below [2]:

Sb (t)=Abcos⁡(ωb∙t+φb)∙cosωc∙t+φc

with ωc representing the CW frequency and ωb is the message frequency.

And the leakage CW signal is modelled as below:

SI (t)=AI∙cosωc∙t+φI

Then the receiver nonlinearity is modelled with a power series below assuming the second-order nonlinear product can be filtered:

yot=a1∙xt+a3∙xt3

Where a1 is the small signal gain of the receiver and a3<0 represents the nonlinear behaviour of the system.

The input signal before LNA is then:

xt=Sb t+SI (t)

Assuming the φb=φc=φI=0, at output of the LNA, only the terms associated with fundamental frequency coswbtcos⁡(ωct) are of interest and they are below:

yoIt=a1Ab+916a3Ab3+94a3AbAI2cosωb∙t∙cosωc∙t

As the CW signal is much larger than backscattered signal, the term 916∙a3∙Ab3 can be dropped and further simplified with:

yoIt=a1Ab+94a3AbAI2cosωb∙t∙cosωc∙t

The voltage of the fundamental signal at output of LNA can be reformulated as:

vt=a1∙Ab∙1+94∙a3a1∙AI2

Since a3<0, we can write:

vt=a1∙Ab∙1-94∙a3a1∙AI2

It can be observed from the above that the wanted signal level is degraded with scaling factor which directly reduces the SNR, therefore the desensitization factor is:

SNRD=20log101-94a3a1∙AI2

Further the IIP3 of LNA can be expressed with:

AIIP3_LNA2=|a1||a3|∙43

Therefore, the SNR degradation can be formulated below:

SNRD=20log101-3∙AI2AIIP3LNA2=20log101-3∙PcwleakgePIIP3LNA

Assuming CW signal is transmitted by A-IoT BS or UE as reader and monostatic antenna configuration, with the return loss of 15 dB, the CW leakage power will be 18 dBm. If the SNR degradation is allowed with 3 dB, the IIP3 of LNA should be 28 dBm. Such high IIP3 is demanding for design. For other combination value for SNR degradation and CW leakage power, it is illustrated in Figure 6.7.5.2.1-2 and the IIP3 value is listed in Table 6.7.5.2.1-1. Referring to TR 38.858, the reported IIP3 for WA BS is from the range of 10 dBm to -27.6 dBm, this in turn demands some CW cancellation capability before LNA.

Table 6.7.5.2.1-1:  IIP3 of LNA with different SNR degradation and CW leakage power

|  |  | SNR degradation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | -1 | -3 | -10 |
| CW leakage power (dBm) | 18 | 32 | 28 | 24 |
|  | 8 | 22 | 18 | 14 |
|  | 0 | 14 | 10 | 6 |
|  | -10 | 4 | 0 | -4 |

Figure 6.7.5.2.1-2: IIP3 of LNA with different SNR degradation and CW leakage power

For CW cancellation, there are passive or active cancellation schemes existing for RFID reader front-end. In both schemes, a copy of the CW leakage signal is created with same amplitude but opposite phase. The cancellation scheme is illustrated in Figure 6.7.5.2.1-3. For the passive cancellation scheme, the cancellation path needs to be calibrated and for active cancellation scheme, the cancellation path can be tuned to adapt to the leakage CW signal. However, the cancellation is limited by either the hardware imperfection or the detecting signal amplitude/phase accuracy. In [6], it is mentioned that for 40 dB CW cancellation, the amplitude error should be 0.09 dB and phase error to be 0.5 degree. This is quite demanding in design considering the current RRM measurement accuracy for RSRP is +/-2 dB in normal condition.

When the CW and A-IoT signal are transmitted from different hardware, the active CW cancellation scheme should be used in this case. This applies to the case where the CW node is co-located with A-IoT BS for D1T1-A2 or D2T2-A2.

To enable the CW cancellation function, reader needs some time to train the circuit to generate the reverse CW leakage with the same amplitude and the opposite phase. The cancellation training time is implementation specific and also relates to the targeted cancellation performance. Considering this time and also the transient time to transmit the CW signal, the timing relation between the R2D, CW and D2R can be illustrated in Figure 6.7.5.1-4.

Figure 6.7.5.1-3: CW cancellation scheme

Figure 6.7.5.1-4: Transient time for CW and R2D transmission and CW cancellation training time before D2R reception

#### 6.7.5.3	Summary

CW cancellation performance is implementation specific. For RF CW cancellation performance, it is trade-off among the allowed receiver desensitization, receiver linearity and the CW leakage power received at antenna. To enable the CW cancellation, additional CW cancellation training time should be considered before D2R reception.

# Conclusions

In this contribution, we present our overview on the A-IoT CW cancellation issues with below observations and proposal:

Proposal-1: CW cancellation preparation time needs to be considered before D2R reception.

# References

- Lu Q, Qin H, He F, Zhang Y, Wang Q, Meng J. Wideband Interference Cancellation System Based on a Fast and Robust LMS Algorithm. Sensors (Basel). 2023 Sep 13;23

- Liu, J.; Ding, T. Digital Self-Interference Cancellation for Full-Duplex Systems Based on CNN and GRU. Electronics 2024, 13, 3041.

# Annex

3GPP TSG-RAN WG4 Meeting # 113						R4-241xyz

Orlando, US,18th–22nd November 2024

Title:	LS on D2R transmission timing

Response to:

Release:	Rel-19

Work Item:	FS_Ambient_IoT_solutions

Source:	RAN WG4

To:	RAN WG1/WG2

Cc:

Contact Person:

Name:	Chunhui Zhang

E-mail Address:	Chunhui (dot) Zhang (at) ericsson (dot) com

Attachments:

1. Overall Description:

RAN4 identify that when CW cancellation is enabled at reader, there is a need to consider the CW cancellation training time before D2R receiving after CW transmission. Reader may fail the D2R reception for early D2R transmission because CW cancellation needs a minimum training time to reach the full cancellation performance.

Additionally, RAN4 identify that there is a transient time between R2D and CW transmission. For device using the CW for energy harvesting, such transient time may impact the device power hold-up time.

RAN4 kindly asks RAN1 to consider above information in protocol design.

2. Actions:

To RAN WG1/WG2 group.

ACTION: 	RAN4 would like RAN1 consider the information above in protocol design.

3. References:

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #114		2025- 02‑17 to 205- 02- 21			Athens

TSG RAN WG4 Meeting #114-bis 	2025‑04‑07 to 2025‑04‑11				China