# 3GPP TSG-RAN WG4 Meeting # 113						R4-2419482

# Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			BS RF requirement overview for LP-WUS

Agenda item:		7.23.3

Document for:		Approval

# Introduction

In this paper, we provide our view on BS RF requirement impact.

# Discussion

In WF[1], the different aspects of the RF impact of the LP-WUS signal are listed and we share our view on these issues in below discussion.

#### Issue 1-2: Concept of LP-WUS power boosting

- WF

  - Compare the Pros and Cons of the two definitions

  - At least the following aspects should be considered

    - Convenience for manufacturer’s declaration

    - Compatibility with other issues relevant to power boosting, e.g. the possible upper bound, applicable lowest CBW, etc.

    - Cases of power boosting with different CBW and LP-WUS / NR RE allocation should be provided

    - Comparison of power boosting with NB-IoT with the same definition should be provided

    - Test aspects of power boosting should be considered

    - Other aspects are not precluded

  - Try to define the power boosting concept next RAN4 meeting with one definition as baseline but allow flexibility to manufacturer declaration of boosting level for the supported CBW

Firstly, for EPRE concept of boosting, the equation should be modified as one reference in [1] may need some improvement.

Assume the average power of a carrier in each RB is Pav in linear scale, and the total number RB is NfullRB within the channel bandwidth, then the total power of the carrier Pcarrier in linear will be

Pcarrier= Pav∙NfullRB

Further assume the RB to be boosted is NRB_WUS which corresponding to the LP-WUS signal and assume the power of the each RB is Pboost, then total power of the boosted LP-WUS signal in linear will be

PWUS= Pboost∙NRB_WUS

The remaining power left to transmit the other NR signal will be

Pcarrier-PWUS= Pav∙NfullRB-Pboost∙NRB_WUS

Assume the boost power is x dB relative to average power of Pav in one RB, we have

PboostPav = 10x10

Therefore, the remaining power left to transmit the other NR signal will be

Pcarrier-PWUS= Pav∙(NfullRB-10x10 ∙NRBWUS)

Futher assume the average of the NR signal power in one RB is Pdeboosting_NR, and we have

Pdeboosting_NR∙(NfullRB-NRBWUS)=Pcarrier-Pboost= Pav∙(NfullRB-10x10 ∙NRBWUS)

Therefore, the NR signal average power in linear is:

Pdeboosting_NR= PavNfullRB-10x10 ∙NRBWUSNfullRB-NRBWUS

The EPRE between WUS RB and NR RB in dB scale is

y=10∙log10PboostPdeboostingNR=10∙log10Pav∙10x10PavNfullRB-10x10 ∙NRBWUSNfullRB-NRBWUS=10*log1010x10 ∙(NfullRB-NRBWUS) NfullRB-10x10 ∙NRBWUS= x+10*log10NfullRB-NRBWUSNfullRB-10x10 ∙NRBWUS

From the above equation, one can observe that EPRE will change when the x is fixed with different channel bandwidth. The power boosting x should be fixed for a cell because the LP-WUS coverage should not be fixed no matter the network transmit WUS or NR at the same time or only transmit the WUS signal without NR.

EPRE is not fixed for different bandwidth when the power boosting factor is fixed for each of bandwidth.

the above NR signal power calculation when NR and WUS signal is transmitted at the same time is mainly for the testing purpose, it should be up to the network to implement whether or not to deboost the NR signal when simulateneous transmit the WUS signal or not.  The reason is that for a UE at the cell edge, network should not deboost the transmission power otherwise the throughput will be impacted. If the LP-WUS is boosted and at the same time, the remaining NR signal not deboosted, the total transmitted power for this carrier may exceed the rated carrier power and this make the specification broken.

Whether to de-boost NR signal transmission power should be up to network implementation.

The NR de-boosting transmission power is for the testing purpose only.

From the course of discussion on the boosting concept, it is not clarified that the WUS boosting power should be fixed or changing depending on the NR signal transmission. RAN4 should align the view on the configured power boosting WUS signal, which if manufacture declare the boosting power of x dB relative to average transmission power, base station should deliver the WUS signal power with the x dB power boosting. Such should be guaranteed no matter the NR signal is transmitted or not.  Then when it comes to the NR signal, it is the same situation, usually there should not be any de-boosting the transmission NR signals because the dynamic range transmission requirement mandate the NR signal transmission already.

Align the RAN4 view on the power boosting of WUS signal with and without the NR signal at the same time:

- The WUS signal should be transmitted always with x dB above the average power

- There is no NR signal de-boosting in normal transmission.

if the above can be aligned in RAN4, there is no need to define the EPRE as it is not allowed to de-boost the NR transmission signal.

One more aspect is good to align also which for option 2, the average power over all REs should be fixed when a cell is configured. There should no be power pooling concept relating to the dynamic range power definition.

    - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

The average power over all REs should be fixed.

Use option 2 in issue 1-2.

Though some companies want to boost the WUS signal up to 6 dB, it is difficult to do so with a closer look on the constellation diagram of a LP-WUS signal in Figure 6. As a matter of fact, to boost the 3 dB of a LP-WUS signal, this can only be limited to OOK1 where the QPSK random data can be mapped to the subcarriers within the LP-WUS signal. Referring the Figure 6,  for a OOK4 signal, the constellation diagram shows some of the points are very close and in our opinion, such a dense constellation may be comparable with a 64QAM or higher modulation 256QAM. The RF requirement is discussed later for OOK4, but for the discussion of the power boosting, we think there should be no power boosting for OOK4 and it will be up to discussion whether the power boosting will be treated the same for OOK1 and OOK4, e.g OOKm in general and m = 1 for OOK1 and m=4 for OOK4.

No power boosting for OOK4.

Limit the power boosting discussion only for OOK1.

Regarding the declaration or not, such discussion can be referred to the test model TM1.2 and this is quoted as below. To understand how the boosted signal is placed, we can take an example:

First dertermin the P using below, assume the NRB=52, the NBWPsize=49, using the configuration 1 in Table 5.1.2.2.1-1 from TS 38.214, we have P =4.

Table 5.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 | Configuration 3 |
| --- | --- | --- | --- |
| 1 – 36 | 2 | 4 | 8 |
| 37 – 72 | 4 | 8 | 16 |
| 73 – 144 | 8 | 16 | 32 |
| 145 – 275 | 16 | 16 | 32 |

Then NRBG=min0.4*49P,  0.5*49+3-44-49+3-44mod2+1=min4, 6+1=4

The location of the boosted RB is RBG#12, RBG#1, RBG#3 and RBG#5

And the boosted RB location is illustrated in Figure 1.

Figure 1: The boosted RB location witin 52 RB grid (10MHz and 15kHz SCS)

When the channel BW is greater than 60 MHz and the RB size for 30kHz is 162, the RGB size P is 16, which is greater than WUS RB size of 11 RB for 30kHz SCS case.

For example, for 60MHz channel bandwidth, the N_RB = 162 for 30kHz SCS, then P=16

NRBG=min0.4*16216,  0.5*159+3-416-159+3-416mod2+1=min4, 4+1=4

The location of the boosted RB is RBG#9, RBG#1, RBG#3 and RBG#5

Figure 2: The boosted RB location witin 162 RB grid (60MHz and 30kHz SCS)

It can be observed that for bandwidth greater than 60MHz, the TM1.2 may be fine to refer to when manufacture declare the LP-WUS power boosting of 3 dB. For smaller bandwidth where RBG is less than 11 RB for 30kHz, a new test model may be needed.

TM1.2 can be referred to for LP-WUS power boosting for wider bandwidth but not for narrow bandwidth.

Define a new test model for power boosting of LP-WUS signal in conformance testing phase.

##### 4.9.2.2.2	FR1 test model 1.2 (NR-FR1-TM1.2)

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

Common physical channel parameters are defined in clause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM1.2 are defined in table 4.9.2.2.2-1.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | NRBG=minxNRB-3P,12NRB-3+3modP-PP-NRB-3+3modP-PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB-3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of PDSCH RBGs which are boosted | NRB-3+3modPP-1 and if NRBG>1, 1, 3, …, 2NRBG-2+1 |
| Level of boosting (dB) | 3 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-3-PNRBG |
| Locations of PDSCH RBGs which are deboosted | Starting at RB#3 and excluding PRBs of RBGs which are boosted |
| Level of deboosting (dB) | 10log10NRB-3-103/10PNRBGNRB-3-PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

%%

Manufacture declaration on LP-WUS power boosting

There will be new test model and test case defined to test the power boosting. The unwanted emission and ACLR should be tested for this purpose. Manufacture can declare the power boosting capability and test it. This should be similar with the NB-IoT and the relevant declaration is listed as below.

The power boosting of LP-WUS should be manufacture declared.

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.41 | NB-IoT operation | Manufacturer shall declare the support of NB-IoT operation in NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. | x |  |
| D.42 | NB-IoT sub-carrier spacing | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare if it supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both for NPUSCH. | x |  |
| D.43 | NB-IoT power dynamic range | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.104 [2] (Note 5). | x |  |

Issue 1-3: Whether to preclude small CBW for consideration of LP-WUS power boosting

For narrow BW of LP-WUS, where the bandwidth is less or equal to LP-WUS bandwidth, boosting the LP-WUS means that the carrier power will be boosted. Though the eMBB coverage is not impacted, it may not be possible for hardware if the BS configure only one carrier and transmit the maximum declared rated power. If there will be multiple-carrier configured at the same time, boosting one carrier power means the power will be stolen from other carriers. This is not testable as power pooling depend on traffic from other cells. So it may be doable, but this will increase the implementation complexity. From this perspective, there is no need to declare the power boosting for narrow bandwidth.

Exclude the narrow bandwidth (less and equal than 5 MHz ) for power boosting of LP-WUS

Manufacture declare the min channel bandwidth to support the power boosting of LP-WUS.

# Sub-topic 1-2 Other considerations apart from power boosting for LP-WUS

As LP-WUS signal is new physical channel introduced in NR, our view is that the legacy RF requirement shall apply when LP-WUS signal is transmitted. As the OOK-m signal is synthesized with OFDM transmitter, such OOK waveform can only be identified at OOK WUR receiver with a channel bandwidth that equals to LP-WUS bandwidth. From transmitter perspective, the OFDM waveform is transmitted and therefore, RAN4 should further discuss what would be the difference when OOKm signal is generated comparing to the OFDM waveform and identify further RF impact if any.

In RAN1#118bis, the below working assumption is agreed:

Working Assumption

For overlaid OFDM sequences for LP-WUS in time or frequency domain, down-select between following two options for OOK-1 and OOK-4 M=1

- Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1.

  - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation

    - It doesn’t preclude additional operation for OOK-4 with M>1.

    - DFT size is 2^n

Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

This working assumption means the time sequence for overlaid OFDM symbol is ZC sequence. This applies both for OOK-1 and OOK-4. RAN1 also agrees on the processing for the OOK signal and this is illustrated in Figure 3.

Figure 3: OOK-4 signal processing and subcarrier mapping before IFFT

In our view, the RF impact analysis should be separated for OOK-1 and OOK-4 (m> 1). This is because we see the potential that no new RF requirement could be required for OOK-1 while more discussion will be needed for OOK-4 (m>1).

For OOK-1, as illustrated in Figure 4, the OOK bit 1 can be mapped to one OFDM symbol with the overlaid ZC sequence in this symbol and OOK bit 0 can be mapped to another OFDM symbol where no allocation within the LP-WUS RBs and therefore no energy would be detected by WUR. Following the RAN1 signal processing in Figure 3, the time domain ZC sequence can be generated in frequency domain and still it is ZC sequence illustrated in Figure 4.  For the frequence domain mapping of a ZC sequence before IFFT, this is the same with LTE PSS generation and there is no corresponding requirement or test for ZC sequence “modulation” in TS 36.104.  Our view is that there is no RF requirement needed for for OOK-1 by following the LTE specification on this aspect. The ZC sequence generation is quoted below from TS. 36.211.

ZC sequence in frequency domain generation is specified for LTE PSS signal

No RF requirement defined for LTE PSS signal.

No new RF requirements is identified for OOK-1.

### 6.11.1	Primary synchronization signal (PSS)

#### 6.11.1.1	Sequence generation

The sequence  used for the primary synchronization signal is generated from a frequency-domain Zadoff-Chu sequence according to

Figure 4: OOK1 subcarrier mapping

Figure 5: ZC sequence in time and frequence domain.

- With OOK-4 (DFT-s or waveform fitting approaches) the gNB may need to support additional DFT (or another transform for LS approach) or pre-generate and store frequency domain symbols. This is illustrated in Figure 6.

Figure 6: OOK-4 subcarrier mapping

- After the DFT transform, the OOK-4 frequency domain pattern is illustrated in Figure 7. Such constellation pattern does not match any existing modulation schemes.  The measurement point for signal quality is defined after FFT according to Figure B.1-1 in TS 38.104. If this measurement point would be kept for signal quality measurement of OOK-4, RAN4 should discuss if a test with frequency domain data generated with a fix time domain OOK-4 pattern using procedure defined by RAN1 should be defined.

RAN4 discuss whether to define a frequency domain testing data for OOK-4 if the measurement point for signal quality would be kept.

According the current WID objective, the below objective still in line with the spirit of the study phase and keep the RF impact as min as possible.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Figure 7: ZC sequence in frequency domain after DFT processing before IFFT

Figure B.1-1 [3]: Reference point for EVM measurement

- Considering WID objective and keeping the min specification impact for OOK-4, we suggest to testing the tolerance of the distortion of the frequency domain data of OOK-4 by remapping the frequency constellation of overlaid ZC sequence to one of existing modulation scheme.  Mapping the generated frequency domain values (before IFFT) to existing sequences/QAM modulations could be considered to reduce the impact on gNB. As one example, for OOK4 waveform, the higher modulation (e.g 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 6) to 64QAM constellation points (right constellation in  Figure 6) for CP-OFDM symbols as illustrated in Figure 6. As Such, gNB can not boost the LP-WUS signal according to the current base station RE dynamic range requirement.  Figure 7 and Figure 8 illustrates the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping, It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

The performance loss for LP-WUS signal detection at WUR due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

From the constellation re-mapping, it can be observed that the time domain signal quality of OOK-4 is quite robust and can be maintained without big degradation of SNR when the frequency domain OOK-4 data is distorted by the amount of EVM equivalent to the 64QAM. The frequency domain data of OOK-4 is degraded if it is distorted by the amount of the EVM equivalent to the 16QAM.

The time domain OOK-4 signal can tolerate a distortion in frequency domain equivalent to and less than the EVM of 64QM before iFFT.

RAN4 discuss whether to introduce the “EVM” requirement in frequency domain for a OOK-4 signal quality measurement.

Figure 8:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 9: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 10: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

For the other requirement than the dynamic range and potential EVM requirement for OOK-4, we think no new requirement is identified for LP-WUS signal.

No other new RF requirement than the dynamic range or potential EVM requirement for OOK-4 is identified for LP-WUS.

# Conclusions

In this contribution, we present our view on the BS RF requirement with below observations and proposals:

Observation 1 EPRE is not fixed for different bandwidth when the power boosting factor is fixed for each of bandwidth.

Observation 2 Whether to de-boost NR signal transmission power should be up to network implementation.

Observation 3 The NR de-boosting transmission power is for the testing purpose only.

Proposal-1: Align the RAN4 view on the power boosting of WUS signal with and without the NR signal at the same time:

Proposal-2: The average power over all REs should be fixed.

Proposal-3: Use option 2 in issue 1-2.

Proposal-4: No power boosting for OOK4.

Proposal-5: Limit the power boosting discussion only for OOK1.

Observation 4 TM1.2 can be referred to for LP-WUS power boosting for wider bandwidth but not for narrow bandwidth.

Proposal-6: Define a new test model for power boosting of LP-WUS signal in conformance testing phase.

Proposal-7: The power boosting of LP-WUS should be manufacture declared.

Proposal-8: Exclude the narrow bandwidth (less and equal than 5 MHz ) for power boosting of LP-WUS

Proposal-9: Manufacture declare the min channel bandwidth to support the power boosting of LP-WUS.

Observation 5 ZC sequence in frequency domain generation is specified for LTE PSS signal

Observation 6 No RF requirement defined for LTE PSS signal.

Proposal-10: No new RF requirements is identified for OOK-1.

Observation 7 RAN4 discuss whether to define a frequency domain testing data for OOK-4 if the measurement point for signal quality would be kept.

Observation 8 WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Observation 9 The performance loss for LP-WUS signal detection at WUR due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

Observation 10 The time domain OOK-4 signal can tolerate a distortion in frequency domain equivalent to and less than the EVM of 64QM before iFFT.

Proposal-11: RAN4 discuss whether to introduce the “EVM” requirement in frequency domain for a OOK-4 signal quality measurement.

Proposal-12: No other new RF requirement than the dynamic range or potential EVM requirement for OOK-4 is identified for LP-WUS.

# References

- R4-2416589, Way Forward for [112bis][308] NR_LPWUS, Huawei

- 38.141-1, Base Station (BS) conformance testing Part 1: Conducted conformance testing

- 38.104