3GPP TSG-RAN WG4 Meeting # 113						R4-2419483

Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			On general issues for WUR

Agenda item:		7.23.1

Document for:		Approval

# Introduction

In this paper, we present our overview on LS on  LP-WUS operation in IDLE/INACTIVE mode.

# Discussion

In [1], RAN1 ask RAN4 to check if there is any issue on below statements:

“In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE.

RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.”

Though the question only relates only idle/inactive mode, from hardware perspective, it will be the same for both connected and idle/inactive mode.

From RAN4 perspective, when one feature is identified to have separate test for RF requirement, such feature usually will be defined as per band and/or band combination associated with UE reporting, as such it is common for this feature only supported by a subset of the bands which are supported by UE. RAN4 will specify the WUR receiver requirement for all the bands so there is no issue for the specification support relating to this.

However, as there is either OFDM WUR and OOK WUR, the common understanding in RAN1 may be true for  OOK WUR as new hardware is needed in this case.  For OFDM WUR, it can share the main receiver radio hardware while the baseband processing will be same for any bands. Therefore, if UE support OFDM WUR, it should be possible to support all bands supported by the UE. Above all, WUR is assuming one Rx branch. For main receiver which has multiple Rx branch, such implementation can save power which is main WUR feature about. Therefore, it should be discussed in RAN4, if OFDM WUR should be per UE if UE would support this optional feature.

When UE support OFDM WUR, this should be per UE feature instead of per band.

# Conclusions

In this contribution, we present our overview on LS on  LP-WUS operation in IDLE/INACTIVE mode with below proposal:

Proposal-1:When UE support OFDM WUR, this should be per UE feature instead of per band.

# References

- R1-2407559, LS on LP-WUS operation in IDLE/INACTIVE mode, Apple