# 3GPP TSG-RAN WG4 Meeting # 113						R4-2419487

# Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			TP for WUR RF requirement of REFSESN, ASC, ASCS

Agenda item:		7.23.2.2

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of REFSENS, ASC, ASCS.

# Discussion

### REFSENS

In WF [2], it is agreed that RAN4 will study the SNR target used for REFSENS calculation. The Channel model is AWGN and no repetition is used.

Issue 2-1-1: Alignment of FR1 SNR calculation/definition in simulation

Agreements:

  - SNR is defined as the ratio of average energy of the LP-WUS signal per sub carrier over the average energy of Noise per sub carrier, over all the LP-WUS RBs.

    - Consider the sub-carriers allocated to LP-WUS excluding Guard RBs.

Issue 2-1-3: Target FR1 SNR value for LP-WUS/WUR

Agreements:

  - RAN4 can further discuss target SNR for OOK-based and OFDM-based receivers.

For OOK WUR, our opinion is that the ASCS LLS simulation result can be referred to derive the SNR target and no need to redo the LLS. The key parameter for ASCS is the filter order and the guard RB. It can be observed that when guard RB greater than 1, the SNR threshold @ 1% MDR has 0.5 dB difference between filter order of 3 and 5 for OOK1 and 1 dB difference between filter order of 3 and 5 for OOK4. Table 1 lists the SNR points for different configuration according to Figure 2 and Figure 3.

Table 1: SNR target for OOK1 and OOK4 with different filter order

|  | OOK1 | OOK4 |
| --- | --- | --- |
| Filter order 3 | -5 dB | - 2.5 dB |
| Filter order 5 | -4.5 dB | -1.5 dB |

For OFDM WUR, the 1% MSD is simulated, and the detection performance is illustrated in Figure 1. It can be observed that the SNR target is around -15 dB for AWGN channel with 4 OFDM symbols which is quite low compared to the OOK WUR SNR point.

Figure 1: OFDM sequence detection performance

Table 2: WUS resources configuration (4.32 MHz WUS bandwidth).

WUS payload (modulation)

1bit (OFDM, SSS-based time-correlation)

1 sequence of word payload

1/4 symbols for overlaid OFDM duration

The SNR for OFDM WUR detection is -3.2 dB for AWGN with 1 Zadoff Chu sequence and 1 symbol of OFDM symbol.

The SNR for OOK WUR detection depends on the filter order OOK1 or OOK4 with range of -5 dB to -1.5 dB.

As the SNR for OOK WUR and OFDM are different in general, RAN4 can try to reach consensus on each of them and if the SNRs are same in the end, the same SNR can be used for both OOK WUR and OFDM WUR.

RAN4 reach the consensus on SNR of OFDM WUR and OOK WUR before discussing whether the same or different SNR to be used in REFSENS.

For NF, last meeting below

Issue 2-2-1: Baseline architecture for FR1 OFDM-based LP-WUS

Agreements:

  - use zero-IF receiver as a baseline RF architecture for OFDM based LP-WUR.

Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR

Agreements:

  - Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed.

  - Encourage companies to also input the justification for each element.

For the NF, in TR 38.869, the NF range is between 10 to 16 as listed in Table 7.1.1a-3 in study phase.  In work item phase, RAN4 should further reduce the NF range and reach consensus based on the practical design.

Table 7.1.1a-3: Relative power consumption and noise figure for OOK-1/2/4 with homodyne/zero-IF architecture with baseband envelope detection

| Source reference | [7A-1] | [7A-2] | [7A-3] | [7A-4] | [7A-5] | [7A-6] | [7A-7] | [7A-8] | [7A-9] | [7A-10] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.09 for single-branch, 0.01 or 0.02 for each additional branch | 0.5 | 0.1~1 | 0.1 | 0.05~ 0.5 | 0.5~1 | 0.1~0.5 | 4 | ~1 | 0.1~0.5 |
| Noise figure (dB) | 15 | 10~15 | [10-16] | 15 | 12 | 15 | 12~15 | 15 | ~15 | 12 |

From pervious RAN4 agreements, the zero-IF is agreed as RF architecture for both OOK WUR and OFDM WUR. In TR 38.869, the RF architecture of the zero-IF is quoted below:

Figure 7.1.1-3: Homodyne/zero-IF architecture with baseband envelope detection based LP-WUR diagram

The NF could be derived by choosing a typical value of the component specification using the Friis cascading equation. As the WUR is not expected to be operated simultaneously with transmitter for FDD band, the insertion loss before LNA could be comparable with TDD or a RedCap UE with HD-FDD mode operation. This means a SAW filter may be used instead of the duplex filter. In Table 3, we use the component specification from [3] with the addition of the LPF NF and also the modification of the insertion loss before the LNA and referencing to [4].

Table 3: Friis cascading equation for WUR homodyne architecture

| Parts | Gain (dB) | NF (dB) | Cascade NF (dB) | Comments |
| --- | --- | --- | --- | --- |
| RF BPF &Matching network | -2.5 | 2.5 | 12.71 | The B3 Saw filter insertion loss of 2.5 dB [4] |
| RF LNA | 15 | 7 | 10.19 |  |
| Mixer | 3 | 10 | 22.38 |  |
| BB AMP | 40 | 25 | 25.14 |  |
| BB LPF | 0 | 50 | 50 |  |
| Total | 56.5 |  | 12.71 |  |

The NF of OOK/OFDM WUR should be 12 ~ 13 dB consider the insertion loss of around 2.5 dB

### ASCS

From the LLS simulation, when filter order is 3rd or 5th. The BLER performance is illustrated in Figure 2 to Figure 5. It shows that larger guard RB size has penalty on the SNR threshold due to less received energy. Such observation holds for both AWGN and TDL-C channels and for OOK1 and OOK4. Because of this, the guard RB of less or equal than 1 RB at each side of WUS signal is good enough. The guard RB of 1 corresponds to the case where 11 RB WUS is configured and LP-WUS signal is confined within a 5MHz frequency block (13 RB in total within 5MHz block).

In Figure 6, it is illustrated that without guard RB, there is 0.5 dB SNR degradation compared with 1 guard RB at each side of WUS signal (WUS signal still 11 RB) for filter order of 3. This means that from testing perspective, half dB relaxation will be needed if there is no guard RB between NR signal and LP-WUS signal.

Larger guard RB size has penalty on the SNR threshold due to less received energy

Maximum of 1 guard RB at each side of WUS signal should be used in ASCS case.

The simulation case for the parameter setting is below.

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | 8/16 bits payload |  |  |
| Chip rate | M=1 | M=1, 2 or 4 |  |
| WUS duration | 16 OFDM symbols |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 1/1.5/2 RB on each side of LP-WUS bandwidth |  |  |
| Filter | - 3rd/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| Wanted signal level | Same PSD with NR signal |  |  |
| Sampling rate | 7.68MHz |  |  |
| Phase noise | Not modelled |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER - The following false alarm rate is used - 0.1% |  |  |

Figure 2: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1, 1.5 and 2) with AWGN channel and OOK1

Figure 3: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1, 1.5 and 2) with AWGN channel and OOK4

Figure 4: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1, 1.5 and 2) with TDL-c channel and OOK1

Figure 5:BLER for ASCS with different LPF order (3 and 5) and guardRB size (1, 1.5 and 2) with TDL-c channel and OOK4

Figure 6: BLER for ASCS with LPF order 3 and guardRB size (0 and 1) for OOK1

### ACS

The simulation assumption from the table below is used.

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8 bits |  |  |
| Chip rate | M=1 | M=2 |  |
| WUS duration | FFS |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3rd/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = 33dB NOTE: decide the interference level depending on SNR |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS |  |  |
| Phase noise | Not modelled |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Phase noise | Phase noise profile. Phase noise (dBc/Hz). Offset; NF 9 dB; NF 12 dB; NF 15 dB. @5MHz; -99.4; -99.4; -99.4. @10MHz; -108.1; -105; -102.1. @15MHz; -120; -117; -114. |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER - The following false alarm rate can be considered - 0.1% |  |  |

According to the WF[2], the phase noise can be considered in the LLS. In LLS simulation, the ACS cases are simulated with ACI =33/30with different shifted RB and filter order. For OOK1 waveform, it is illustrated in Figure 7 that there is a need of 3 shifted RB to combat the phase noise for filter order of 3. In Figure 8, it can be seen that if ACI=30 is used, there is only SNR increase when the phase noise is added and in this case, there is no need of the shifted RB.

For OOK4, in Figure 9 it can be seen that filter order of 3 and 4 shifted RB cannot reach 1% BLER floor but increase the filter order to 5 and use 4 shifted RB can reach the 1% BLER for ACI = 33. In Figure 10, it can be observed that the shifted RB needs to be 2 only when ACI is relaxed to 30.

For OOK1, there is a need of 3 shifted RB for filter order of 3 and ACI=33. No shifted RB is needed if ACI is relaxed to 30.

For OOK4, the ACI needs to be relaxed to 30 and 3 shifted RB will be needed for filter order of 3.

Figure 7: The BLER for ACI=33 with LPF order of 3, OOK1 and phase noise ON/OFF

Figure 8:The BLER for ACI=30 with LPF order of 3 , OOK1 and phase noise ON/OFF

Figure 9:The BLER for ACI=33 with LPF order of 3 & 5, OOK4 and phase noise ON/OFF

Figure 10: The BLER for ACI=30 with LPF order of 3, shift RB number of 0/1/2 ,  OOK4 and phase noise ON/OFF

There are two test cases for ACS, case #1 and case #2.  For case #1, the ACS interferer is set relative to the wanted signal with the power difference of the selectivity value. From the LLS simulation, WUR can achieve the same selectivity with the MR for OOK1. For OOK4, ACI needs to be relaxed with 3 dB to fit the lower filter order of 3.  As such, the WUR ACS can be tested with the same selectivity with MR if the test waveform is OOK1.

In LLS, the ACS interferer is set to ACS value relative to the WUR received signal. If the OOK1 is to be used, the ACS interference can be set relative to LP-WUS signal.  As the WUR has a higher REFSENS than REFSENS of MR, this means that the ACS interferer could be higher than ACS interfere of the MR.

Another approach is that ACS value is the attenuation between the ACS interference set by MR and the WUR REFSENS plus 14 dB. In this way, the ACS would be relaxed as the REFSENS of WUR is higher than REFSENS of MR. This approach can be used if OOK4 waveform would be tested instead of OOK1.

Specify the same ACS selectivity for WUR with MR for OOK1. The ACS interferer can be set relative to the WUR REFSENS plus 14 dB plus ACS value.

Specify the ACS selectivity with 3 dB relaxation compared to the ACS of MR if testing the OOK4. The ACS interferer can be set the same with MR and set relative to the WUR REFSENS plus 14 dB plus the ACS value.

# Conclusions

In this contribution, we present our view on WUR RF requirement for REFSENS, ASCS and ACS with below proposal and observations:

Observation 1 The SNR for OFDM WUR detection is -3.2 dB for AWGN with 1 Zadoff Chu sequence and 1 symbol of OFDM symbol.

Observation 2 The SNR for OOK WUR detection depends on the filter order OOK1 or OOK4 with range of -5 dB to -1.5 dB.

Observation 3 Larger guard RB size has penalty on the SNR threshold due to less received energy

Proposal-1: RAN4 reach the consensus on SNR of OFDM WUR and OOK WUR before discussing whether the same or different SNR to be used in REFSENS.

Proposal-2: The NF of OOK/OFDM WUR should be 12 ~ 13 dB consider the insertion loss of around 2.5 dB

Proposal-3: Maximum of 1 guard RB at each side of WUS signal should be used in ASCS case.

Observation 4 For OOK1, there is a need of 3 shifted RB for filter order of 3 and ACI=33. No shifted RB is needed if ACI is relaxed to 30.

Observation 5 For OOK4, the ACI needs to be relaxed to 30 and 3 shifted RB will be needed for filter order of 3.

Proposal-4: Specify the same ACS selectivity for WUR with MR for OOK1. The ACS interferer can be set relative to the WUR REFSENS plus 14 dB plus ACS value.

Proposal-5: Specify the ACS selectivity with 3 dB relaxation compared to the ACS of MR if testing the OOK4. The ACS interferer can be set the same with MR and set relative to the WUR REFSENS plus 14 dB plus the ACS value.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo etc

- R4-2417112, WF on LP-WUS UE RF requirements, vivo

- R4-2415707, Fundamental RX requirements for LP-WUR, Nokia

- R4-146904, RX filter insertion loss comparison between HD and FD, Sony