3GPP TSG-RAN WG4 Meeting # 113						R4-2419489

Orlando, US, 18th – 22nd  November, 2024

Source:		Ericsson

Title:			On in-band deployment of NTN NB-IoT in NR carrier

Agenda item:		5.9.1

Document for:		Approval

# Introduction

In this paper, we present our overview on the in-band deployment of NTN NB-IoT in NR carrier.

# Discussion

The in-band operation addition for NTN NB-IoT in a NR carrier was discussed in RAN4#112-bis and WF [2] listed the open issues for the IB (In-Band)/GB (Guard Band) operation.

Feedback from some companies last meeting indicates that the Rel-17 NTN NB-IoT has been deployed in the field and thus it would not be possible to add IB/GB operation for these devices. Therefore, mandating the IB/GB operation in Rel-17 would not be possible, and RAN4 should discuss from which release the IB/GB operation can be supported and what is the specification impact by introducing IB/GB mode.

Having Rel-17 NTN NB-IoT already deployed in field means IB/GB operation cannot be mandated from Rel-17.

Rel-18 for IoT NTN enhancement core requirement was completed in December 2023 and Rel-19 is ongoing. At least the IB/GB operation shall be supported in rel-19.

IB/GB operation should be supported at least from Rel-19.

In WF [2], whether the IB/GB operation should be mandatory feature or optional feature is not decided. As the Rel-18 limit the NTN NB-IoT to standalone operation, it would be good to confirm the UE behavior when it makes the initial access. In our understanding, a Rel-17 NTN NB-IoT UE will first check the MIB operationModeInfo and if the operation mode is standalone, it will continue to access the network, otherwise, the cell is barred for this UE.

Confirm that a Rel-17 NTN NB-IoT UE will be barred on a cell if the cell broadcast operation mode other than standalone mode.

An NTN NB-IoT UE when supporting the IB/GB/SA mode, it can access any NTN NB-IoT cell and as such the deployment mode from network appears to be agnostic to NTN NB-IoT UE so in our view, the IB/GB mode should be mandatory feature. As the barring cell for Rel-17 NTN NB-IoT UE is only because of the release 18 WID objective limitation, therefore, there is no other justified reason for later release UE to continue to support the SA mode only. For additional signaling, we do not think it is necessary. Such capability does not help the network to enable the IB/GB mode as SA-only NB-IoT UE will not initiate MSG1 at all (the cell is barred for such UE) and network has no use of signaling of new capability.

IB/GB support should be mandatory feature.

In TN NR coexisting with NB-IoT, the network can indicate the guardband mode to NB-IoT UE if the NB-IoT carrier is inside NR transmission bandwidth. This needs the network to blank the NR RB if there is conflicting between NB-IoT and NR.

Regarding the cross group impact, our understanding is that the legacy MIB NB field (e.g guardband mode) is good enough to indicate to NTN NB-IoT UE which operation mode is deployed and therefore, the UE can calculate differently for the F_DL of non-anchor carrier for SA mode and F_DL of non-anchor carrier for IB/GB mode.

It may be possible to modify the Inband-SamePCI or Inband-DifferentPCI so that a NTN NB-IoT UE is aware of the NR cell instead of the LTE cell,  as this will impact the other group, we think RAN4 can discuss whether the TN NR IB deployment of the NB-IoT should be used as baseline or not.

RAN4 discuss whether to use the TN NR IB mode deployment as baseline.

Legacy OperationModeInfo MIB should be used by NTN NB-IoT UE to differentiate different operation mode.

Network can indicate the guardband mode to NTN NB-IoT UE for IB mode deployment.

  - operationModeInfo

    - Deployment scenario (in-band/guard-band/standalone) and related information. See TS 36.211 [21] and TS 36.213 [23].

    - Inband-SamePCI indicates an in-band deployment and that the NB-IoT and LTE cell share the same physical cell id and have the same number of NRS and CRS ports.

    - Inband-DifferentPCI indicates an in-band deployment and that the NB-IoT and LTE cell have different physical cell id.

    - guardband indicates a guard-band deployment.

    - standalone indicates a standalone deployment.

No cross group impact to add IB/GB mode if TN NR coexisting with NB-IoT is baseline.

For the CR type B or C, in TR 21.900 it depend on whether it is a new feature or any functional modification. In our opinion, the IB/GB is a new feature and therefore type B should be used in a frozen release.

Type B should be used for a new feature of IB/GB operation for NTN NB-IoT UE.

Table 4A: Categories of Change Requests [3]

| Category | Meaning | Remarks |
| --- | --- | --- |
| A | Corresponds to a change to an earlier Release | Used to reflect functionally equivalent changes made to an earlier Release of the same Specification. NOTE: The proposed change to the later Release of the Specification need not be absolutely identical to the proposed change to the earlier Release, since it is possible that, due to earlier change requests, the affected text is not identical in each Release. Category A should be used when the functional objective of the proposed changes is equivalent in the earlier and later Releases. |
| B | Addition or deletion of feature | The new feature is to be added to the Release; the reference is not to the Specification itself. This will normally correspond to an identified Work Item. This category shall not be used for a frozen Release, except for alignment CRs as described in clause 4.7. |
| C | Functional modification of feature | Any functional modification shall correspond to an identified Work Item. However backward compatibility shall be ensured when the issue has an impact on the UE. This category shall not be used for a frozen Release, except for alignment CRs as described in clause 4.7. |
| D | Editorial modification | Editorial modifications shall have no impact on an implementation. An editorial modification CR to a frozen Release shall not be permitted. |
| E | (not used) |  |
| F | Correction | Used: 1 to correct an error in the specification (i.e. a clear instruction in the specification which leads to incorrect operation of the system); or 2 to correct an ambiguity in the specification which could lead to different implementations which cannot inter-operate; or 3 (void); or 4 to remedy the incorrect implementation of a previously approved CR; or 5 to correct a misalignment between the specifications (stage 1, stage 2 & stage 3) for a feature or service when not introducing a new function or functional change. |

For other specification impact, we think TS 36.307 in addition of the WF[2] once RAN4 reach consensus on which release the NTN NB-IoT UE should support from.

Table 3A.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 36.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RF, RRM and demodulation requirements for NB-IoT standalone operation over NTN | Rel-17 | Table F.1-1 for UE RF requirements, Table F.2-1 for RRM requirements Table F.3-1 for UE demodulation requirements | Rel-18 WI LTE_NBIoT_eMTC_NTN_req introduced RF, RRM and demodulation requirements for NB-IoT standalone operation over NTN with bands specified in Rel-18 TS 36.102. See tables F.1-1, F.2-1, F.3-1. |

- TS 36.307 needs to be added as one additional specification once RAN4 decides the release from which NTN NB-IoT support IB/GB operation.

If the rel-19 to be the first release to support IB/GB operation, our opinion is that a new WID is more appropriate to add this feature instead of the TEI.

A new WID is more appropriate if IB/GB support for an NTN NB-IoT UE starts from Rel-19.

# Conclusions

In this contribution, we present our overview on the  in-band deployment of NTN NB-IoT in NR carrier with below proposal and observations:

Observation 1 Having Rel-17 NTN NB-IoT already deployed in field means IB/GB operation cannot be mandated from Rel-17.

Observation 2 IB/GB operation should be supported at least from Rel-19.

Proposal-1: Confirm that a Rel-17 NTN NB-IoT UE will be barred on a cell if the cell broadcast operation mode other than standalone mode.

Proposal-2: IB/GB support should be mandatory feature.

Observation 3 Legacy OperationModeInfo MIB should be used by NTN NB-IoT UE to differentiate different operation mode.

Proposal-4: No cross group impact to add IB/GB mode if TN NR coexisting with NB-IoT is baseline.

Proposal-5: Type B should be used for a new feature of IB/GB operation for NTN NB-IoT UE.

Proposal-5:TS 36.307 needs to be added as one additional specification once RAN4 decides the release from which NTN NB-IoT support IB/GB operation.

Proposal-6: A new WID is more appropriate if IB/GB support for an NTN NB-IoT UE starts from Rel-19.

# References

- RP-223437, Revised WID on NB-IoT/eMTC core & performance requirements for NTN, MTK

- R4-2417184, WF for NTN NB-IoT inband operation, Ericsson

- TR 21.900