3GPP TSG-RAN WG4 Meeting #113	R4-2419504

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	5.9.1

Source:	MediaTek Inc.

Title:	Discussion on UE RF A-MPR for B254

Document for:	Approval

- Introduction

The WI for IoT-NTN B254 was closed and frozen. But there is still RF core requirement of A-MPR not finished yet. For 2023 November to now, there is no any response from ETSI and no further action from 3GPP. In this contribution, the further views on specifying B254 A-MPR are provided.

- Discussion

2.1		Missing A-MPR for B254

The n254 RF core requirements to meet ETSI requirements were all specified.

Observation 1: The NR-NTN n254 RF core requirements to meet ETSI requirements were all specified.

Observation 2: The IoT-NTN B254 RF core requirements of A-MPR is TBD from 2023/Nov to today.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 | TBD |  |
| NS_05N | 6.5B.4.4.6 | 254 | TBD |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |

Observation 3: Actually, A-MPR values have been proposed for B254, but no consensus has been reached to capture them into TS. Thus, these A-MPR values are only captured in TR.

To further evaluate the 1-tone or 3-tones ACLR1 value at inner-spectrum allocation. It is observed that 1-tone or 3-tones ACLR1 value could be lower than ETSI in-band emission requirement of -5dBm/30kHz.

Figure 1: 3 15kHz tones at edge-spectrum allocation

Figure 2: 3 15kHz tones at inner-spectrum allocation

Figure 3: Single 15kHz tone at inner-spectrum allocation

Observation 4: Based on the ACLR1 evaluation for 1 tone or 3 tones at inner-spectrum allocation, it is observed that 1-tone or 3 tones ACLR1 value could be lower than ETSI in-band emission requirement of -5dBm/30kHz.

Proposal 1: To agree the proposed A-MPR values as indicated in CR.

Proposal 2: If there is any different result to indicate that ETSI in-band emission requirement of -5dBm/30KHz could not be met for NB-IoT UE when enabling 1 tone or 3 tones at inner-spectrum allocation, then the UE ACLR1 performance needs to be checked for 1-tone and 3-tones inner-spectrum allocation.

3 Conclusion

In this paper, our views on views specifying B254 A-MPR are provided.

Observation 1: The NR-NTN n254 RF core requirements to meet ETSI requirements were all specified.

Observation 2: The IoT-NTN B254 RF core requirements of A-MPR is TBD from 2023/Nov to today.

Observation 3: Actually, A-MPR values have been proposed for B254, but no consensus has been reached to capture them into TS. Thus, these A-MPR values are only captured in TR.

Observation 4: Based on the ACLR1 evaluation for 1 tone or 3 tones at inner-spectrum allocation, it is observed that 1-tone or 3 tones ACLR1 value could be lower than ETSI in-band emission requirement of -5dBm/30kHz.

Proposal 1: To agree the proposed A-MPR values as indicated in CR [1].

Proposal 2: If there would be any different result to indicate that ETSI in-band emission requirement of -5dBm/30KHz could not be met for NB-IoT UE when enabling 1 tone or 3 tones at inner-spectrum allocation, then the UE ACLR1 performance needs to be checked for 1-tone and 3-tones inner-spectrum allocation.

5 Reference

- R4-2419505, “(IoT_NTN_FDD_LS_band) CR to TS 36.102 addressing incomplete Rel-18 requirements in clause 6.2B.3”, MediaTek Inc.