3GPP TSG-RAN WG4 Meeting #113	R4-2419517

Orlando, USA, November 18th – 22nd, 2024

Agenda Item: 7.4.3

Source: MediaTek inc.

Title: Discussion on core part for FR1 less than 5MHz

Document for: Discussion

# Introduction

The open issues for the discussion of FR1 <5MHz are captured in the way forward (WF). In this document, we provide our views on NW indication on PBCH BW in MO configuration and/or HO command [1].

Sub-topic 1-3 Network indication on SSB bandwidth Issue 1-3-1b: NW indication on PBCH BW in MO configuration: Agreement: - Unless it will be agreed in RF session that the SCell SSB block with 12 PRB Bandwidth should be only on sync raster in all the cases in the WI, otherwise it is agreed in RRM session that: - Introduce network signalling in the MO on the target SSB bandwidth. Issue 1-3-1c: NW indication on PBCH BW in HO command/SCell addition command/SIB4 message: Agreement: NW indication on PBCH BW in HO command/SCell addition command: - Further check the case of Scell addition during handover during this meeting, and not introduce new network indication for the other cases.

# Discussion on the indication of PBCH BW

The SSB index reading is based on reading the DMRS in PBCH signals. For legacy NR requirements, the SSB is made of 20 PRBs, which is the case for all previous NR releases for different features. For < 5MHz UE operating with 3 MHz, the UE is required to measure a truncated SSB, which is made of 12 PRBs. Given that the SSB index reading is impacted on whether the PBCH is 12 or 20 PRBs, thus, it is essential to the UE to know whether the PBCH is 12 PRBs or 20 PRBs when performing measurements and handover. In the previous meeting, RAN4 agreed to introduce network signalling in the MO on the target SSB bandwidth. However, RAN4 discussed whether there is a need to introduce network signalling for the case of SCell addition during handover. In our view the NW indication on the PBCH BW is needed for the handover command with direct SCell activation.

- RAN4 to send LS to RAN2 and ask to add the information of whether the SSB is 12 or 20 PRBs in the handover command (HO) with SCell activation.

- RAN4 shall send the following LS as shown in this document:

- Overall Description: Based on discussions in RAN4#112-bis on the WI of NR_FR1_lessthan_5MHz_BW_Ph2, RAN4 has reached consensus that there is a need for a further network indication to assist UE to differentiate whether the SSB is 12 PRBs or 20 PRBs during the following scenarios: - The measurement object (MO) on the target SSB bandwidth; and - Handover command with direct SCell activation. Therefore, RAN4 would like to ask RAN2 to introduce network signalling in the measurement object (MO) and handover command with direct SCell activation on the target SSB bandwidth (i.e. whether the SSB is 12 PRBs or 20 PRBs). 3. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account and add the indication on whether the SSB is 12 PRBs or 20 PRBs in the measurement object and handover command with direct SCell activation.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 to send LS to RAN2 and ask to add the information of whether the SSB is 12 or 20 PRBs in the handover command (HO) with SCell activation.

Proposal 2: RAN4 shall send the following LS as shown in this document:

- Overall Description: Based on discussions in RAN4#112-bis on the WI of NR_FR1_lessthan_5MHz_BW_Ph2, RAN4 has reached consensus that there is a need for a further network indication to assist UE to differentiate whether the SSB is 12 PRBs or 20 PRBs during the following scenarios: - The measurement object (MO) on the target SSB bandwidth; and - Handover command with direct SCell activation. Therefore, RAN4 would like to ask RAN2 to introduce network signalling in the measurement object (MO) and handover command with direct SCell activation on the target SSB bandwidth (i.e. whether the SSB is 12 PRBs or 20 PRBs). 3. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account and add the indication on whether the SSB is 12 PRBs or 20 PRBs in the measurement object and handover command with direct SCell activation.

# References

- R4-2416849, “WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2”, Intel, Hefei, China, October 2024.