3GPP TSG-RAN WG4 Meeting #113	R4-2419529

# Orlando, United States, 18 – 22 November 2024

Agenda item:		7.4.2

Source: 		Qualcomm Inc.

Title: 		LS to RAN2 on sync raster agreements

Document for:		Approval

## 1	Introduction

In RAN4#112bis RAN4 agreed to restrict SSB positions for 3 MHz cells and 5 MHz cell with 20 RB maximum transmission bandwidth strictly to same sync raster points as defined for PCell. It would be useful to convey this to RAN2, so they can also make necessary specification accommodations, if any. In this contribution potential impact is discussed and LS is proposed.

## 2	Discussion

in RAN4#112bis RAN4 reached the agreement to restrict the Scell positions and following agreement was captured in the WF [1].

Correspondingly the restrictions were endorsed also in the big draft CR [2]

Furthermore, in RAN4#112bis, RAN4 further agreed to restrict 12 and 20 RB transmission bandwidth configurations only to band n100 and only to the sync raster points in Table 5.4.3.1-3 when 12 and 20 RB are used as Scell. This was captured in endorsed big draft CR [2] as show below.

RAN4 and RAN2 have had multiple LS exchanges related to sync raster and less-than-5MHz configurations, e.g. on neighbour cell measurements and recently we received a question on SIB1 configuration.

It would be useful to ensure that the whole RAN2 group is aware of the Scell sync restriction. Overall, it would be better if this update is also clearly captured in RAN2 specification to ensure that there are no misunderstandings and network configuration in real deployment takes the restriction into account.

One potential update in RAN2 specifications could be an update on the definition of IE absoluteFrequencySSB, which currently states that cell-defining SSB of the Pcell is always on the sync raster. Given the new RAN4 agreements, it could be clarified that in addition SSB for less-than-5MHz carriers is always on the sync raster aligned with Tables 5.4.3.1-2 and 5.4.3.1-3 in TS 38.101-1.

Proposal 1: Send an LS to RAN2 to inform them on recent agreement to restrict SSB of less-than-5MHz Scell to sync raster as specified in Tables 5.4.3.1-2 and 5.4.3-3 of TS 38.101-1.

## 3	Conclusion

In this contribution the previous agreement on SSB positions for less-than-5MHz Scells was discussed and following proposal was made.

Proposal 1: Send an LS to RAN2 to inform them on recent agreement to restrict SSB of less-than-5MHz Scell to sync raster as specified in Tables 5.4.3.1-2 and 5.4.3-3 of TS 38.101-1.

Draft LS is provided in the appendix.

## 4	References

[1]	R4-2417192, WF on UE RF requirements for Rel-19 NR channel BW less than 5 MHz for FR1 phase 2

[2]	R4-2416083, Draft Big CR for less than 5MHz UE RF requirements in Rel-19, Inter Corporation, Nokia, ZTE

## Appendix

Title:	draft LS on SSB position restrictions for less-than-5MHz Scells

Response to:

Release:	Rel-19

Work Item:	NR_FR1_lessthan_5MHz_BW_Ph2-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact person:	Toni Lähteensuo

tlaehtee@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 has agreed to restrict SSB positions of less-than-5MHz NR cells to be always on the sync raster and capture the restriction in Tables 5.4.3.1-2 and 5.4.3.1-3 in TS 38.101-1. Restriction applies independent of whether the operation is in CA, NR-DC or EN-DC. The endorsed draft CR showing the detailed updates can be found in R4-2416083.

RAN4 observed that TS 38.331 captures in the definition of absoluteFrequencySSB that cell-defining SSB of the Pcell is always on the sync raster. Given the RAN4 agreement, the definition of absoluteFrequencySSB should be extended to cover also less than 5 MHz Scells.

RAN4 would like to respectfully ask RAN2 to consider the SSB position restriction in any future work and also consider the necessity to capture this restriction in RAN2 specifications.

# Actions

ACTIONS:

To RAN WG2: RAN4 respectfully asks RAN2 to consider the SSB position restriction in any future work and also consider the necessity to capture this restriction in RAN2 specifications.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #114	Feb 17 - 21, 2025	Athens, Greece

TSG-RAN4 Meeting #114bis	Apr 07 - 11, 2025	TBD, China