3GPP TSG-RAN WG4 Meeting #113	R4-2419535

# Orlando, United States, 18 – 22 November 2024

Agenda item:		8.2

Source: 		Qualcomm Inc.

Title: 		On carrierBandwidth configuration for less-than-5MHz carriers

Document for:		Approval

## 1	Introduction

RAN4 has received an LS from RAN2 on carrierBandwidth configuration for less-than-5MHz carriers [1]. In this contribution our views and suggested reply to RAN2 is provided.

## 2	Discussion

Contents of the RAN2 LS are reproduced below

When RAN4 specified the less-than-5MHz support, it was agreed that 12 RB transmission bandwidth configuration follows the RF requirements of 3 MHz channel bandwidth, and similarly 20 RB transmission bandwidth configuration follows the RF requirements of 5 MHz channel bandwidth. As such, from RF requirement perspective 12 and 20 RB cases are still equivalent to 3 and 5 MHz channel bandwidth.

Observation 1: From RF requirement perspective 12 RB and 20 RB transmission bandwidth configurations are equivalent to 3 and 5 MHz channel bandwidths, respectively.

From RAN4 perspective, the maximum transmission bandwidth configuration RAN2 specification refers to, means the maximum transmission bandwidth configuration of the channel bandwidth, aligned with Table 5.3.2-1 in TS 38.101-1

That is, based on the value indicated in carrierBandwidth, UE picks the channel bandwidth which is equal to or smaller than the value indicated in carrierBandwidth. Therefore, 12 and 20 RB configurations are invisible here and the legacy procedure of indicating the carrierBandwidth to UE is still valid. As shown in the snippet from TS 38.331 in the LS, UE would consider the cell barred if 12 RBs is indicated for 3 MHz channel bandwidth.

The sync points defined for 12 and 20 PRB transmission bandwidth configurations were defined with the understanding that channel bandwidth is defined as if full 15 or 25 RBs are available. That is, the RB grid is exactly the same as for 15 and 25 RB transmission bandwidth configuration, but only 12 or 20 lowest RBs are available to use. This is described e.g. in [3] and [4]. This also fully aligns with the design target to facilitate migration from GSM-R to NR and keep the NR RBs as close to the low end of the band as possible.

Figure 1: Illustration of RBs used for 12/20 RB configurations in 15/25 RB (3 or 5 MHz) channel bandwidths

Observation 2: 12 and 20 PRB transmission bandwidth configurations in band n100 do not need to be accounted for in carrierBandwidth indication, which from RAN4 perspective relates to maximum transmission bandwidth configuration of the RF channel bandwidth. RB grid is based on 15 RB and 25 RB, but only 12 or 20 RB within that grid and at low edge of the channel is used.

UE can still uniquely identify the cases where 12 and 20 PRB transmission bandwidth configuration is used, as specific sync raster points were defined for these cases.

Furthermore, in RAN4#112bis, RAN4 further agreed to restrict 12 and 20 RB transmission bandwidth configurations only to band n100 and only to the sync raster points in Table 5.4.3.1-3 when 12 and 20 RB are used as Scell. This was captured in endorsed big draft CR [2] as show below.

Observation 3: UE can identify 12 and 20 RB configurations both in single carrier and CA/DC operation based on used sync raster point.

Given the discussion and explanations provided, it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

Proposal 1: Reply to RAN2 that from RAN4 perspective it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

## 3	Conclusion

In this contribution carrierBandwidth configuration for less-than-5MHz carriers was discussed. Following observations and proposals were made.

Observation 1: From RF requirement perspective 12 RB and 20 RB transmission bandwidth configurations are equivalent to 3 and 5 MHz channel bandwidths, respectively.

Observation 2: 12 and 20 PRB transmission bandwidth configurations in band n100 do not need to be accounted for in carrierBandwidth indication, which from RAN4 perspective relates to maximum transmission bandwidth configuration of the RF channel bandwidth. RB grid is based on 15 RB and 25 RB, but only 12 or 20 RB within that grid and at low edge of the channel is used.

Observation 3: UE can identify 12 and 20 RB configurations both in single carrier and CA/DC operation based on used sync raster point.

Proposal 1: Reply to RAN2 that from RAN4 perspective it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

## 4	References

[1]  	R4-2417510/R2-2408409, LS on carrierBandwidth configuration for less-than-5MHz carriers, RAN2

[2]	R4-2416083, Draft Big CR for less than 5MHz UE RF requirements in Rel-19, Inter Corporation, Nokia, ZTE

[3]	R4-2308539, Spectrum less than 5 MHz - System parameters, Ericsson

[4]	R4-2309721, System parameters for dedicated spectrum, Qualcomm Inc.