3GPP TSG-RAN WG4 Meeting #113	R4-2419537

# Orlando, United States, 18 – 22 November 2024

Agenda item:		5.33.1

Source: 		Qualcomm Inc.

Title: 		On NTN NB-IoT in-band operation with NR

Document for:		Approval

## 1	Introduction

In recent meetings there has been maintenance and TEI discussions on adding a new feature of NTN NB-IoT in-band operation into TS 36.102. In RAN4#112 bis a way forward listing related open issues was agreed [1]. This contribution discussed how to proceed with the work.

## 2	Discussion

NTN NB-IoT was originally introduced in release 18 work item [2]. The work item objectives limited the scope to standalone operation:

The work item was preceded by a study on NB-IoT and eMTC support for NTN. The study item is documented in TR 36.763 [3]. For IoT NTN scenarios the study recommended to prioritize standalone deployment for NB-IoT / eMTC for support in Rel-17 timeframe.

RAN1/2/3 did the relevant specification work in release 17, but due to workload concern the corresponding UE RF requirement specification TS 36.102 was introduced only in rel-18. Release independence specification TS 36.307 makes it clear that rel-17 UEs supporting operating bands specified in rel-18 TS 36.102 shall meet the requirements of the rel-18 specifications.

NTN NB-IoT in-band operation was scoped for rel-19 RAN package, but it was not included.

Observation 1: Only standalone operation is covered in current specification.

Observation 2: Both rel-17 and rel-18 UEs comply with rel-18 version of TS 36.102

Given that the rel-18 work item setting the requirements for rel-17 and rel-18 UEs was closed already in Dec 2022 and that release 18 has overall been frozen a long time already, it is not possible to introduce new mandatory features to release 18. Depending on the final scope of the specification changes also in other working groups, it could be acceptable to make support optional in release 18, e.g. by making the changes to rel-19 specification and applying release independence.

Proposal 1: NTN NB-IoT Inband operation can at most be optional in rel-18. It is recommended to do specification updates to open release and allow release independence to rel-18.

In RAN1 specifications NB-IoT in-band operation was never optimized for in-band operation with NR, instead the specifications consider E-UTRA. When operation mode of NB-IoT is set as in-band TS 36.211 and 36.213 set expectations on LTE CRS and rate matching around it. As CRS does not exist for NR, this functionality is not needed. If specification changes in RAN1 want to be avoided, the correct approach is to indicate operationModeInfo in MIB as guardband operation. Guardband operation is similar to standalone in the sense that no CRS accommodations are required, but instead only raster shifts need to be applied.

Indicating operationModeInfo as guardband can avoid the impact in RAN1, but otherwise it would be confusing to a specification reader why this is done. Therefore it would be at least necessary to clearly capture in RAN4 specifications that that MIB indicates guardband when operation in fact is in-band with NR, and that MDL is adjusted in this case.

Proposal 2: For NTN NB-IoT Inband operation with NR, operationModeInfo shall be indicated as guardband. RAN4 specification needs to clearly capture this, formulas for channel frequency, and which MDL apply when operationModeInfo is indicated as guardband.

When it comes to the MDL values, a change compared to TN specifications was done in rel-17 to enable gNB to transmit both anchor and non-anchor carriers in standalone mode with 180 kHz center frequency spacing, i.e. conveniently with a single IFFT. This improvement should not be lost.

(TS 36.101) FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1); standalone anchor-carrier MDL = -0.5

(TS 36.102) FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL); standalone anchor-carrier MDL = 0

Due to this update the same MDL values specified for in-band operation in TS 36.101 no longer work for in TS 36.102, as there is 2.5 kHz difference and NB-IoT carrier is no longer aligned with NR RB and NR subcarrier grid.

Overall, the cleanest approach would be to create a new sub-clause for NTN NB-IoT in-band operation with NR. This would avoid any unintentional change to standalone operation, and provide a container to describe Having a separate clause also could be helpful in further indicating which requirements are mandatory and which are optional.

Proposal 3: Add a new clause to TS 36.102 for channel raster, carrier frequency and EARFCN for NTN NB-IoT in-band operation with NR

This new clause can be used to describe the MIB indication, and provide new formulas (i.e. formulas from TS 36.101) to be used when MIB indicates guardband operation, i.e. operation is in-band with NR. The items to be captured are need to include at least

- For NB1/NB2 in-band operation with NR, MIB indicates operationModeInfo as guardband

- When operationModeInfo is indicated as guardband

  - FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

  - For the carrier including NPSS/NSSS for in-band operation, MDL is selected from {-2,- 1,0,1}.

Proposal 4: In the new clause describe the MIB indication, formulas used to define the FDL, and applicable MDL for in-band operation with NR.

## 3	Conclusion

In this contribution NTN NB-IoT in-band operation with NR was discussed. Following observations and proposals were made.

Observation 1: Only standalone operation is covered in current specification.

Observation 2: Both rel-17 and rel-18 UEs comply with rel-18 version of TS 36.102

Proposal 1: NTN NB-IoT Inband operation can at most be optional in rel-18. It is recommended to do specification updates to open release and allow release independence to rel-18.

Proposal 2: For NTN NB-IoT Inband operation with NR, operationModeInfo shall be indicated as guardband. RAN4 specification needs to clearly capture this, formulas for channel frequency, and which MDL apply when operationModeInfo is indicated as guardband.

Proposal 3: Add a new clause to TS 36.102 for channel raster, carrier frequency and EARFCN for NTN NB-IoT in-band operation with NR

Proposal 4: In the new clause describe the MIB indication, formulas used to define the FDL, and applicable MDL for in-band operation with NR.

## 4	References

[1]  	R4-2417184, WF for NTN NB-IoT Inband operation, Ericsson

[2]	RP-223437, Revised WID on NB-IoT/eMTC core & performance requirements for NTN, MediaTek Inc

[3]	TR 36.763, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)