3GPP TSG-RAN WG4 Meeting #113	R4-2419539

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.8.2.3.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN HPUE TX RF requirements

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable high power UE (HPUE) with defining RF requirements targeting at support of IoT-NTN UEs for FR1 single carrier scenario.

In this contribution, our views about HPUE TX RF requirements for IoT-NTN UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

In RAN4#112bis meeting, the agreed WF [2] provides the following works for specifying TX requirements (e.g., ACLR, MPR) for IoT-NTN under HPUE operation. The contents are shown as follows.

NTN HPUE UE RF – General & Feasibility Issue 1-1-3: For NB-IoT based IoT-NTN, whether PC1 is feasible for handheld device. - Proposals - Option 1: Yes, (vivo, Qualcomm) - Based on measurements for two different PAs. (vivo) - Handheld satellite phones which have very high output power exist.(Qualcomm) - Option 2: Others - WF - Continue discussion in next meeting. NTN HPUE UE RF – Tx requirements Issue 1-2-2: ACLR - Proposals - Proposal 1: Discuss whether to reuse IoT-NTN PC3 ACLR values for PC2, PC1.5, and PC1 ACLR because the NTN-TN coexistence evaluation [5] shows IoT-NTN ACLR values can be lower than 15dB for PC2 to PC1 HPUE. (MediaTek) - Proposal 2: Based on our co-ex study results, we suggest ACLR for NB IoT-NTN HPUE PC2 can be 24.5 dBc. And this can apply to both handheld and non-handheld devices. (Samsung) - WF - Encourage more input next meeting. Issue 1-2-3: SEM - Proposals - Proposal 1: Regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing SEM requirements when the existing SEM requirements are more stringent than the PC2, PC1.5 and PC1 ACLR requirements. (Mediatek) - Proposal 2: others. - Recommended WF - Encourage more input next meeting. Issue 1-2-4: MPR - Proposals - Proposal 1: If IoT-NTN PC3 ACLR value could be reused for IoT-NTN HPUE ACLR and relaxing SEM would be possible under HPUE operation, discuss whether reusing IoT-NTN PC3 MPR values for PC2 MPR would be one possible way - Proposal 2: Others - Recommended WF - Encourage more input next meeting. Issue 1-2-5: MOP tolerance - Proposals - Proposal 1: Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE. (Ericsson) - Proposal 2: Others - Recommended WF - FFS.

Observation 1: Regarding NTN NB-IoT, the evaluation result by using GSM PA with EDGE waveform of 3dB PAPR can reach the 30.4dBm output power in typical case. The TX signal is close to PA saturation point and has adjacent channels spectrum growth. Because PAPR of 1 tone of 15kHz with BPSK or QPSK is lower than 2dB, more measurements are needed to verify the feasibility for PC1 with GSM PA.

Proposal 1: Because PAPR is lower than 2dB for 15kHz 1 tone with BPSK or QPSK, to consider further evaluation or measurement to verify the feasibility for PC1 with GSM PA for 1 tone of 15kHz SCS.

Observation 2: There would be NTN-TN coexistence result for IoT-NTN HPUE ACLR. These results can be used as reference for further discussion on specifying final IoT-NTN HPUE ACLR requirements.

Proposal 2: Use NTN-TN coexistence result as reference for further discussion on specifying final IoT-NTN HPUE ACLR requirements.

Observation 3: When UE CBW is much narrower compared to BS CBW, there is discussion about considering relaxing ACLR and SEM in other WI.

Proposal 3: When UE PC2 ACLR is decided, regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing UE SEM requirements within SAN CBW but still meet ITU SEM regulatory requirements.

Observation 4: MPR requirements are associated with ACLR and SEM requirements.

Proposal 4: MPR requirements could be specified after ACLR and SEM requirements are determined.

Regarding the eMTC/NB-IoT NTN UE RF requirements, the TX maximum output power (MOP) requirements can be associated with RF HD-FDD operation.

It is observed that the HD-FDD RF front-end insertion loss (FE IL) is lower compared to FD-FDD FE IL and can improve the RX REFSENS and TX MOP simultaneously.

Observation 5: HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

From EC Decision 2016/687[3], it is observed that a tolerance of up to + 2 dB is to take account of operation under extreme environmental conditions and production spread. Regarding UE TX output power, the power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be mobile or nomadic.

Observation 6: From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

Observation 7: The actual NTN-IoT PC2 HD-FDD TX output power upper bound could be 28.8dBm (i.e., 26.8dBm+2dB) but exceeds conventional PC2 TX upper power bound of 28dBm.

From ETSI EN 301 908[4], it is observed that PC3 output power upper bound can be specified as 23dBm +2.7dB or 23dBm +3.

Observation 8: From ETSI EN 301 908, PC3 TX output power upper bound can be specified higher (i.e., 23dBm +2.7dB or 23dBm +3dB).

Additionally, in our understanding, the regulatory requirements for in-band TX output power are associated with SAR testing.  The UE TX output power upper bound would be considered when performing SAR testing. Compared with FD-FDD TX SAR, HD-FDD TX SAR issues have been further alleviated.

To consider the solution for HD-FDD TX MOP upper bound higher than FD-FDD TX MOP upper bound for all IoT-NTN bands is needed.

Proposal 5: Regarding solution for PC2 HD-FDD TX MOP upper bound higher than FD-FDD TX MOP upper bound, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC3 and PC5. The options are as follows.

- Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD]. |  |  |  |  |  |  |

- Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment.

- Option 2-1:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB. |  |  |  |  |  |  |

- Option 2-2:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 255 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 254 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 253 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 252 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd. |  |  |  |  |  |  |

- Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD).

3 Conclusion

In this paper, our views about HPUE TX RF requirements for IoT-NTN UE are provided as follows:

Observation 1: Regarding NTN NB-IoT, the evaluation result by using GSM PA with EDGE waveform of 3dB PAPR can reach the 30.4dBm output power in typical case. The TX signal is close to PA saturation point and has adjacent channels spectrum growth. Because PAPR of 1 tone of 15kHz with BPSK or QPSK is lower than 2dB, more measurements are needed to verify the feasibility for PC1 with GSM PA.

Proposal 1: Because PAPR is lower than 2dB for 15kHz 1 tone with BPSK or QPSK, to consider further evaluation or measurement to verify the feasibility for PC1 with GSM PA for 1 tone of 15kHz SCS.

Observation 2: There would be NTN-TN coexistence result for IoT-NTN HPUE ACLR. These results can be used as reference for further discussion on specifying final IoT-NTN HPUE ACLR requirements.

Proposal 2: Use NTN-TN coexistence result as reference for further discussion on specifying final IoT-NTN HPUE ACLR requirements.

Observation 3: When UE CBW is much narrower compared to BS CBW, there is discussion about considering relaxing ACLR and SEM in other WI.

Proposal 3: When UE PC2 ACLR is decided, regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing UE SEM requirements within SAN CBW but still meet ITU SEM regulatory requirements.

Observation 4: MPR requirements are associated with ACLR and SEM requirements.

Proposal 4: MPR requirements could be specified after ACLR and SEM requirements are determined.

Observation 5: HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

Observation 6: From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

Observation 7: The actual NTN-IoT PC2 HD-FDD TX output power upper bound could be 28.8dBm (i.e., 26.8dBm+2dB) but exceeds conventional PC2 TX upper power bound of 28dBm.

Observation 8: From ETSI EN 301 908, PC3 TX output power upper bound can be specified higher (i.e., 23dBm +2.7dB or 23dBm +3dB).

Proposal 5: Regarding solution for PC2 HD-FDD TX MOP upper bound higher than FD-FDD TX MOP upper bound, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC3 and PC5. The options are as follows.

- Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD]. |  |  |  |  |  |  |

- Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment.

- Option 2-1:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB. |  |  |  |  |  |  |

- Option 2-2:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 255 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 254 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 253 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 252 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd. |  |  |  |  |  |  |

- Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD).

4 Annex

EC Decision 2016/687

The power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be fixed or installed and as total radiated power (TRP) (2) for terminal stations designed to be mobile or nomadic.

Total radiated power (TRP) is a measure of how much power the antenna actually radiates. The TRP is defined as the integral of the power transmitted in different directions over the entire radiation sphere.

ETSI EN 301 908

4.2.2 	Transmitter Maximum Output Power

4.2.2.1 	Transmitter maximum output power for Single Carrier

4.2.2.1.1 	Definition

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub-frame (1 ms).

4.2.2.1.2 	Limits

The UE maximum output power shall be within the shown value in table 4.2.2.1.2-1.

Table 4.2.2.1.2-1: UE power classes

| E-UTRA Band | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| 1 | 23 | ±2,7 |
| 3 | 23 | ±2,7 (see note) |
| 7 | 23 | ±2,7 (see note) |
| 8 | 23 | ±2,7 (see note) |
| 20 | 23 | ±2,7 (see note) |
| 22 | 23 | +3,0/-4,5 |
| 28 | 23 | +2,7/-3,2 |
| 31 | 23 | ±2,7 |
| 33 | 23 | ±2,7 |
| 34 | 23 | ±2,7 |
| 38 | 23 | ±2,7 |
| 40 | 23 | ±2,7 |
| 42 | 23 | +3,0/-4,0 |
| 43 | 23 | +3,0/-4,0 |
| 65 | 23 | ±2,7 |
| 68 | 23 | ±2,7 |
| NOTE: For transmission bandwidths (ETSI TS 136 521-1 [1], clause 5) confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1,5 dB (tolerance = +2,7/-4,2). |  |  |

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2417205, “WF on HPUE for IoT NTN”, Vivo

- EC Decision 2016/687

- ETSI EN 301 908-13 V13.2.0 (2021-11)