3GPP TSG-RAN4 Meeting #113	R4-2419543

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0052 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE_NBIOT_eMTC_NTN_req-Core) CR to 36.102 for A-SEM and additional spurious emission arrangement improvements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MeidaTek Inc., Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core, NR_NTN_LSband |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The additional SEM (A-SEM) and addition spurious emisssion requirements are mixed together in one cluase. To move the A-SEM to correct clause is needed. Besides, some duplicate requirements can be merged. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move A-SEM requirements from additional spurious emissions clause to A-SEM clause. - For NW signallings (e.g., NS_04N, NS_05N) have the same spurious emission requirements, duplicate requirements are merged. It is noted that the requirements are unchanged by this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The placement of certain requirements that belong to A-SEM would be still in additional spurious emissions clause, which cause ambiguirity and misalignment when compared to NR NTN specification. Some duplicate requirements remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.2, 6.5A.3.3, 6.5B.3.3, 6.5B.3.4, 6.5A.4.4, 6.5B.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================Start of the change 1==========================

6.5A.3.2	Spectrum emission mask

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned E-UTRA channel bandwidth. For frequencies offset greater than ΔfOOB as specified in Table 6.5A.3.2-1 the spurious requirements in subclause 6.5A.4 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.5A.3.2-1 for the specified channel bandwidth.

Table 6.5A.3.2-1: Category M1 spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | Measurement bandwidth |
| 0-1 | -10 | 30 kHz |
| 1-2.5 | -10 | 1 MHz |
| 2.5-2.8 | -25 | 1 MHz |

NOTE1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

====================End of the change 1==========================

====================Start of the change 2==========================

6.5A.3.3	Additional Spectrum Emission Mask for category M1

6.5A.3.3.1	Requirements for network signalling value "NS_02N” and “NS_03N”

When "NS_02N" or "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1-1.

Table 6.5A.3.3.1-1: Additional requirements for "NS_02N" and "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ±0 – 0.7 | -2 for PC3 -5 for PC5 | 4 kHz |
| ±0.7 – 2.8 | -12 for PC3 -15 for PC5 | 4 kHz |
| ±>2.8 | -13 for PC3 and PC5 | 4 kHz |

6.5A.3.3.2	Requirements for network signalling value "NS_04N"

When "NS_04N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.2-1 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5A.3.3.2-1: Additional requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

6.5A.3.3.3	Requirements for network signalling value “NS_05N"

When "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-1 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5A.3.3.3-1: Additional requirements for "NS_05N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

====================End of the change 2==========================

====================Start of the change 3==========================

6.5B.3.3	Additional Spectrum Emission Mask for category NB1 and NB2

6.5B.3.3.1	Requirements for network signalling value "NS_02N”

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.1-1.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± [0.09 – 0.28] | -2 for PC3 -5 for PC5 | 4 kHz |
| ± [0.28 – 0.85] | -12 for PC3 -15 for PC5 | 4 kHz |
| ± [>0.85] | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

6.5B.3.3.2	Requirements for network signalling value "NS_03N"

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.2-1.

Table 6.5B.3.3.2-1: Additional requirements for "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± [0.09 – 0.28] | -2 for PC3 -5 for PC5 | 4 kHz |
| ± [0.28 – 0.85] | -12 for PC3 -15 for PC5 | 4 kHz |
| ± [0.85 – 1.7] | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

6.5B.3.3.3	Requirements for network signalling value "NS_04N"

As specified in 6.5A.3.3.2.

6.5B.3.3.4	Requirements for network signalling value “NS_05N"

As specified in 6.5A.3.3.3.

====================End of the change 3==========================

====================Start of the change 4==========================

6.5B.3.4	Adjacent Channel Leakage Ratio for category NB1 and NB2

Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The assigned category NB1or NB2 channel power and adjacent channel power are measured with filters and measurement bandwidths specified in Table 6.5B.3.4-1. If the measured adjacent channel power is greater than –50dBm then the category NB1 or NB2 UE ACLR shall be higher than the value specified in Table 6.5B.3.4-1. GSMACLR requirement is intended for protection of GSM system. UTRAACLR requirement is intended for protection of UTRA and E-UTRA systems.

Table 6.5B.3.4-1: Category NB1 and NB2 UE ACLR requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 20 dB | 37 dB |
| Adjacent channel center frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

====================End of the change 4==========================

====================Start of the change 5==========================

6.5A.4.4	Additional spurious emissions

6.5A.4.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

6.5A.4.4.2	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2.

Table 6.5A.4.4.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4MHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

6.5A.4.4.3	Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3-1.

Table 6.5A.4.4.3-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band 256. |  |  |

6.5A.4.4.4	Minimum requirement (network signalled value "NS_03N")

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.4-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth.

Table 6.5.4.4.4-1: Additional out-of-band requirements for "NS_03N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

6.5A.4.4.5	Minimum requirement (network signalled value "NS_04N" and "NS_05N")

When "NS_04N" or "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.5-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth.

Table 6.5A.4.4.5-1: Additional out-of-band requirements for "NS_04N" and "NS_05N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

6.5A.4.4.6	Void

====================End of the change 5==========================

====================Start of the change 6==========================

6.5B.4.4	Additional spurious emissions

6.5B.4.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

6.5B.4.4.2	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2.

Table 6.5B.4.4.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

6.5B.4.4.3	Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.3-1.

Table 6.5B.4.4.3-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band 256. |  |  |

6.5B.4.4.4	Minimum requirement (network signalled value "NS_03N")

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.4-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.

6.5B.4.4.5	Minimum requirement (network signalled value "NS_04N" and "NS_05N")

When "NS_04N" or "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.5-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.

6.5B.4.4.6	Void

====================End of the change 6==========================