3GPP TSG-RAN WG4 Meeting # 113												R4-2419553

Orlando, US, 18th – 22nd November, 2024

Source: 	Qualcomm Incorporated

Title: 	TP to 38.769 on co-existence evaluation methodology

Agenda Item:	7.21.2.1

Document for:	Approval

- Introduction

In previous meetings, the co-existence evaluation methodology were discussed and the agreements were captured in TR 37.769 v1.0.0. In this meeting we provide text proposal on coexistence evaluation methodology based on the latest agreements.

- Text proposal

## <<Start of Change>>

### 6.6.4	Co-existence simulation methodology

#### 6.6.4.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

1)	Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader or intermediate UE based on coupling.

-	For A-IoT devices are associated to A-IoT reader: In Table 6.6.3.1-1, two A-IoT readers are activated in one drop and active readers are 60m apart. The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other and meanwhile the receiving power meets the -36 dBm device receiver sensitivity. Following this access control algorithm, the performance of device can not be guaranteed since the reader that a device accesses at a certain time is not necessarily a good one. Therefore, the advanced access control algorithm needs to consider allowing the device access a reader that is closer to it at a certain time in the future. The access control based on device side determination and/or reader side determination.

3)	Once association is done, round robin scheduling is used.

4) For inter-system interference (between A-IoT and NR):

-	If SINR degradation is smaller than and equal to [1] dB, it can be considered that inter-system interference is negligible.

-	If SINR degradation is larger than [1] dB, consider the criteria: Outage percentage consider SINR level with [10%] BLER

-	Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

5) For intra-system interference (between A-IoT and A-IoT): Outage percentage consider SINR level with [10%] BLER

#### 6.6.4.2	SINR definition

SINR definition for D2R:

- SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference+residual CW interference after cancellation

- SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intra_system interference + inter_system interference+residual CW interference after cancellation

For the CW outside scenarios, the impact to SINR due to phase noise are also evaluated. The baseline D2R SNR for CW outside can be calculated by

baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterference+phase noisewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

Phase noise can be modelled as XdBc/Hz, e.g., -112dBc/Hz compared to the received power for D2R.

The noise and intra-system interference are within total receiver bandwidth, and the residual CW interference after cancellation is in linear scale.

SINR definition for R2D:

- signal power of device to the noise and interference within 10 MHz is baseline assumption

- Consider [180 kHz] noise and interference bandwidth after BB LPF as optional

#### 6.6.4.3	Coupling loss

The Coupling Loss (CL) is defined as the loss in signal between NR BS-to- NR UE, A-IoT reader -to- A-IoT device and intermediate UE -to- A-IoT device. CL is defined as the loss including propagation loss and antenna gains.

## <<End of Change>>

- References

[1] 3GPP TR 36.942 v18.0.0