3GPP TSG-RAN WG4 Meeting #113		 			                        R4-2419557
Orlando, USA, 18th – 22nd November, 2024
 
Agenda Item: 7.9.3

Source:	SKY Perfect JSAT

Title: Ku-band Efficient channel utilization analysis

Document for:	Discussion and Decision

# Introduction

In the RAN4#112 WF in R4-2413519[1], it is agreed that the five following technical factors are used to determine the feasibility of FR1 and FR2.

The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology - UL timing synchronization - Phase noise - Efficient channel utilization - Beam hopping granularity - Link budget UL/DL – as a function of SCS and channel bandwidth

In this contribution we provide our views on efficient channel utilization.

# Analysis and Discussion

## NR Channel Allocation on SDSs

In recent years, the softwarization of satellite transponders has advanced, leading to the emergence of GEO Software Defined Satellites (SDSs). Although there are some limitations with SDSs, the satellite channel allocation is flexible, allowing it to be adjusted to meet NR specifications.

SDSs can also utilize beamforming technology to create numerous spot beams, exceeding several hundred.

In GEO-SDS (HTS) operations, optimizing satellite capacity is paramount. This optimization hinges on two critical factors: the number of frequency divisions (often referred as the number of color) and the number of repetitions.

In Ku-band, there is a limited bandwidth, especially for uplink of 500 MHz bandwidth in total if we use the band 14.0 – 14.5 GHz. The optimization of color number in the limited bandwidth to increase the total capacity of the satellite result in the satellite channel BW allocated to a single beam becoming smaller.

In addition, Ku-band GEO satellites are now widely used worldwide in a variety of use cases, and use cases using the DVB standard are expected to continue for some time in parallel with the adoption of NR Therefore, satellite operators will need to allocate satellite channel BW not only for NR services but also for other use cases such as DVB services and it is not always possible to allocate  more than 100 MHz bandwidth continuously available for NR, and it may not be possible to flexibly change the satellite channel BW allocated to NR.

Observation 1: In the limited bandwidth of the Ku-band, increasing the number of frequency divisions of GEO-SDS(HTS) result in the satellite channel BW allocated to a single beam becoming smaller.

Observation 2: It is not always possible for satellite operators to allocate more than 100 MHz bandwidth contiguously to NR because these satellite channel BWs should be allocated not only to NR but also to other use cases such as DVB services.

In the contribution from the previous meeting [2], it is explained that the FR1 reduces wasted bandwidth compared to FR2 when using legacy satellite transponder. If the channel bandwidth allocation to NR becomes smaller than 100 MHz and non-flexible, the amount and flexibility of the channel bandwidth allocation for NR in SDSs will be similar to a legacy satellite transponder. Therefore, RAN4 should consider that FR1 can reduce the wasted bandwidth further compared to FR2, allowing for more efficient use even though using future satellites like SDSs.

Proposal 1: RAN4 to consider that FR1 can reduce the wasted bandwidth further compared to FR2, allowing for more efficient use even though using future satellites like SDSs.

## NR Channel Allocation on Legacy Satellites

In the contribution from the previous meeting [3], SKY Perfect JSAT raised some observations and issues when using legacy satellites for NR services as follows.

For FR1, by using the FR1 channel definitions, it is possible to utilize channels with a bandwidth smaller than the legacy satellite transponder bandwidth without impacting RAN4 requirements.

Observation 3: By using the FR1 channel definitions, it is possible to utilize NR channels with a bandwidth slightly smaller than the legacy satellite channel bandwidth without impacting RAN4 requirements.

On the other hand, for FR2, as Figure 2.2-1 shows, when the legacy satellite transponder spacing is smaller than the smallest FR2 channel bandwidth of 50 MHz and the FR2 channel that is closest to the satellite transponder spacing is used to allocate NR channels without overlap, the NR channel boundaries are placed within the usable satellite transponder bandwidth, which makes it inefficient to fully utilize the legacy satellite transponder bandwidth.

Figure 2.2-1 (FR2) Using a Closest NR Channel Bandwidth to the Legacy Satellite’s Transponder Bandwidth Without Overlap

Observation 4: For FR2, the larger channel bandwidths make it difficult to efficiently utilize the legacy satellite transponders having smaller bandwidth, since the NR channel boundaries fall within the usable satellite bandwidth.

Proposal 2: To efficiently utilize legacy GEO satellite which are widely equipped with smaller satellite transponder BW than the smallest FR2 channel BW of 50 MHz with minimal impact to RAN4, RAN4 to prioritize the channel definitions of FR1 over those of FR2.

## Co-existence of SDSs and Legacy Satellites

The GEO-SDSs, which allow for flexible changes to satellite channel bandwidth, are currently under development and are expected to become popular in the future. However, GEO satellite lifetime is generally quite long, more than 15 years, and the replacement of legacy satellites with SDSs will take time. During its transition period, legacy satellites will remain the main capacity for Ku-band NTN, therefore, it is preferable to utilize both SDSs and legacy satellites for NR services in order to widely deploy terminals in a timely manner.

Observation 5: GEO-SDSs are expected to become popular in the future, however, considering the lifetime of the GEO satellites, the replacement from legacy satellites into SDSs will take time. To ensure timely-manner terminal deployment, it’s preferable to utilize both SDSs and legacy satellites for NR services.

As mentioned above, we should promote NR services with not only SDSs but also legacy satellites, and FR1 is preferable when implementing NR service in both SDSs and legacy satellites.

Proposal 3: RAN4 to consider  FR1 is preferable when implementing NR service with both SDSs and legacy satellites because NR services should be promoted with not only SDSs but also legacy satellites.

There are some inputs that FR2 channel has advantages to support wider bandwidth, but even if FR1 channel is selected, we believe that Carrier Aggregation (CA) will be one way to support wider bandwidth. Since the support for NTN CA is not in the scope of current WID of Ku-band NR NTN, it is better for us to be capable of wider bandwidth by introducing Carrier Aggregation (CA) in a future Release, like Rel-20.

Proposal 4: It would be good to introduce Carrier Aggregation (CA) in a future release to support wider bandwidth with FR1 channel.

# Conclusions

Observation 1: In the limited bandwidth of the Ku-band, increasing the number of frequency divisions of GEO-SDS(HTS) result in the satellite channel BW allocated to a single beam becoming smaller.

Observation 2: It is not always possible for satellite operators to allocate more than 100 MHz bandwidth contiguously to NR because these satellite channel BWs should be allocated not only to NR but also to other use cases such as DVB services.

Observation 3: By using the FR1 channel definitions, it is possible to utilize NR channels with a bandwidth slightly smaller than the legacy satellite channel bandwidth without impacting RAN4 requirements.

Observation 4: For FR2, the larger channel bandwidths make it difficult to efficiently utilize the legacy satellite transponders having smaller bandwidth, since the NR channel boundaries fall within the usable satellite bandwidth.

Observation 5: GEO-SDSs are expected to become popular in the future, however, considering the lifetime of the GEO satellites, the replacement from legacy satellites into SDSs will take time. To ensure timely-manner terminal deployment, it’s preferable to utilize both SDSs and legacy satellites for NR services.

Proposal 1: RAN4 to consider that FR1 can reduce the wasted bandwidth further compared to FR2, allowing for more efficient use even though using future satellites like SDSs.

Proposal 2: To efficiently utilize legacy GEO satellites which are widely equipped with smaller satellite transponder BW than the smallest FR2 channel BW of 50 MHz with minimal impact to RAN4, RAN4 to prioritize the channel definitions of FR1 over those of FR2.

Proposal 3: RAN4 to consider FR1 is preferable when implementing NR service with both SDSs and legacy satellites because NR services should be promoted with not only SDSs but also legacy satellites.

Proposal 4: It would be good to introduce Carrier Aggregation (CA) in a future release to support wider bandwidth on FR1 channel.

# References

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General; Moderator (Eutelsat Group), RAN4#112

- R4-2415379, Analysis of NTN channel bandwidths for optimal efficiency; Intelsat, SKY Perfect JSAT, RAN4#112bis

- R4-2413458, Ku Band Channel Allocation for Legacy Satellite System; SKY Perfect JSAT, RAN4#112