TSG-RAN Working Group 4 (Radio) meeting #113	R4-2419585

Orlando, US, 18th to 22th November 2024

Source:	Ericsson

Title:	Improvement of TRP measurement methods in TS 38.141-2, Annex I

Agenda item:	7.11.4.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#112-bis in Hefei) the sub-topic related to TRP improvements was allocated a separate discussion thread. In this contribution we provide some more details in improvements to be considered to include feedback received from regulators.

# Discussion

For NR BS type 1-O and NR BS type 2-O, Total Radiated Power (TRP) is used as figure of merit for BS output power and unwanted emission requirements. TRP can be measured using either Reverberation Chamber (RC) or far-field/near-field test range based test environments in Anechoic Chamber (AC) by averaging over spatially discrete EIRP samples.

Currently, the conformance test specification (TS 38.141-2) describes a collection of different methods to be used under different circumstances as alternative methods for TRP measurement in AC. It is not clear how the end result depends on the choices, and the dependencies between the selected methods is unclear.

It has been noticed that the different EIRP averaging methods (i.e. through integration) described as  viable alternatives are characterised by very different uncertainty of the results, which is not always acceptable for  claiming compliance to to licence condition and regulatory requirements, such as output power and unwanted emission requirements.

Observation 1: Multiple measurement methods characterised by different MUs are defined for TRP requirements, such as BS output power, OBUE, ACLR and spurious emissions. Simpler methods based on low number of test points (e.g. 2-cut ) are usually producing less accurate results than methods involving a better sampling of the measurement space with large number of points (e.g. full sphere with reference angular steps). All methods should be quantified by a given correction factor ΔTRP.

The measurement uncertainty evaluation in TR 37.941 does not capture aspects related to equivalence of alternative test methods and options in TS 38.141-2, Annex I.

Observation 2: The MU evaluation for TRP measurements in CATR, direct far field chambers, etc. do not capture aspects related to the applicability and equivalence of the options and alternatives in TS 38.141-2, Annex I.

The feedback from several regulators is that a single measurement method shall exist in the standard, and alternative methods are only acceptable if they are proved to lead to the same outcome (i.e. pass or fail in meeting the limit).

The technical background for measuring BS output power, ACLR and spurious emission is very different. Based on feedback from regulators we propose to make an effort and improve the description of TRP measurement method with the intention so reduce overlap, options, and alternative approaches in TS 38.141-2, Annex I.

As an overview the issues with current description can be summarized as:

- General test method description improvement and applicability overview improvement: Describe the situations when a specific test method can be used, e.g., for a certain requirement specifically what methods that may apply and what additional information (antenna geometry, physical size of the antenna, etc.) that may be needed to be declared by the BS manufacturer. Add missing declarations to manufacturers declarations in TS 38.141-2, subclause 4.6. There is a great potential for improvements of Annex I to increase readability.

- Reducing overlapping methods: Some test method alternatives are overlapping or describes a non-relevant special case, e.g., Uniform Linear Array (ULA) for AAS BS is not very relevant. Overlapping and non-relevant cases can be removed in TS 38.141-2, Annex I.

- Pre-scan description improvement: Improve the description of the pre-scan procedure essential for spurious emission measurements. Depending on emission type (harmonic, image, LO-leakage, etc.) the beamforming will be ranging from full antenna gain to sub-array gain. The pre-scan procedure that is a quick search method in the frequency domain to identify emission, can be improved by adding more information related to special frequencies where high beam forming is expected.

- Enhanced measurement uncertainty evaluation: Improve Measurement Uncertainty (MU) evaluation for TRP methods in anechoic camber. Currently, a common MU evaluation is adopted for all alternative test approaches in TS 38.141-2, Annex I. It is not clear if measurement uncertainty contributions assumed by the common MU evaluation approach is applicable for all test methods.

- Clear method process: The overall method should be presented as a clear path starting with pre-scan, continuing with a range of increasingly more precise methods at problem frequencies, and if needed ending with the reference method (dense fullsphere using reference steps). There is a need to have one path for inband and one for outband, but with the same final step (reference method).

One option is to have a TRP measurement method described as a deterministic sequence (pre-scan, two-cut, three-cut, sparse sphere, dense sphere = reference method) that can never lead to a pass result where the reference method leads to failure. The reference method should here be a single method, preferably a full sphere EIRP average using reference angular steps (dense sphere). All included methods should have a correction factor (Delta TRP >= 0) that quantifies their fidelity compared to the reference method.

With better guidance and a reduced set of alternatives the description in TS 38.141-2, Annex I can be significantly improved resolving issues raised by regulatory bodies and test laboratories. Ideally, the regulators would like to see only one method for a specific class of products or frequency range, for example.

In current WID [1] the following statement is captured:

Investigate and if possible, simplify BS TRP test methods for FR1 and FR2 by improving the applicability and not removing any of the existing TRP test methods.

At the end of this contribution the original Annex I is attached as reference. It can be noticed that listed issues above would require a significant update to Annex I, including removal of information, moving information between sections to improve readability and addition of missing information.  Therefore, it seems necessary to update the WID at next RAN meeting. The WID update should be formulate such that RAN4 gets the mandate to improve Annex I.

Proposal 1: In the next WID revision refine objective to allow for improvements of TRP test method description, including aspects such as reducing test method overlap, pre-scan description improvement, enhancement of measurement uncertainty evaluation.

# Conclusion

In this contribution we have identified an issue related to the WID objective for the sub-topic of improving TRP measurement methods description in TS 38.141-2.

In this contribution we made the following observations:

Observation 1: Multiple measurement methods characterised by different MUs are defined for TRP requirements, such as BS output power, OBUE, ACLR and spurious emissions. Simpler methods based on low number of test points (e.g. 2-cut ) are usually producing less accurate results than methods involving a better sampling of the measurement space with large number of points (e.g. full sphere with reference angular steps). All methods should be quantified by a given correction factor ΔTRP.

Observation 2: The MU evaluation for TRP measurements in CATR, direct far field chambers, etc. do not capture aspects related to options and alternatives in TS 38.141-2, Annex I.

Based on the observations we propose following:

Proposal 1: In the next WID revision refine objective to allow for improvements of TRP test method description, including aspects such as reducing test method overlap, pre-scan description improvement, enhancement of measurement uncertainty evaluation.

# References

[1]	RP-240829, “New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE

TS 38.141-2, Annex I:

Annex I (normative):
TRP measurement procedures

# I.1	General

The annex describes various procedures for BS OTA TRP measurments. These procedures can provide either an accurate or an over-estimate of TRP values. The procedures for an accurate estimate can be applied to all TRP requirements. However, if a TRP requirement does not need accurate TRP estimate then the procedures for over-estimate of TRP may be used in order to have a reasonable OTA test time. Pre-scan does not provide an accurate TRP estimate or over-estimate of TRP. Pre-scan is a fast but coarse method that is used to identify the spurious emission frequencies with emission power as described in annex I.13. A sequential measurement is then made at the emission frequencies, to assess the TRP as described in annex I.2 to annex I.9.

When making TRP measurements the alignment between EUT and measurement antenna is important to achieve expected measurement uncertainty:

1.	The measurement antenna needs to be aligned tangential to the measurement surface forming a sphere around the EUT, in order to correctly measure the TRP properly.

2.	Test methods described in clauses I.5.1, I.5.2, I.10, I.11 and I.12 require angular alignment between the selected measurement grid and EUT radiation pattern in order to measure peak values in the main beams. Angular misalignment can lead to differences in the actual and measured angular positions of the intended maximum EIRP.

3.	Test methods described in clause I.5.3, I.6 and I.9 are designed to be independent of rotations of the angular grid, and hence angular alignment between the measurement grid and EUT is not needed.

# I.2	Spherical equal angle grid

## I.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in I.2.2.

## I.2.2	Reference angular step criteria

The reference angular steps Δϕref and Δθref in degrees are defined as:

Δϕref= min⁡(180∘πλDcyl,15∘)

Δθref=⁡min(180∘πλD,15∘)

The upper limit for these reference angular steps of 15∘ ensures a low Summation Error (SE) when λ is large compared to the EUT dimensions.

Dcyl and D are calculated as:

`	Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in figure I.2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure I.2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Optionally, in the case of Uniform Linear Array (ULA), when d is negligible (d ≈0) and the EUT is mounted along the yz plane as shown in figure I2.2-2, the reference angular steps, in degrees, can be determined by

∆θref=min(180°πarcsin⁡(λDz),15°)

Δϕref=min(180°πarcsin⁡(λDy),15°)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis and  is wavelength for the measured frequency.

Figure I2.2-2: Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

SF=maxΔθgridΔθref,ΔϕgridΔϕref

Where Δθgrid and Δϕgrid are the actual angular steps used in the measurement.

Alternatively, when EUT radiating dimensions are not known, for each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

# I.3	Spherical equal area grid

TRPEstimate is defined as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified as

The sampling intervals Δθref and Δϕref are described in annex I.2.2. Each (θn,ϕn) is a sampling point.

# I.4	Spherical Fibonacci grid

TRPEstimate is defined as

TRPEstimate=1Nn=0N-1EIRP(θn,ϕn)

N is the total number of samples and specified as

The sampling intervals Δθref and Δϕref are described in annex I.2.2. Each (θn,ϕn) is a sampling point, where θn and ϕn, in degrees, are defined as:

θn=cos-11-2n+1N

ϕn=360°nΨ,  Ψ=1+52

# I.5	Orthogonal cut grid

## I.5.1	General

Here, at least two cuts (default) shall be used, an optional third cut can be used. The alignment of the cuts must be along the symmetry planes of the antenna array. No alignment is required for spurious emissions.

When alignment is required:

1)	The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

2)	The second mandatory is a vertical cut passing through the peak direction of the main beam. Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

3)	The third optional cut is a vertical cut orthogonal to the first and the second cut.

When alignment is not required, the cuts can be aligned arbitrarily.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points in the cut. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the interclause points are measured multiple times and the repeated values can be removed from the samples before averaging.

When two cuts measurements are used, a conditional pattern multiplication can be applied. The following are the conditions for applying pattern multiplication:

i.	The vertical cut (and the main beam) is in the xz -plane

ii.	The frequency of the emission is within the downlink operating band.

iii.	The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

iv.	The emission appears/disappears when the Tx power is turned on/off.

v.	The antenna arrays of the EUT

1)	Have rectangular grids of antenna element positions

2)	Have symmetry planes that are vertical and horizontal.

3)	Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRPcut1ϕ at θ=θH and a vertical cut with data EIRPcut2θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemispheres. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in figure I.2.2-1 the u and v coordinates are defined as:

u=sinθsinϕv=cosθ

Note that only the data on the cuts are measured.

Calculate power density/EIRP values outside the two cardinal cuts as

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

NOTE:	The numerical singularity at u2+v2=1 must be treated with care, e.g. by change of variables.

## I.5.2	Operating band unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for the first two mandatory cuts and calculate the TRPEstimate.

2)	Compare the TRPEstimate to the limit.

3)	If the TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 2.

## I.5.3	Spurious unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for two cuts and calculate the preliminary TRPEstimate.

2)	Add the appropriate correction factor ΔTRP according to table I.5.3-1 to ensure overestimation with 95% confidence.

3)	Compare the corrected TRPEstimate (including ΔTRP) to the limit.

4)	If the corrected TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 3.

Table I.5.3-1: The correction factor for two or three cuts dense sampling

|  | Three cuts | Two cuts |
| --- | --- | --- |
| Correction factor ΔTRP (dB) | 2.0 | 2.5 |

# I.6	Wave vector space grid

If EUT is mounted along the yz plane as shown in figure I.2.2-1, the reference step in wave vector space can be determined by

Δuref=λDy

Δvref=λDz

where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be represented as following:

u=sinθsinϕ, v=cos⁡(θ)

The total radiated power (TRP) in the wave vector space is determined by:

TRPestimate=ΔurefΔvref4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

For spurious Tx or Rx emissions and where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

SF=max∆ugrid∆uref,∆vgrid∆vref

Where ∆ugridand ∆vgrid are the actual steps used in the wave vector space in the measurement and the upper bound of their value is 1/12, corresponding to 15 degree steps close to boresight.

# I.7	Void

# I.8	Void

# I.9	Full sphere with sparse sampling

The procedure is as follows:

1)	Set the angular grid:

a.	Non-harmonic frequencies: choose the angular steps Δϕ and Δθ smaller than or equal to 15 degrees. Calculate the sparsity factor (SF) as

SF=maxΔϕΔϕref,ΔθΔθref

and the correction factor as:

ΔTRP=SF-1SFmax-1⋅1.0 dB,

where SFmax corresponds to 15 degrees angular step. If the sparsity factor is smaller than 1, the correction factor ΔTRP is 0 dB.

Harmonic frequencies with fixed beam test signal: choose the angular steps smaller than or equal to the reference angular steps Δϕref and Δθref. Correction factor ΔTRP is 0 dB.

Harmonic frequencies with beam sweeping test signal: set the angular steps to 15 degrees. Correction factor is ΔTRP 0 dB.

2)	Apply a suitable numerical integration to calculate the preliminary TRPEstimate.

3)	Add the appropriate correction factor ΔTRP according to step 1 to ensure an overestimation with 95 % confidence.

4)	Compare the corrected TRPEstimate (including ΔTRP) with the limit. If the corrected TRPEstimate is above the limit, choose a smaller angular step and repeat steps 2-4. If the sparsity factor is less than one, no significant improvement of accuracy is expected.

# I.10	Beam-based directions

Beam-based direction can be used in the base station operating band only if the directivity of the radiation pattern of the emssions being measured is known. TRPEstimate is defined as

TRPEstimate=EIRPpeakDEUT, where EIRPpeak is the maximum EIRP in the beam peak direction within a particular beam direction pair and DEUT is the directivity of the EUT.

# I.11	Peak method

The peak method can be used when frequencies with unwanted peak emissions are identified during pre-scan. The method does not provide an estimate of TRP.

For each peak emission frequency identified during pre-scan, measure peak EIRP or power density as follows:

1)	Move EUT and test antenna to the same position where the peak emission is recorded during the pre-scan.

2)	Move the EUT around the position and test antenna orientation to find the final peak EIRP or power density.

3)	The measured peak power density or EIRP shall be used to demonstrate conformance.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarized components.

# I.12	Equal sector with peak average

Equal sector with peak average can be performed on frequencies with unwanted peak emission, which are considered by the peak method for further measurements.

The spherical angle ϕ is divided into K equal sectors. If the largest dimension of EUT is less than 60 cm, then each sector is a half quadrant of 45°.

For each peak emission frequency, measure peak EIRP of beams belonging to different sectors of the sphere as follows:

1)	Move EUT and test antenna to the same position where the emission peak is recorded during the pre-scan.

2)	Move EUT around the position and test antenna orientation to find the final peak EIRP.

3)	Repeat Steps 1 to 2 until all sectors are covered.

4)	Calculate TRPEstimate as

TRPEstimate=1Kk=1KEIRPk, where EIRPk is the peak EIRP in the kth sector.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarized components.

# I.13	Pre-scan

Pre-scan is used to identify frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP. An emission frequency identified by a pre-scan may be further investigated by any of the TRP measurement methods in this annex.

The procedure for pre-scan is as follows:

1)	Scan the entire surface around EUT.

2)	Rotate test antenna to cover all possible polarizations of emissions to detect maximum emissions.

3)	Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

4)	Emissions which 20 dB or more below the specified limit shall not require further measurements.