TSG-RAN Working Group 4 (Radio) meeting #113	R4-2419586

Orlando, US, 18th – 22nd November, 2024

Source:	Ericsson

Title:	Discussion on conformance test aspects for EEIRP and TP to TR 38.908

Agenda item:	7.11.4.1

Document for:	Approval

# Introduction

In this contribution we present information relevant for BS RF core and conformance specifications and propose text to be included in the clauses of TR 38.908. We focus on how to determine reasonable complex but relevant test beam configurations, general OTA test aspects, and initial thoughts related to the measurement uncertainty evaluation for EEIRP.

For AAS BS conformance testing the concept including a set of test beams was proposed by ITU-R, where the test beams are supposed to be spread out within the intended coverage range. The number of test beams, their direction and weights were not specified by ITU-R.

# Discussion

  - BS RF test approach

During the test the AAS base station is configured with a test baseband functionality that will use the test models and apply beam forming weight vector, as shown in Figure 2.1-1. With the beamforming weight vector, test beams will be generated according to requirement definitions in TS 38.141-2. In current version of the conformance specification, there is no description on how the beamforming weights from declared test beam directions are applied to the test model signal. For the OTA spatial emission requirement, with extended set of specified test beams, it would be reasonable to describe how the test beams are allocated to the test model signal, see example in Figure 2.1-2.

The test is conducted in a Line-of-Sight (LOS) environment without feedback from the measurement receiver. Unlike the UE conformance testing, the BS Down-link and Up-link are tested separately with no dependencies.

# Figure 2.1-1: Down-link test object configuration

The base station manufacturer shall declare the range of angles intended for directing the narrow beam under normal operation (also referred as CAR – Coverage Angular Range). This declaration will depend on intended deployment scenario. This would mean that if a specific base station implementation is intended for multiple deployment scenarios, (e.g., rural, sub-urban and urban), multiple declarations are needed.

# Figure 2.1-2: Test beam direction application to test model

The concept of static test beams allows for conformance testing in conventional antenna test ranges using standard test equipment (network analyser, signal generator, spectrum analyser, etc.).

Currently, the specification does not describe how beam-forming weights associated to test beams are applied to the test signal (test model) during test. Based on declared beam-peak direction, beam-forming weights are derived and applied to the test signal.

Observation 1: With the introduction of OTA spatial emission requirement RAN4 may need to consider how beamforming weights should be applied to the test model to guarantee a relevant test condition. If required, RAN4 need to capture a description of how beam weights associated to test beam directions are applied to the test model signal in TS 38.141-2.

  - OTA test aspects for EEIRP

For EEIRP testing an enhanced set of test beams are essential to guarantee measurement uncertainty and repeatability for different base station implementations. Unlike, traditional OTA testing with 5 test beam directions now significantly larger amount of test beams can be considered. If traditional test approaches for EIRP pattern is adopted, the test time will grow very fast. The test time can be approximated as:

Ttot=KMNTmeas+Tmove

, where K is the number of test beams, M is the number of sampling points along the vertical axis, N is the number of sampling point along the horizontal axis, Tmeas is the measurement time and Tmove is the average time to move the positioner to next location.

With following assumptions:

K equal to 25 (25 test beams according to test beam set 1)
M equal to 90 (90 samples along the vertical axis covering the upper hemisphere)
N equal to 360 (360 samples along the horizontal axis)
Tmeas equal to 20 ms (2 frames)
Tmove equal to 1 s (Average time required to move between positioner locations)

For every position location the EIRP total power is measured (If single polarization is measured the total test time is doubled). One RF channel is considered.

The total test time required for the EEIRP measurement will be Ttot equal to 229.5 h (approximately 10 days).

Observation 2: It is essential to minimize the movement of the positioner to reduce the total test time.

There are many ways to optimize test time in an OTA test range. For AAS base stations it would be reasonable to adopt a test approach where the base station is looping though different beam directions in a sequence per positioner location. With such approach the time to move the test object can be significantly reduced. The positioner will only stay in the location of each sample point only one time during the measurement, as visualised in Figure 2.2-1.

# Figure 2.2-1: Sequential beam testing

With the sequential beam testing approach the total test time can be reduced. In addition, if the beams are allocated to the test model in such a way that several beams are mapped to two frames all beams can be measured within the period of time equal to Tmeas . The total test time for the sequential test can be calculated as:

Ttot=MNTmeas+Tmove

, for sequential testing the test time Ttot is equal to 9.2 hours. Considering the time required for a product in an OTA test range including setup, configuration, calibration, and measurement. Total time of 9.2 hours. The test time can be reduced even further by applying more optimizations.

Observation 3: The test time can be reduced for EEIRP adopting a test approach where the time to move the positioner is minimized and the base station sequentially loops though the test beams per positioner location.

  - Test beam evaluation

Find the appropriate test beam set to included in TS 38.141-2 for EEIRP conformance testing require careful analysis where different aspects should be carefully considered.

For the selection of test beam set to be specified the following principles need to be considered:

- The beam set needs to include sufficient amount of beam directions for an accurate average radiation. This can be done by modelling different beam constellations and comparing the results with a reference case (e.g. an excessive number of beams distributed with the steering range).

- The considered beam set should produce a EEIRPi profile with small deviation with respect the reference case for different array antenna geometries (e.g. 3x1, 4x1 or any other sub-array structures and array size).

- The number of test beams and distribution included in the beam set should be relevant considering number of beams and distribution used for normal operation. Hence, we cannot include all beams in the layer 1 codebook, but a reasonable number of beams should be included.

- Moving test beam directions away from the steering range outer boundary is equivalent to applying a non-uniform weighting factor to the beam average.

To evaluate the impact of test beam set configuration on measured EEIRP different test beam sets (section 2.3.1) and antenna geometries (section 2.3.2) have been modelled. The deviation to a reference beam set was calculated to analyse deviations.

For this evaluation a CAR (aka. steering range) of 00≤θ≤16o in elevation and -600≤ϕ≤60o in azimuth was considered.

The sampled space for the representation of the antenna pattern is:

- 0.5 degree in the vertical plane spanning between 0 and 180 degrees.

- 1 degree in the horizontal plane spanning between -180 to 180 degrees.

    - Considered beam sets

In the following section the beam sets are visualised with vertical steering range 0 to 16 degrees.

To be able to calculate a reference EEIRP result, a beam set with a large amount of test beams and uniform weights is defined. The total number of considered beam directions within the steering range grid is 17x121 = 2057 beams, according to Figure 2.3.1-1

# Figure 2.3.1-1: Reference beam set

Beam set 1is created based on original 5 test beam directions and additional 20 test beam directions spread out uniformly within the steering range. The total number of considered directions within the steering range grid is 5x5=25 beams, according to Figure 2.3.1-2.

# Figure 2.3.1-2: Beam set 1 (25 beams in uniform constellation)

Beam set 2 is created based on original 5 test beam directions and additional 4 test beam directions spread out uniformly within the steering range. The total number of considered directions within the steering range grid is 3x3=9 beams, according to Figure 2.3.1-3.

# Figure 2.3.1-3: Beam set 2 (9 beams in rectangular constellation)

Beam set 3 is created based on original 5 test beam directions and additional 4 test beam directions spread out within the steering range, as listed in Table 2.3.1-1.

Table 2.3.1-1: Beam set 3 directions

| Beam | Down-tilt q degrees | Scan-angle j degrees |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 4 | -30 |
| 3 | 4 | 30 |
| 4 | 8 | -60 |
| 5 | 8 | 0 |
| 6 | 8 | 60 |
| 7 | 12 | -30 |
| 8 | 12 | 30 |
| 9 | 16 | 0 |

The total number of considered directions within the steering range grid is 9 beams, according to Figure 2.3.1-4.

# Figure 2.3.1-4: Beam set 3 (9 beams in diamond constellation)

Beam set 4 is created based on original 5 test beam directions and additional 12 test beam directions spread out within an ellipsoid shape within the steering range, as listed in Table 2.3.1-2.

Table 2.3.1-2: Beam set 4 directions

| Beam | Down-tilt q degrees | Scan-angle j degrres |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 2 | -45 |
| 3 | 2 | 45 |
| 4 | 6 | -15 |
| 5 | 6 | 15 |
| 6 | 4 | -30 |
| 7 | 4 | 30 |
| 8 | 8 | -60 |
| 9 | 8 | 0 |
| 10 | 8 | 60 |
| 11 | 12 | -30 |
| 12 | 12 | 30 |
| 13 | 14 | -45 |
| 14 | 14 | 45 |
| 15 | 10 | -15 |
| 16 | 10 | 15 |
| 17 | 16 | 0 |

The total number of considered directions within the steering range grid is 17 beams, according to Figure 2.3.1-5.

# Figure 2.3.1-5: Beam set 4 (17 beams in ellipsoid constellation)

    - Antenna configurations

The baseline antenna configurations are stated in Table 2.3.2-1.

Table 2.3.2-1: Antenna configurations

| Parameter | Config 1 | Config 2 | Config 3 |
| --- | --- | --- | --- |
| Vertical HPBW (degrees) | 90 | 90 | 65 |
| Horizontal HPBW (degrees) | 90 | 90 | 90 |
| Element gain (dBi) | - | - | - |
| Front-to-back ratio (dB) | 30 | 30 | 30 |
| Element power (dBm/100 MHz) | 0 | 0 | 0 |
| Number of rows | 16 | 32 | 8 |
| Number of columns | 8 | 16 | 16 |
| Sub-array size | - | - | 3x1 |
| Sub-array pre-tilt (degrees) | - | - | 6 |
| Vertical element separation (l) | 0.5 | 0.5 | 2.1 |
| Horizontal element separation (l) | 0.5 | 0.5 | 0.5 |

Observation 4: Antenna array Config 1 and Config 2 is modelling a single array antenna structure. This structure is not seen as relevant for AAS BS implementations for band n104.

    - Simulation results

In this section the deviation values will be presented for the different beam-sets compared to reference beam-set (dense grid) for all different configurations so to observe which reduced beam-set ‘replicates’ better the dense grid. A positive value of deviations means that the particular beam-set has over-estimated the EEIRP level (i.e. has calculated a higher level compared to dense grid) while a negative deviation value means that the particular beam-set has under-estimated the EEIRP (i.e. i.e. has calculated a lower level compared to dense grid). Ideally, the deviation should be zero.

For the purpose of comparison, the ‘average deviation’, ‘max over-estimation’ and ‘max under-estimation’ metrics will be used, where the average and max values are taken over the different EEIRP elevation buckets. The smaller the values the better estimation is given from the beam-set.

The simulation results are listed for all considered beam sets for different antenna configurations in Table 2.3.3-1, Table 2.3.3-2, Table 2.3.3-3 and Table 2.3.3-4.

Table 2.3.3-1: Deviations for beam set 1 (25 beams in uniform constellation)

| Elevation bin | Elevation angular range | Deviation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0o≤θ≤5o | 1.344 | 2.613 | 2.541 |
| 2 | 5o≤θ≤10o | 0.215 | 0.722 | 1.331 |
| 3 | 10o≤θ≤15o | 0.447 | -0.407 | 0.64 |
| 4 | 15o≤θ≤20o | -0.078 | 0.445 | -0.571 |
| 5 | 20o≤θ≤30o | -0.256 | -0.283 | 0.615 |
| 6 | 30o≤θ≤60o | -0.675 | -0.684 | -0.411 |
| 7 | 60o≤θ≤90o | -1.106 | -1.408 | -0.558 |
| Average deviation |  | 0.61 | 1.01 | 1.01 |
| Max over-estimation |  | 1.34 | 2.61 | 2.54 |
| Max under-estimation |  | 1.11 | 1.41 | 0.57 |

Table 2.3.3-2: Deviations for beam set 2 (9 beams in rectangular constellation)

| Elevation bin | Elevation angular range | Deviation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0o≤θ≤5o | 2.96 | 4.304 | 4.17 |
| 2 | 5o≤θ≤10o | -0.384 | 1.369 | 2.456 |
| 3 | 10o≤θ≤15o | 0.643 | -0.302 | 1.728 |
| 4 | 15o≤θ≤20o | 0.873 | 0.403 | -5.159 |
| 5 | 20o≤θ≤30o | -1.097 | -0.21 | 1.208 |
| 6 | 30o≤θ≤60o | -1.534 | -1.918 | -0.561 |
| 7 | 60o≤θ≤90o | -1.142 | -0.606 | 2.477 |
| Average deviation |  | 1.31 | 1.54 | 2.81 |
| Max over-estimation |  | 2.96 | 4.30 | 4.17 |
| Max under-estimation |  | 1.53 | 1.92 | 5.16 |

Table 2.3.3-3: Deviations for beam set 3 (9 beams in diamond constellation)

| Elevation bin | Elevation angular range | Deviation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0o≤θ≤5o | 0.187 | 1.375 | 1.376 |
| 2 | 5o≤θ≤10o | 0.176 | 0.356 | 0.546 |
| 3 | 10o≤θ≤15o | 0.33 | -0.33 | -0.266 |
| 4 | 15o≤θ≤20o | -0.051 | 0.624 | 0.372 |
| 5 | 20o≤θ≤30o | 0.079 | -0.188 | -0.029 |
| 6 | 30o≤θ≤60o | 0.035 | 0.13 | 0.525 |
| 7 | 60o≤θ≤90o | 0.064 | 0.342 | 2.68 |
| Average deviation |  | 0.13 | 0.50 | 0.92 |
| Max over-estimation |  | 0.33 | 1.38 | 2.68 |
| Max under-estimation |  | 0.05 | 0.33 | 0.27 |

Table 2.3.3-4: Deviations for beam set 4 (17 beams in ellipsoid constellation)

| Elevation bin | Elevation angular range | Deviation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0o≤θ≤5o | -0.506 | -0.409 | -0.377 |
| 2 | 5o≤θ≤10o | -0.04 | -0.066 | -0.026 |
| 3 | 10o≤θ≤15o | 0.003 | -0.019 | -0.615 |
| 4 | 15o≤θ≤20o | -0.08 | -0.038 | 0.354 |
| 5 | 20o≤θ≤30o | 0.038 | 0.049 | -0.514 |
| 6 | 30o≤θ≤60o | 0.049 | 0.041 | 0.019 |
| 7 | 60o≤θ≤90o | 0.097 | 0.062 | 0.383 |
| Average deviation |  | 0.12 | 0.10 | 0.33 |
| Max over-estimation |  | 0.10 | 0.06 | 0.38 |
| Max under-estimation |  | 0.51 | 0.41 | 0.62 |

Observation 5: It’s clear that the number of test beams and distribution of test beams within the declared steering range will affect the measured EEIRP level deviation.

All results are summarized in Table 2.3.3-5.

Table 2.3.3-5: Summary

| Beam set | Average deviation (dB) | Max over-estimation (dB) | Max under-estimation (dB) |
| --- | --- | --- | --- |
| Beam set 1 (25 beams in uniform constellation) | 1.01 | 2.61 | 1.41 |
| Beam set 2 (9 beams in rectangular constellation) | 2.81 | 4.30 | 5.16 |
| Beam set 3 (9 beams in diamond constellation) | 0.92 | 2.68 | 0.33 |
| Beam set 4 (17 beams in ellipsoid constellation) | 0.33 | 0.38 | 0.62 |

Observation 6: It is observed that Beam-set 4 (17 beams ellipsoid) has the smallest worst average deviation, by far the smallest worst over-estimation and among the smallest worst under-estimation.

Proposal 1: Based on our simulation results and considering a reasonable amount of test beams directions, we propose Beam set 4 (17 beams) to be adopted for EEIRP testing.

  - Test description

In this section information related to initial conditions, test object configuration and test object placement is captured. To prepare for specification drafting of “initial condition” and “procedure” sections in TR 38.141-2 for OTA spatial emission above horizon requirements, the structure and format from TS 38.141-2 is used as baseline.

Draft specification text for “initial conditions” and “procedure”.

x.y.4.1	Initial conditions

The test environment: Normal condition, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: M; see TS 38.141-2, subclause 4.9.1 using widest supported carrier bandwidth capable of maximum supported Power Spectral Density (PSD) level.

Beams to be tested: For each declared CAR determine the set of test beams, (red points in example below) as follows:

-	4 points on the extremes of the largest elipse/circle inside the declared CAR region

-	7 points on each diagonal, equally distanced, but exclude the extremes of the CAR region.

Figure 2.4.1-1: Example of approximate test beam position in the declared CAR region (green contour)The test object is located on the positioner with mechanical tilt set to 0 degrees.

Since this requirement is capturing power transmitted towards the sky, it is essential that the BS is mounted on the positioner such that blocking effects due to the positioner are minimized.

NOTE: 	For regular antenna pattern measurements typically, the focus is on the horizontal angle and then we often see blocking effects in along vertical angle.

For this requirement the same test model as for radiated transmit power in TS 38.141-2, subclause 6.2 should be used.

x.y.4.2	Procedure

The following high level test procedure describes essential steps required to measure the EEIRP profile.

- Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3.

- Place the test object at the positioner, such that blocking effect in the vertical domain is minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

- Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

- Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1.. N.

- Enable test beam  k for k=1..K.

- Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 6 and 7 for all K test beams.

- Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

- Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TR 38.141-2, Annex XX.

Observation 7: Since the OTA spatial emission above horizon requirement is based on an absolute power level EEIRP mask. The calibration of the test range will be essential to guarantee an acceptable measurement uncertainty.

Observation 8: A test method for EEIRP profile would require EIRP samples to be measured for many test beam directions. Therefore, it is essential to capture a description in the conformance test specification of how EIRP samples are collected for different test beam directions.

Proposal 2: Use draft specification text for “initial conditions” and “procedure” as baseline for TS 38.141-2 draft CR. RAN4 need to capture the high-level test procedure above in BS RF conformance test specification in the new sub-clause for OTA spatial emission.

Traditionally, the measurement can be divided into two different stages: Test range calibration and measurement. For an Expected EIRP measurement a third stage is introduced.

- Test range calibration

- EIRP pattern measurement

- Post processing

The post-processing stage is required for some test methods, e.g., TRP test methods which are based on spatial EIRP samples. For EEIRP, the post processing stage will be even more complex which would lead to that a new Annex would be required in the conformance test specification for the technical background information.

In the calibration stage the antenna test range transmission loss is characterized with the intention to minimize error sources related to test setup. A common approach used is the substitution method, where a reference antenna with known antenna gain is used to determine the transmission loss in the test range. The test range calibration is described in detail in TR 37.941, subclause 8.3.

In addition, the measurement uncertainty can be improved by calibrating the absolute power measurement uncertainty of the measurement receiver (spectrum analyser with fairly high absolute power measurement uncertainty) using a power meter (with very low measurement uncertainty). A detailed description of suitable test ranges and corresponding calibration procedures can be found in TR 37.941, subclause 8.3.1.

Proposal 3: RAN4 need to create entry in TR 37.941 for OTA spatial emission requirement and corresponding conformance test aspects, such as measurement procedure, calibration procedure and measurement uncertainty evaluation.

For each test beam, the EIRP pattern above the horizon (in the upper hemi-sphere) is measured for horizontal angles within the range -π≤φ≤π and elevation angles within the range 0≤θ≤π2 .

The discrete EIRP pattern EIRPkθn,φm measured per test beam using a specific sampling grid with a specific sampling grid resolution. For the reference test method described in this document a uniform sampling grid is considered. Considering practical measurement aspects, such as test complexity and test time the EIRP pattern per test beam is measured for a set of discrete spatial angles θn,φm, where m=1..M and n=1..N.

The EIRP pattern measurement stage and corresponding details on maximum allowed sampling resolution needs to be documented in an Annex in TS 38.141-2.

In technical background information from ITU-R [1] the EEIRP profile is calculated from an average EIRP pattern followed an integration over vertical bins and horizontal angles.

In the first stage the average EIRP radiation pattern over K test beams are calculated as:

EIRPθn,φm=1Kk=1KEIRPkθn,φm

, where EIRPkθn,θm  is the EIRP spectral density radiation pattern measured for the k-th test beam.

In the second average stage the EEIRP level for the i-th elevation range is calculated as:

EEIRPi=θHi-θLisinθHi-sinθLiMN∙n=nLinHim=1MEIRPθn,φmcosθn

where N  is the number of sampling points within the θLi and θHibounding range over upper hemisphere in the elevation range, nLiand nHi are the lowest and highest elevation sampling angles within the  (θLi, θHi) bounding range, with  N= nHi-nLi ; M is the number of sampling points in the azimuth range.

The post-processing stage with details description of the averaging calculations should be captured in a dedicated Annex in TS 38.141-2.

In TS 38.141-2 clause 4.14 there is a definition of reference coordinate system used for the present requirements. For this requirement, RAN4 may need to define the elevation angle according to conventions used in ITU-R.

Proposal 4: RAN4 need to describe the post-processing calculation in discrete form in the conformance test specification in new Annex.

  - Measurement uncertainty evaluation

Unlike previous OTA requirement the new requirement for protecting FSS UL is constructed as a vertical profile with different absolute Power Spectral Density (PSD) maximum levels for a set of vertical ranges (often referred to as vertical bins). This means that the requirement unlike all previous requirements relies on information over a defined vertical range with different pass/fail absolute levels. This aspect requires careful consideration in the Measurement Uncertainty (MU) evaluation part of the general description of BS RF OTA test method.

At the first glance the requirement has some similarities with OTA test methods described in TS 38.141-2, Annex I and MU evaluation captured in TR 37.941. However, the details and relevant uncertainties to be considered differ. Therefore, it is not appropriate to adopt uncertainties from TRP test methods in an Anechoic Chamber (AC) without careful analysis.

For the OTA spatial emission above horizon requirement RAN4 needs to evaluate the Measurement Uncertainty (MU) for at least one test method. It would be reasonable to start the work after the test method is completely described in a draft CR to TS 38.141-2.

The conformance testing framework adopted by RAN4 for BS RF OTA requirement is described in TR 37.941, subclause 5.1. The framework is summarized considering EEIRP as:

- Multiple test methods may exist for each requirement. For EEIRP it would be appropriate to follow guidance on test method indicated by condition defined by WRC-23. Later, other test methods can also be considered.

- Each test method will require its own test procedure.

- A single conformance requirement applies for each core requirement, regardless of test procedure.

- Common maximum accepted test system uncertainty applies for all test methods addressing the same test requirement. Test methods producing significantly worse uncertainty than others at comparable cost should not impact the common maximum accepted test system uncertainty assessment.

- Common Test Tolerances (TT) apply for all test methods addressing the same test requirement. For EEIRP the requirement is defined by regulation, hence the TT is set to zero.

- A common way of establishing the uncertainty result from all test methods' individual budgets is established.

- A common method of making an uncertainty budget (not a common uncertainty budget) is established.

- Establish budget format examples for each addressed test method in the form of lists of uncertainty contributions. Contributions that may be negligible with some BS and substantial with others should be in this list. For each combination of measurement method and test parameter, develop a list with measurement uncertainties.

- Describe potential OTA test methods relevant for testing radiated requirements (e.g. directional, TRP, or co-location requirements). The description requires information about the applicable test range architectures and test procedures. Addressing each item in each uncertainty budget with respect to the expected distribution of the errors, the mechanism creating the error and how it interacts with properties of the BS. The EEIRP requirement is a new type of requirement based on a spatial requirement pass/fails mask.

- Providing example uncertainty budgets will be useful in order to demonstrate the way a budget should be defined and how calculating its resulting measurement uncertainty is done, but the figures used in the examples will clearly be only examples and not applicable in general.

- Each test instance may require an individual uncertainty budget applicable for the combination of the test facility, the BS and the test procedure and property tested. Here, the tester demonstrates that the uncertainty requirement is fulfilled during the conformance testing.

Further details on how the uncertainty calculation is done can be found in TR 37.941, subclause 5.2.

Observation 9: For EEIRP test method development it is essential to continue the direction set out in the outcome from WRC-23. Based on WRC-23 guidance define a test method and corresponding MU evaluation. At a later stage, if needed document additional test methods.

Observation 10: Since the EEIRP requirement cannot be categorized as Directional requirement, TRP requirement or Co-location requirement. It would be appropriate to define a new separate category for EEIRP based requirements.

Observation 11: The OTA spatial emission requirement is a regulatory requirement, the TT=0 which would result in that eventual MU needs to be accounted for in the design as power back-off or other mitigation.

For the OTA spatial emission requirement above horizon based on EEIRP following technical test challenges should be considered in the MU evaluation:

- The MU impact due to limited measurement receiver dynamic range. Since the EEIRP requirement profile is based on multiple vertical bins with different absolute power level the impact on OTA test range dynamic range should be studied.

- The MU impact due to limited number of used test beam directions. It has been concluded that the expected MU will depend on the number of considered test beams and where they are located in the CAR.

- The MU impact due to selected distribution of test beam directions and usage of the beam weight. If a concept with declarable beam weights is adopted the final MU will be declarable. This approach will not be accepted by regulation.

- The MU impact due to the selected EIRP patten spatial sampling resolution. The average calculation requires a certain number of samples to guarantee small variations. RAN4 need to define required maximum allowed sampling resolution.

- The MU impact due to number of power samples measured by the measurement receiver per test beam.

- The MU impact due to the absolute power measurement uncertainty determined by the calibration process.

- The MU impact due to positioner blocking effects in the vertical domain.

Proposal 5: Before starting work with MU evaluation, first focus on BS RF core draft specification text and BS RF conformance specification text. When draft CRs for TS 38.104 and TS 38.141-2 are complete then focus on measurement method MU evaluation in TR 37.941.

As a baseline for further discussions, a MU table for EEIRP have been created in Table 2.5-1. The starting point for EEIRP have been influenced by the MU evaluation for BS output power (TRP) measured in a CATR chamber. Even though EEIRP and TRP is related to each other, some additional work is required to figure out measurement uncertainty contribution that is unique for EEIRP.

Table 2.5-1: Baseline for EEIRP MU table

| UID | Uncertainty source | Uncertainty value (dB) | Distribution of the | Divisor based on | ci | Standard uncertainty ui (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 6425 < f ≤ 7125 MHz | probability | distribution shape |  | 6425 < f ≤ 7125 MHz |
| Stage 2: BS measurement |  |  |  |  |  |  |
| TBD | Misalignment and pointing error of BS | 0.30 | Rectangular | 1.73 | 1 | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.26 | Normal | 1.00 | 1 | 0.260 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | U-shaped | 1.41 | 1 | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Normal | 1.00 | 1 | 0.001 |
| A2-4a | QZ ripple experienced by BS | 0.09 | Normal | 1.00 | 1 | 0.093 |
| A2-12 | Frequency flatness of test system | 0.25 | Normal | 1.00 | 1 | 0.250 |
|  | [Sampling error] |  |  |  |  |  |
|  | [Dynamic range error] |  |  |  |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.20 | Normal | 1.00 | 1 | 0.20 |
| A2-5a | Mismatch or receiver chain between receiving antenna and measurement receiver | 0.33 | U-shaped | 1.41 | 1 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | Rectangular | 1.73 | 1 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Normal | 1.00 | 1 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | U-shaped | 1.41 | 1 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.43 | Rectangular | 1.73 | 1 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | Exp. normal | 2.00 | 1 | 0.00 |
| TBD | Misalignment and pointing error of calibration antenna | 0.50 | Exp. normal | 2.00 | 1 | 0.25 |
| A2-9 | Rotary joints | 0.05 | U-shaped | 1.41 | 1 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | U-shaped | 1.41 | 1 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna | 0.01 | Normal | 1.00 | 1 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | Rectangular | 1.73 | 1 | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  | TBD |

MU IDs marked in “red” required further discussion. MU IDs marked in “orange” require confirmations from test equipment vendors. Some new MU contribution sources unique for EEIRP is added within [].

Observation 12: Additional work is needed to establish a complete MU evaluation table for EEIRP.

  - EIRP pattern resolution

Considering the requirement for OTA spatial emission above the horizon there are two aspects to consider in relation on how to define the EIRP pattern spatial resolution required to guarantee accurate measurement of EEIRP:

- The sampling resolution required to capture the radiation above the horizon. For this sufficient number of spatial samplers are needed to sufficiently capture emission generated in the upper hemisphere including side lobes and grating lobes.

- The size of the vertical bins varies over the elevation range from 5 degrees at low elevations to 30 degrees at high elevation angles. This means that the number of vertical samples with is limited by the bin size.

The first aspect can be resolved by taking enough spatial samples required the spatial sampling Nyquist criteria formulated as:

Δθ<λD180π

, where D is the diameter of a sphere enclosing the radiating source in m (in other words the antenna aperture of the base station) and l is the wavelength in m. The diameter D is calculated as:

D=w2+h2+d2

, where w is the radiating aperture width equal to Ndh and h is the radiating aperture height equal to MsubMdv and d is the depth of the BS enclosure (here assumed to be 0.1 m).

To investigate the required sampling resolution, different antenna geometries have been considered. In Table 2.6-1, the required sampling resolution is listed for band n104 centre frequency (6775 MHz) for different array antenna geometries assuming the base station enclosure depth of 0.1 m.

Table 2.6-1: Required maximum sampling resolution

| Array antenna configuration (MsubxNsub, MxN) | Diameter D (m) | Element separation (m) dv, dh | Required sampling resolution (degrees) Δθ |
| --- | --- | --- | --- |
| 3x1, 4x8 | 0.42 | 0.7l, 0.5l | 6.0 |
| 3x1, 8x16 | 0.83 | 0.7l, 0.5l | 3.0 |
| 3x1, 16x32 | 1.65 | 0.7l, 0.5l | 1.5 |

Considering the integration method for the second stage of EEIRP calculation from WRC-23 and different types of array antenna sizes it would be safe to define maximum allowed resolution to 1 degree.

With 1 degree resolution the average process within respective bin is also secured. Even for the narrowest bin with only 5 samples and average value can be calculated.

Proposal 5: Based on technical analysis considering array sizes suitable for operation within band n104 the sampling resolution can be set to 1.0 degree both along vertical axis and horizontal axis.

Alternative, the specification can describe how the sampling resolution can be determined based on the physical size of the antenna array aperture. Such approach would require additional declarations to be introduced.

  - Indicative measurement results

Using the test method described in previous sections in this contribution we have conducted a measurement of the EEIRP profile.

For this purpose, AAS base station test object operating at 3.5 GHz was selected. For simplicity, 14 test beams based on different PMI beams from the layer 1 codebook was selected. The selected test object was not designed to meet the EEIRP requirement for band n104.

The PMI was selected to spread beams both along vertical and horizontal axis, as visualized in Figure 2.7-1. For each test beam, a full sphere EIRP pattern was measured with 1 degree resolution along vertical and horizontal axis.

# Figure 2.7-1: Full sphere pattern for each test beam

Based on the 14 EIRP patterns, the average EIRP pattern is calculated and plotted in left plot in Figure 2.7-2. The measured EEIRP profile is plotted in right plot in Figure 2.7-2.

# Figure 2.7-2: Average EIRP pattern (left) and EEIRP profile (right)

The indicative measurement result presented here shows that EEIRP can be measured in a traditional OTA test range with some modifications to minimize test time. It can be concluded that EEIRP can be tested within reasonable total test time.

# Conclusion

In this contribution we present an overview of the new requirement required to protect FSS UL within band n104 and aspects to consider when the requirement is included in RAN4 BS specifications.

Based on the information in this contribution we have observed following:

Observation 1: With the introduction of OTA spatial emission requirement RAN4 may need to consider how beamforming weights should be applied to the test model to guarantee a relevant test condition. If required RAN4 need to capture a description of how beam weights associated to test beam directions are applied to the test model signal in TS 38.141-2.

Observation 2: It is essential to minimize the movement of the positioner to reduce the total test time.

Observation 3: The test time can be reduced for EEIRP adopting a test approach where the time to move the positioner is minimized and the base station sequentially loops though the test beams per positioner location.

Observation 4: Antenna array Config 1 and Config 2 is modelling a single array antenna structure. This structure is not seen as relevant for AAS BS implementations for band n104.

Observation 5: It’s clear that the number of test beams and distribution of test beams within the declared steering range will affect the measured EEIRP level deviation.

Observation 6: It is observed that Beam-set 4 (17 beams ellipsoid) has the smallest worst average deviation, by far the smallest worst over-estimation and among the smallest worst under-estimation.

Observation 7: Since the OTA spatial emission above horizon requirement is based on an absolute power level EEIRP mask. The calibration of the test range will be essential to guarantee an acceptable measurement uncertainty.

Observation 8: A test method for EEIRP profile would require EIRP samples to be measured for many test beam directions. Therefore, it is essential to capture a description in the conformance test specification of how EIRP samples are collected for different test beam directions.

Observation 9: For EEIRP test method development it is essential to continue the direction set out in the outcome from WRC-23. Based on WRC-23 guidance define a test method and corresponding MU evaluation. At a later stage, if needed document additional test methods.

Observation 10: Since the EEIRP requirement cannot be categorized as Directional requirement, TRP requirement or Co-location requirement. It would be appropriate to define a new separate category for EEIRP based requirements.

Observation 11: The OTA spatial emission requirement is a regulatory requirement, the TT=0 which would result in that eventual MU needs to be accounted for in the design as power back-off or other mitigation.

Observation 12: Additional work is needed to establish a complete MU evaluation table for EEIRP.

To further progress the work, we propose following:

Proposal 1: Based on our simulation results and considering a reasonable amount of test beams directions, we propose Beam set 4 (17 beams) to be adopted for EEIRP testing.

Proposal 2: Use draft specification text for “initial conditions” and “procedure” as baseline for TS 38.141-2 draft CR. RAN4 need to capture the high-level test procedure above in BS RF conformance test specification in the new sub-clause for OTA spatial emission.

Proposal 3: RAN4 need to create entry in TR 37.941 for OTA spatial emission requirement and corresponding conformance test aspects, such as measurement procedure, calibration procedure and measurement uncertainty evaluation.

Proposal 4: RAN4 need to describe the post-processing calculation in discrete form in the conformance test specification in new Annex.

Proposal 5: Based on technical analysis considering array sizes suitable for operation within band n104 the sampling resolution can be set to 1.0 degree both along vertical axis and horizontal axis.

# References

[1]	WRC-23, Final Acts - Resolution 220: Terrestrial component of International Mobile Telecommunications (IMT) within the frequency band 6 425-7 125 MHz

TP to TR 39.908

Clause 4:

# Background

The frequency band of 6 425 to 7125 MHz and parts thereof was identified for International Mobile Telecommunications (IMT) use by WRC 23 in different ITU region as in the RR Footnotes 5.6A12, 5.6B12 and 5.6C12 with associated technical condition of limits on the Expected Equivalent Isotropically Radiated Power (EEIRP) spectral density of IMT base-stations for protecting Earth-to-space Fixed Satellite Services (FSS) as in Resolution 220 (WRC-23). The EEIRP is subject to a new regulatory requirement specified as a mask for over the horizon emissions. The Annex to the Resolution 220 (WRC-23) [1] outlines a calculation of the EEIRP of an IMT base station for assessing the compliance of the IMT base station equipment with the  EEIRP limits.

The aim of this report is the following:

- To capture the technical background information relevant for the core requirement in BS RF core specification (i.e. TS 38.104 and other nodes like IAB, Repeater and NCR)

- To capture the technical background information relevant for the test procedures in the conformance test specification (i.e. TS 38.141-2 and other nodes like IAB, Repeater and NCR).

Having the requirement included in a 3GPP standard would guarantee a harmonized terminology and conformance test methodologies. Eventually, the concept for conformance testing can be adopted to similar situations involving co-channel spectrum sharing between IMT and FSS UL.

As part of the WRC-23, resolution 220, EEIRP for an IMT BS was introduced based on calculating average radiating power for seven elevation bins. Since the protection of FFS UL receiver is based on aggregated power from many base stations, the requirement breakdown to a single BS will be based on an average EIRP pattern for a single BS, as used during normal operation. WRC-23 decided to adopt a concept where the average EIRP pattern is extracted based on measuring the EIRP pattern for many sequentially generated narrow beams. The choice of these narrow beams should be relevant for the coverage area considered for the BS.

Hence, there will be a link between the time over which the average is calculated and the number of beams included in the average EIRP pattern used as input for the EEIRP calculation.

Sub-clause 6.3:

## 6.3	Test beams

The BS capability to provide coverage for different deployment situations is declared by the BS manufacturer. The declaration relying on declaring parameters for minimum angle along the q axis, maximum angle along the q axis, minimum angle along j axis and maximum angle along j axis, as vizulised in Figure 6.3-1. The BS manufacture can declare multiple steering ranges to be used for different types of deplymet scenarios.

# Figure 6.3-1: Declaration of steering range

{Editors note: Equations for each test beam direction to be added in Table 6.3-1 when test beam constellation is established}

The test beam directions for declared steering range is listed in Table 6.3-1.

Table 6.3-1: Test beam directions

| Beam | Down-tilt q degrees | Scan-angle j degrres |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |
| 16 |  |  |
| 17 |  |  |

Sub-clause 6.4:

## Spatial sampling grid

Considering the requirement for OTA spatial emission above the horizon there are two aspects to consider in relation on how to define the EIRP pattern spatial resolution required to guarantee accurate measurement of EEIRP:

- The sampling resolution required to capture the radiation above the horizon. For this sufficient number of spatial samplers are needed to sufficiently capture emission generated in the upper hemisphere including side lobes and grating lobes.

- The size of the vertical bins varies over the elevation range from 5 degrees at low elevations to 30 degrees at high elevation angles. This means that the number of vertical samples with is limited by the bin size.

The first aspect can be resolved by taking enough spatial samples required the spatial sampling Nyquist criteria formulated, similar as for TRP (see TS 38.141-2, Annex I).

Considering the minimum bin size of 5 degrees in the WRC-23 outcome, it would be safe to set the maximum vertical sampling resolution to 1 degree.

For the horizontal sampling resolution, the maximum sampling resolution can be determined by the physical size of the array antenna panel or the narrowest beamwidth produced by the array antenna. Both requires additional manufacturer declarations.

Sub-clause 6.5:

## 6.5	Method of test

In this section information related to initial conditions, test object configuration and test object placement is captured. To prepare for specification drafting of “initial condition” and “procedure” sections in TR 38.141-2 for OTA spatial emission above horizon requirements, the structure and format from TS 38.141-2 is used as baseline.

Draft specification text for “initial conditions” and “procedure”.

x.y.4.1	Initial conditions

The test environment: Normal condition, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: M; see TS 38.141-2, subclause 4.9.1 using widest supported carrier bandwidth capable of maximum supported Power Spectral Density (PSD) level.

Beams to be tested: For a declared steering range test specified test beams, see TS 38.141-2, Annex X.

The test object is located on the positioner with mechanical tilt set to 0 degrees.

Since this requirement is capturing power transmitted towards the sky, it is essential that the BS is mounted on the positioner such that blocking effects due to the positioner are minimized.

NOTE: 	For regular antenna pattern measurements typically, the focus is on the horizontal angle and then we often see blocking effects in along vertical angle.

For this requirement the same test model as for radiated transmit power in TS 38.141-2, subclause 6.2 should be used.

x.y.4.2	Procedure

The following high level test procedure describes essential steps required to measure the EEIRP profile.

1. Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3

2. Place the test object at the positioner, such that blocking effect in the vertical domain is minimized.

3. Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

4. Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

5. Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1.. N.

6. Enable test beam  k for k=1..K.

7. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

8. Repeat step 6 and 7 for all K test beams.

9. Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

10. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TR 38.141-2, Annex Y.

Traditionally, the measurement can be divided into two different stages: Test range calibration and measurement. For an expected EIRP measurement a third stage is introduced.

Test range pathloss calibration and measurement receiver absolute power level calibration

EIRP pattern measurement

Post processing

The post-processing stage is required for some test methods, e.g., TRP test methods which are based on spatial EIRP samples. For EEIRP, the post processing stage will be even more complex which would lead to that a new Annex would be required in the conformance test specification for the technical background information.

In the calibration stage the antenna test range transmission loss is characterized with the intention to minimize error sources related to test setup. A common approach used is the substitution method, where a reference antenna with known antenna gain is used to determine the transmission loss in the test range. The test range calibration is described in detail in TR 37.941, subclause 8.3.

In addition, the measurement uncertainty can be improved by calibrating the absolute power measurement uncertainty of the measurement receiver (spectrum analyser with fairly high absolute power measurement uncertainty) using a power meter (with very low measurement uncertainty). A detailed description of suitable test ranges and corresponding calibration procedures can be found in TR 37.941, subclause 8.3.1.

For each test beam, the EIRP pattern above the horizon (in the upper hemi-sphere) is measured for horizontal angles within the range -π≤φ≤π and elevation angles within the range 0≤θ≤π2 .

The discrete EIRP pattern EIRPkθn,φm measured per test beam using a specific sampling grid with a specific sampling grid resolution. For the reference test method described in this document a uniform sampling grid is considered. Considering practical measurement aspects, such as test complexity and test time the EIRP pattern per test beam is measured for a set of discrete spatial angles θn,φm, where m=1..M and n=1..N.

The EIRP pattern measurement stage and corresponding details on maximum allowed sampling resolution needs to be documented in an Annex in TS 38.141-2.

In technical background information from WRC-23 the EEIRP profile is calculated from an average EIRP pattern followed an integration over vertical bins and horizontal angles.

In the first stage the average EIRP radiation pattern over K test beams are calculated as:

EIRPθn,φm=1Kk=1KEIRPkθn,φm

, where EIRPkθn,θm  is the EIRP spectral density radiation pattern measured for the k-th test beam.

In the second average stage the EEIRP level for the i-th elevation range is calculated as:

EEIRPi=θHi-θLisinθHi-sinθLiMN∙n=nLinHim=1MEIRPθn,φmcosθn

where N  is the number of sampling points within the θLi and θHibounding range over upper hemisphere in the elevation range, nLiand nHi are the lowest and highest elevation sampling angles within the  (θLi, θHi) bounding range, with  N= nHi-nLi ; M is the number of sampling points in the azimuth range.

The post-processing stage with details description of the averaging calculations should be captured in a dedicated Annex in TS 38.141-2.

In TS 38.141-2 clause 4.14 there is a definition of reference coordinate system used for the present requirements. For this requirement, RAN4 may need to define the elevation angle according to conventions used in ITU-R.

Sub-clause 6.6:

## 6.6	Measurement uncertainty

The Measurement Uncertianty (MU) evalaution for EEIRP measured in CATR is captured in Table 6.6-1. The MU evaluation is based on TRP in CATR from TR 37.941 with adaptations required for EEIRP.

For EEIRP MU evaluation some new error sources have been added to account for dynamic range errors due to the fact that spatial emission off main beam is measured and errors related to the selection of the test beam constellation.

Table 6.6-1: EEIRP in CATR

| UID | Uncertainty source | Uncertainty value (dB) | Distribution of the | Divisor based on | ci | Standard uncertainty ui (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 6425 < f ≤ 7125 MHz | probability | distribution shape |  | 6425 < f ≤ 7125 MHz |
| Stage 2: BS measurement |  |  |  |  |  |  |
| TBD | Misalignment and pointing error of BS | 0.30 | Rectangular | 1.73 | 1 | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.26 | Normal | 1.00 | 1 | 0.260 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | U-shaped | 1.41 | 1 | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Normal | 1.00 | 1 | 0.001 |
| A2-4a | QZ ripple experienced by BS | 0.09 | Normal | 1.00 | 1 | 0.093 |
| A2-12 | Frequency flatness of test system | 0.25 | Normal | 1.00 | 1 | 0.250 |
|  | [Dynamic range error] |  |  |  |  |  |
|  | [Sampling error] |  |  |  |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.20 | Normal | 1.00 | 1 | 0.20 |
| A2-5a | Mismatch or receiver chain between receiving antenna and measurement receiver | 0.33 | U-shaped | 1.41 | 1 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | Rectangular | 1.73 | 1 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Normal | 1.00 | 1 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | U-shaped | 1.41 | 1 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.43 | Rectangular | 1.73 | 1 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | Exp. normal | 2.00 | 1 | 0.00 |
| TBD | Misalignment and pointing error of calibration antenna | 0.50 | Exp. normal | 2.00 | 1 | 0.25 |
| A2-9 | Rotary joints | 0.05 | U-shaped | 1.41 | 1 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | U-shaped | 1.41 | 1 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna | 0.01 | Normal | 1.00 | 1 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | Rectangular | 1.73 | 1 | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  | TBD |