# 3GPP TSG-RAN WG4 Meeting #113	R4-2419622

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.8.3.1

Source: 	Samsung

Title: 	System parameters for Less than 5MHz supporting in NTN

Document for:	Discussion

1		Introduction

For Rel-19 Enhanced requirements and conductive test methodology for NR NTN and IoT NTN WI, supporting 3MHz CHBW for NTN operation is one of major objective with detailed WID as following:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signalling to support the above objectives

In this contribution, views on remaining open issues as list in the WF [2] were provided:

System parameters Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - Way forward: - Option 1: Mandatory - Option 2: Optional Issue 1-2: Clarification on maximum transmission bandwidth configuration - Agreement: - Follow the previous agreement in WF R4-2413528 as: Issue 1-1-3: Maximum transmission bandwidth configuration(NRB) - Agreement - Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN.. Issue 1-3: Signalling of SS raster - Way forward: - Option 1: The exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations. - Option 2: Defer the UE capability discussion for 3MHz in FR1-NTN after all the requirements are clear. - Option 3: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN.

2		Discussion

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

3MHz CHBW was introduced with punctured PBCH (12 RB). The sync raster location for 3MHz CHBW is separate compared to normal sync raster with other CHBWs.

Supporting 3MHz CHBW is required additional UE processing complexity in terms of initial cell search on new sync raster, baseband processing on punctured PBCH/control channel, RF capability of supporting 3MHz CHBW.

For TN specification, UE feature list (51-1/-2) were introduced for supporting 3MHz CHBW and 12 RB coreset #0 under NR_FR1_lessthan_5MHz_BW WI. And in TN specification, supporting 3MHz CHBW was agreed as optional feature as per band basis.

Supporting this feature can be release independent to Rel-18 given 3MHz PBCH channel was introduced in Rel-18. For mandatory or optional in Rel-19, we prefer to be optional meanwhile we can consider to mandate per band basis in RAN4 specification if strong demands justified.

| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2] for TN FR1 bands and clause 5.3.6 of TS 38.101-5 [x] for FR1 NTN bands.. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2] for FR1 TN bands and clause 5.3.6 of TS 38.101-5 [x] for FR1 NTN bands, this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2] for FR1 TN bands and 5.4.3.3-X in TS 38.101-5 [x] for FR1 NTN bands . This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |

Regarding UE capability, either extend existing UE feature list/capability signalling (51-1, 51-2) to cover NTN bands as per band basis or new separate UE feature list can be introduced for NTN 3MHz CHBW similar as TN. We believe extending existing UE feature list shall be more proper way given this feature was introduced by RAN1 with basic physical layer update which for NTN operation we reuse the design from TN without any changes on RAN1.

Proposal 1: Support 3MHz CHBW for NR-NTN in FR1-NTN bands is optional feature with capability signalling as per FR1 NTN band basis in general.

- Further discuss whether can be mandatory on certain bands with Rel-19 onwards as per band basis if strong needs justified.

Proposal 2: Extend existing UE capability signaling “support3MHz-ChannelBW-Symmetric-r18” and “support3MHz-ChannelBW-Asymmetric-r18” to cover NTN FR1 bands.

3		Conclusion

In this contribution, we provide views on remaining open issues for supporting 3MHz.

Proposal 1: Support 3MHz CHBW for NR-NTN in FR1-NTN bands is optional feature with capability signalling as per FR1 NTN band basis in general.

- Further discuss whether this feature can be mandatory on certain bands with Rel-19 onwards as per band basis if strong needs justified.

Proposal 2: Extend existing UE capability signaling “support3MHz-ChannelBW-Symmetric-r18” and “support3MHz-ChannelBW-Asymmetric-r18” to cover NTN FR1 bands.

We also provide a template for work split of CR drafting on supporting 3MHz CHBW to trigger discussion on work split in RAN4#113 meeting at BDaT session.

| TS 38.101-5 NTN UE RF |  |  |
| --- | --- | --- |
| Sections | Scope | Volunteer on CR drafting |
| Big CR | Big draft CR/CR on TS 38.101-5 UE RF |  |
| Section 5 | System parameters |  |
| Section 6.2 | UE Tx transmitted power requirements |  |
| Section 6 except 6.2 | Other Tx RF requirements except 6.2 |  |
| Section 7 | UE Rx RF requirements |  |

Reference

[1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2] R4-2416587, Way Forward for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi