# 3GPP TSG-RAN WG4 Meeting #113		        R4-2419636

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	6.20.2

Source: 	Huawei, HiSilicon

Title:	On UE RF requirements for band n28

Document for:	Approval

# 1	Introduction

This is the last meeting planned for the WI on introducing PC2 and 40MHz CBW for band n28 [1]. This paper continues to discuss the remaining open issues in the WF [2].

# 2	Discussion

Issue 1-2-2 NS_18 A-MPR for PC2 for BW<=40MHz

- Agree on the regions in Table 1 and 2.

- For A-MPR values in Table 3:

- Further check the A-MPR values for 2Tx

- Yellow highlighted values will be further confirmed by measurements.

- Values in [] are to be confirmed

For the current NS_18 A-MPR requirements, the frequency range of UL transmission bandwidth configuration is limited to 703-733MHz. Hence, it’s unnecessary to expand the frequency range to 703-748 as described in the WF.

Proposal 1: For NS_18 A-MPR requirements, limit the frequency range of channel bandwidth to 703-743.04 MHz for BW=25/30/40MHz.

More explicitly,

Table 1: PC3 region definition for 25MHz, 30MHz and 40MHz channel bandwidth

| Channel Bandwidth, MHz | Frequency range of channel bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~743.04 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | 703~743.04 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
| 40 | 703~743.04 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |

Table 2: PC2 region definition for 25MHz, 30MHz and 40MHz channel bandwidth

| Channel Bandwidth, MHz | Frequency range of channel bandwidth, MHz | Regions (PC2) |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~743.04 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~743.04 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 40 | 703~743.04 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 ≤(LCRB*12*SCS)/2+11.7 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

Issue 1-2-6 channel location

- Update the NOTE 7 as follows:

  - NOTE 7:	For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] 703-733MHz or 718-748MHz [for the 20, 25 and 30MHz bandwidth].

- Introduce the new note as follows:

  - For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] [703-743.04MHz] or 718-748MHz [for the [20, 25, and] 30MHz bandwidth. And for the 40MHz bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz.]

- FFS the content in []

Essentially, there’re two options for the wording of the two notes.

Option 1:

  - NOTE 7: For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-733MHz or 718-748MHz for the 20, 25 and 30MHz bandwidth.

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz or 718-748MHz for the 30MHz channel bandwidth. And for the 40MHz bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to 703-743.04MHz.

Option 2:

  - NOTE 7: For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to 703-733MHz or 718-748MHz.

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz or 718-748MHz.

Option 1 is an extension of the current wording used in the spec, while option 2 is much simplified. Technically, the two options are equivalent after considering the underlying dual-duplexer assumption.

Proposal 2: Regarding the channel location, adopt the simplified wording, i.e.,

  - NOTE 7: For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to 703-733MHz or 718-748MHz.

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz or 718-748MHz.

Issue 1-2-7 channel raster

- Alt1: Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

- Alt2: Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz.

Since a UE is mandated to support the enhanced channel raster capability on band n28 from Rel-18, it’s feasible to adopt Alt2 in support of 40MHz CBW.

As a result, the dependency on the enhanced channel raster capability should be stated in the spec, so that UE implementations utilising the release independence mechanism are aware of this requirement.

Proposal 3: Adopt the enhanced channel raster capability in support of 40MHz CBW, and add this requirement to the new note as follows in order to facilitate release independence:

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz or 718-748MHz. The UE shall also support the enhanced channel raster capability on band n28.

Issue 1-2-8 Release independence

Agreement:

- Option 1: Rel-16

- Option 2: Rel-15

As per the RRC spec, the capability of enhanced channel raster is early implementable from Rel-16.

Since the 40MHz CBW would depend on the enhanced channel raster capability, the earliest version for release independence is Rel-16.

Proposal 4: Release independence from Rel-16 for 40MHz CBW can be supported.

# 3	Conclusion

In this paper, we continue to discuss the remaining open issues for the Rel-19 WI on band n28, and make the following proposals:

Proposal 1: For NS_18 A-MPR requirements, limit the frequency range of channel bandwidth to 703-743.04 MHz for BW=25/30/40MHz.

Proposal 2: Regarding the channel location, adopt the simplified wording, i.e.,

  - NOTE 7: For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to 703-733MHz or 718-748MHz.

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz or 718-748MHz.

Proposal 3: Adopt the enhanced channel raster capability in support of 40MHz CBW, and add this requirement to the new note as follows in order to facilitate release independence:

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz or 718-748MHz. The UE shall also support the enhanced channel raster capability on band n28.

Proposal 4: Release independence from Rel-16 for 40MHz CBW can be supported.

# References

- RP-241660 New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN

- R4-2417082 WF on PC2 and 40MHz CBW for n28, CMCC, RAN4#112bis