3GPP TSG-RAN4 Meeting #113	R4-2419648

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on Ku-band UE RF requirements

Source:	Samsung

Agenda item:	7.9.4

Document for:	Approval

1	Introduction

In the RAN4#112bis meeting, the WF R4-2416563 [1] has captured the agreed way forward for the open issues for UE RF of Ku-band NTN. While most of the open issues are related to the system parameters discussion and co-existence outcomes, we’d like to express our views for some of the remaining open issues.

2	Tx requirements

In WF [1], one of the open issues is the maximum TRP and EIRP requirements, and the wayforward was agreed as follows:

Issue 1-2-1: Maximum TRP and EIRP for the NTN Ku band - FFS on the following options - Option 1: No maximum TRP and EIRP defined - Option 2: Maximum TRP and EIRP need to be defined, same approach as the NTN Ka band in the current specification - Option 3: Define maximum TRP and EIRP based on declaration by manufacturer

In our view, the minimum peak EIRP is a minimum requirement to guarantee the UL link budget to reach an agreed minimum performance and to guarantee the target use scenario. While the maximum TRP and EIRP requirement was more about to make sure the transmitted power will follow the requirement defined by the regulatory bodies.

Moreover, from co-ex study point of view, the higher Tx TRP and/or EIRP would also inroduce higher interference into the adjacent channel, which may require different levels of ACLR or ACS to mitigate the interference. In that sense, it may introduce un-acceptable interference into the adjacnet channel system if the UE would adopt a higher Tx EIRP than it has been studied.

Observation 1: From interference point of view, the NTN VSAT may create harmful interference to the adjacent channel system if it transmit in a higher Tx EIRP level than the power level that had been studied.

Proposal 1: We prefer to have the maximum TRP and EIRP specified in the spec, to ensure the maximum EIRP assumed in co-ex study and the ACLR/ACS derived from that study can be meaningful.

For the antenna patterns and modelling, we had proposed a proposal for co-ex study which is to use the parabolic antenna model provided in TR 38.811 as starting point to progress the study.

In WF[1], it is listed as a open issue which is to:

Issue 1-2-2: Antenna patterns and modeling • Encourage Ku-band VSAT vendors to contribute the flat-panel or phase-array antenna patterns and modeling.

And considering the off-axis eirp denstiy limits was agreed to be considered as in [1]:

Issue 1-2-4: Off-axis EIRP density limits • For the UL band 14.0 - 14.5 GHz (E-to-S) for region 1: o The off-axis EIRP requirements standardized by the ETSI should be considered for Ku-Band VSAT UE.  Fixed VSAT based on EN 301 428  Mobile VSAT based on EN 302 340, EN 302 977

Observation 2: The off-axis requirements, including EIRP density limit and spurious requirement, would have impacts on maximum EIRP and TRP. And in order to meet such regulatory requirment from regulatory bodies, like ETSI or FCC, the applicable antenna model should be specified first and then maybe the maximum EIRP and TRP can be introduced properly.

With the above consideration, in last meeting, we proposed to encourage companies to provide the phase array antenna modeling and its typical parameters.

3	Conclusion

For the NTN Ku-band VSAT RF requirements, we proposed the following.

Observation 1: From interference point of view, the NTN VSAT may create harmful interference to the adjacent channel system if it transmit in a higher Tx EIRP level than the power level that had been studied.

Proposal 1: We prefer to have the maximum TRP and EIRP specified in the spec, to ensure the maximum EIRP assumed in co-ex study and the ACLR/ACS derived from that study can be meaningful.

Observation 2: The off-axis requirements, including EIRP density limit and spurious requirement, would have impacts on maximum EIRP and TRP. And in order to meet such regulatory requirment from regulatory bodies, like ETSI or FCC, the applicable antenna model should be specified first and then maybe the maximum EIRP and TRP can be introduced properly.

4	Reference

[1] R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL