3GPP TSG-RAN WG4 Meeting #113									                        R4-2419662

Orlando, US, November 18 – 22, 2024

Agenda item:			7.8.3.2

Source:	Ericsson

Title:	On UE RF requirements for less than 5 MHz for FR1-NTN

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In RAN4#112-bis, the WF has been approved in [2]. In this contribution, we focus on the remaining UE RF requirements for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands and we provide our view on the open issues.

# Discussion

## Tx requirements

Other than A-MPR requirements, the following issues are still open:

Issue 2-1-9: General spurious emission

- Way forward

  - FFS

In our view, NTN should reuse the TN UE approach for 3 MHz channel bandwidth, i.e. define OOB boundary FOOB to be 6 MHz.

Proposal 1: For general spurious emissions, reuse the TN UE approach for 3 MHz channel bandwidth, i.e. define OOB boundary FOOB to be 6 MHz.

Additional spurious emissions requirements are FFS in the approved WF [2]:

Issue 2-1-10: Additional spurious emissions

- Way forward

  - Option 1: New requirements need define for NS_02N, 03N, 04N, 05N

  - Option 2: For the additional spurious emissions requirements, add 3 MHz CBW into

    - Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

    - Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

    - Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N".

    - Add Table 6.5.3.3.5-1 Additional out-of-band requirements for "NS_24"

  - Option 3: Introduce 3 MHz channel bandwidth for all network signalling values

Each of NS_02N, NS_03N, NS_04N, NS_24 and NS_05N is associated with the additional spurious emissions requirements (originating from ETSI or FCC regulatory requirements), which are common for all channel bandwidths since those regulatory requirements are defined for ranges of frequencies. 3 MHz CBW can also fit into the same ranges, and thus we propose:

Proposal 2: For the additional spurious emissions requirements, add 3 MHz CBW into:

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N"

- Add Table 6.5.3.3.5-1 Additional out-of-band requirements for "NS_24"

## Rx requirements

Regarding the REFSENS requirements, the following was captured in the approved WF [2]:

Issue 2-2-1: Reference sensitivity

- Way forward

  - Option 1: 2.2dB improvement compared with 5 MHz CBW;

  - Option 2: 1.7dB improvement compared with 5 MHz CBW;

For the reference sensitivity requirements (REFSENS), 3 MHz CBW shall be added to Table 7.3.2-1. For narrower channel bandwidths, such as 3MHz, the impact of IIP2 (i.e. self-mixing and effect of the 2nd order non-linearity) is more pronounced: narrower the channel bandwidth, larger the REFSENS degradation due to Tx OOB emission at the Rx frequency, and better IP2 performance (receiver linearity) is needed. Duplexing distance/band gap is an important factor for Tx-Rx isolation performance and for determining the second-order response.

Due to a very large duplexing distance for band n254, it is quite safe to say that 2.2dB REFSENS improvement is feasible. Also, for bands n256 and n255, due to a fairly large duplexing distance 2.2dB REFSENS improvement seems to be feasible .

Regarding the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be adopted.

Proposal 3: For the REFSENS requirements, 3 MHz CBW shall be added to Table 7.3.2-1:

- Use 2.2dB improvement compared with 5 MHz CBW as the starting point for all bands;

- For the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be adopted.

Regarding the other Rx requirements, the following was captured in the approved WF [2]:

Issue 2-2-2: Maximum input level

- Way forward

  - FFS on the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm)

In our view, 3 MHz shall be added in table 7.4-1 into the same column as for 5, 10, 15, 20 MHz channel BW, which is the same principle as for TN.

Proposal 4: For the maximum input level, 3 MHz shall be added in table 7.4-1 into the same column as for 5, 10, 15, 20 MHz channel BW.

Issue 2-2-3: ACS

- Way forward

  - Option 1: Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE.

  - Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

As one of the proponents of Option 2 in RAN4#112-bis, we support the same option in this meeting as well.

Proposal 5: For the ACS requirement, 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: For general spurious emissions, reuse the TN UE approach for 3 MHz channel bandwidth, i.e. define OOB boundary FOOB to be 6 MHz.

Proposal 2: For the additional spurious emissions requirements, add 3 MHz CBW into:

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N"

- Add Table 6.5.3.3.5-1 Additional out-of-band requirements for "NS_24"

Proposal 3: For the REFSENS requirements, 3 MHz CBW shall be added to Table 7.3.2-1:

- Use 2.2dB improvement compared with 5 MHz CBW as the starting point for all bands;

- For the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be adopted.

Proposal 4: For the maximum input level, 3 MHz shall be added in table 7.4-1 into the same column as for 5, 10, 15, 20 MHz channel BW.

Proposal 5: For the ACS requirement, 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2416587, “Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi, RAN4#112-bis.