3GPP TSG-RAN WG4 Meeting #113	R4-2419665

# Orlando, Florida, USA, November 18 – 22, 2024

Source:	Nokia

Title:	RRM Performance Requirements for RedCap Positioning

Agenda item:	5.22.2.4

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning includes the enhancements for RedCap positioning. The WID depicts the RAN4 work related to this accuracy improvement technology. The RAN4#112bis meeting [2] has made the agreements on RRM performance requirements for RedCap positioning.

This contribution further investigates impacts to RRM performance requirements for NR positioning enhancements.

Discussion

The impacts to RRM performance requirements for some of the above enhancements are discussed in this section.

Group delay calibration margin for RedCap positioning

Issue 5-1: Group delay margin for accuracy requirements Agreement  A margin to be specified, based on per-hop BW, but the margin is one number for the entire measurement including all hops. Issue 5-2: Frequency drift margin for accuracy requirements Agreement  Reuse frequency drift margin values, Y values in existing specification, for single PFL and multiple PFLs defined for non-RedCap UE to specify impact of frequency drift margin value on RSTD measurement performed by RedCap UE with FH on single PFL and multiple PFLs.

For the group delay calibration margin, it is agreed that the group delay margin will be specified and the margin a value covering the entire measurement of all hops. In this case, the margin for the measurement can be specified by reusing the existing value in the legacy specifications (e.g., Table 10.1.23.2-5 and .-6 of TS38.133) depending on per-hop BW of RedCap UE.

RAN4 to specify the group delay calibration margin as given in the table below.

| Case | Per-hop BW [MHz] | Margin [Tc] |
| --- | --- | --- |
| FR1 | 10 ≤ BW < 20 | 72 |
|  | 20 ≤ BW | 36 |
| FR2 | 20 ≤ BW < 50 | 72 |
|  | 50 ≤ BW < 100 | 32 |
|  | BW = 100 | 16 |

Conclusion

The following proposal is made in this contribution.

- RAN4 to specify the group delay calibration margin as given in the table below.

| Case | Per-hop BW [MHz] | Margin [Tc] |
| --- | --- | --- |
| FR1 | 10 ≤ BW < 20 | 72 |
|  | 20 ≤ BW | 36 |
| FR2 | 20 ≤ BW < 50 | 72 |
|  | 50 ≤ BW < 100 | 32 |
|  | BW = 100 | 16 |

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning,” Intel Corporation, RAN#99.

- R4-2416869, “WF on RRM requirements for NR_pos_enh2_Part1,” Ericsson, RAN4#112bis.