3GPP TSG-RAN WG4 Meeting #113															R4-2419677

Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 		Discussion on the lower frequency threshold for the AAS BS specifications

Agenda Item:	4.3

Document for:	Discussion

# Introduction

WF on the low bands consideration for AAS BS was approved last meeting in R4-2417198, extracted below:

- Consider introduction of the following clarification note for AAS BS lower operating band frequency limit. “NOTE 3: Operating bands below [AAS_low_limit] MHz are not considered as supported by this specification. Operating bands below [AAS_low_limit] MHz are still applicable for protection requirements (i.e. Tx spurious co-location and co-existence, Rx blocking co-location).” - Companies are encouraged to share their preferences on the following aspects for November 2024 meeting: - Preferred [AAS_low_limit] frequency boundary to apply such clarification note. - Preferred release for implementation of such clarification note. - List of expected specifications to be impacted (e.g. AAS-only, or also AAS-based NR BS specifications, etc.). Related work-split to be arranged during the meeting based on the received interest. - It is not intended use the Note in bullet 1 to block any future implementation of AAS BS in low bands (limit can be changed if requested). - The [AAS_low_limit] value may be derived as: min (proposal_1, …, proposal_n)

In this contribution, we provide feedback on the preferred approach for the low bands consideration in AAS BS specifications.

# Discussion

## AAS frequency limit consideration

Discussion on potential introduction of lower frequency limit in AAS BS specifications was trigger by the aim to avoid adding unnecessarily low frequency operating bands to the AAS BS specifications, where certain low bands were seen as meaningless from the AAS BS products point of view due to practical antenna array sizes required.

Current AAS BS specification does not list operating bands explicitly, and refers to the MSR BS specification TS 37.104. While there are many AAS BS products available for the operating bands around 2 GHz, we have filtered out MSR operating bands below ~1.5 GHz:

Table 1: Paired bands in MSR BS specification

| MSR Band number | Supported RATs and Band Numbers |  |  |  |  | Uplink (UL) BS receive, UE transmit (MHz) | Downlink (DL) BS transmit, UE receive (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA | NB-IoT | UTRA | GSM/EDGE |  |  |
| 5 | n5 | 5 | X | V | GSM 850 | 824 – 849 | 869 – 894 |
| 6 | - | - | - | VI | - | 830 – 840 | 875 – 885 |
| 8 | n8 | 8 | X | VIII | E-GSM | 880 – 915 | 925 – 960 |
| 12 | n12 | 12 | X | XII | - | 699 – 716 | 729 – 746 |
| 13 | n13 | 13 | X | XIII | - | 777 – 787 | 746 – 756 |
| 14 | n14 | 14 | X | XIV | - | 788 – 798 | 758 – 768 |
| 17 | - | 17 | X | - | - | 704 – 716 | 734 – 746 |
| 18 | n18 | 18 | X | - | - | 815 – 830 | 860 – 875 |
| 19 | - | 19 | X | XIX | - | 830 – 845 | 875 – 890 |
| 20 | n20 | 20 | X | XX | - | 832 – 862 | 791 – 821 |
| 26 | n26 | 26 | X | XXVI | - | 814 – 849 | 859 – 894 |
| 27 | - | 27 | - | - | - | 807 – 824 | 852 – 869 |
| 28 | n28 | 28 | X | - | - | 703 – 748 | 758 – 803 |
| 29 | n29 | 29 | - | - | - | N/A | 717 – 728 |
| 31 | n31 | 31 | X | - | - | 452.5 – 457.5 | 462.5 – 467.5 |
| 67 | n67 | 67 | - | - | - | N/A | 738 – 758 |
| 68 | - | 68 | - | - | - | 698 – 728 | 753 – 783 |
| 71 | n71 | 71 | X | - | - | 663 – 698 | 617 – 652 |
| 72 | n72 | 72 | X | - | - | 451 – 456 | 461 – 466 |
| 73 | - | 73 | X | - | - | 450 – 455 | 460 – 465 |
| 85 | n85 | 85 | X | - | - | 698 – 716 | 728 – 746 |
| 87 | - | 87 | X | - | - | 410 – 415 | 420 – 425 |
| 88 | - | 88 | X | - | - | 412 – 417 | 422 – 427 |
| 106 | n106 | 106 | X | - | - | 896 – 901 | 935 - 940 |

Table 2: Unpaired bands in MSR BS specification

| MSR Band number | Supported RATs and Band Numbers |  |  |  | Uplink (UL) BS receive, UE transmit (MHz) | Downlink (DL) BS transmit, UE receive (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA | NB-IoT | UTRA |  |  |
| 44 | - | 44 | - | - | 703 – 803 | 703 – 803 |

Similar for NR BS specification TS 38.104 (which covers requirements for both non-AAS and for AAS BS architectures), the following operating bands were already defined below ~1.5 GHz:

Table 3: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- | --- |
| n5 | 824 – 849 | 869 – 894 | FDD |
| n8 | 880 – 915 | 925 – 960 | FDD |
| n12 | 699 – 716 | 729 – 746 | FDD |
| n13 | 777 – 787 | 746 – 756 | FDD |
| n14 | 788 – 798 | 758 – 768 | FDD |
| n18 | 815 – 830 | 860 – 875 | FDD |
| n20 | 832 – 862 | 791 – 821 | FDD |
| n26 | 814 – 849 | 859 – 894 | FDD |
| n28 | 703 – 748 | 758 – 803 | FDD |
| n29 | N/A | 717 – 728 | SDL |
| n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD |
| n67 | N/A | 738 – 758 | SDL |
| n71 | 663 – 698 | 617 – 652 | FDD |
| n72 | 451 – 456 | 461 – 466 | FDD |
| n81 | 880 – 915 | N/A | SUL |
| n82 | 832 – 862 | N/A | SUL |
| n83 | 703 – 748 | N/A | SUL |
| n85 | 698 – 716 | 728 – 746 | FDD |
| n89 | 824 – 849 | N/A | SUL |
| n91 | 832 – 862 | 1427 – 1432 | FDD |
| n92 | 832 – 862 | 1432 – 1517 | FDD |
| n93 | 880 – 915 | 1427 – 1432 | FDD |
| n94 | 880 – 915 | 1432 – 1517 | FDD |
| n100 | 874.4 – 880 | 919.4 – 925 | FDD |
| n105 | 663 – 703 | 612 – 652 | FDD |
| n106 | 896 – 901 | 935 – 940 | FDD |
| n109 | 703 – 733 | 1432 – 1517 | FDD |

Additionally, new Rel-19 low frequency operating bands are currently under development:

| n68 | 698 – 728 | 753 – 783 | FDD |
| --- | --- | --- | --- |
| n87 | 410 - 415 | 420 – 425 | FDD |
| n88 | 412 - 417 | 422 - 427 | FDD |
| n110 | 1390-1395 | 1432-1435 | FDD |

Observation 1: there is a long list of operating bands below 1.5 GHz already defined for AAS BS and for NR BS specifications.

Initial attempt behind the WF in R4-2417198 was to limit or restrict introduction of unnecessarily low bands into the AAS specifications based on the fact that certain low bands are just non-practical from the implementation, or from deployment point of view. There were also complaints, that introduction of low bands into AAS specifications (AAS BS, as well as NR BS, etc.) may be seen as technically not justified, decreasing quality of RAN4 specifications.

At this stage, it shall be also noted, that the topic of low bands in AAS specifications was discussed at early stage of AAS specifications developments around Rel-13 timeframe.

Taking the above into account, this topic was further investigated, with the following findings:

1. Sub-1 GHz Massive MIMO consideration: during recent Global Mobile Broadband Forum 2024, one of the solutions considered for 5.5G was the sub-1 GHz Massive MIMO, which will rely on AAS BS architecture to provide extensive coverage, low-band Massive MIMO improved downlink, uplink, and low-latency experiences for various RATs.

2. Consideration of Massive MIMO for low bands in towards-6G research: one can find that Massive MIMO evolution directions are also addressing low bands, e.g. in concepts like modular massive MIMO [3], where antenna arrays are de-composed to overcome the issue of physical size of typical planar antenna array. Proof of concept was considered in [3] with the carrier frequency ranging from 600 MHz to 3.5 GHz.

Observation 2: Sub-1 GHz operating bands shall be also considered in AAS BS specifications.

With the above observation, identification of suitable lower frequency boundary for AAS BS specifications becomes more tricky. The initially identified candidate of 1 GHz limit does not seem to be valid anymore, and selection of proper sub-1 GHz limit becomes questionable.

Proposal 1: Do not define explicit limit for the lower frequency boundary in AAS BS specifications.

Instead, to address any potential claim of the AAS BS specifications not being suitable for low frequency bands (at least based on existing product designs), the following alternative approach is proposed:

instead of setting explicit hard limit (even if it would be captured as informative note), we suggest to introduce clarification Note saying the following:

“NOTE:	This specification does not define any practical limit for the lower frequency boundary of the AAS BS, as those aspects are implementation specific.”

With the above, the following is proposed:

Proposal 2: Adopt the following Note in impacted specifications (in Operating bands clause):

“NOTE:	This specification does not define any practical limit for the lower frequency boundary of the AAS BS, as those aspects are implementation specific.”

## Impacted specifications

Implementation of Proposal 2 shall be considered for all network-nodes specifications, where AAS architecture is considered. i.e.

AAS BS: TS 37.105

NR BS: TS 38.104

NR Repeater: TS 38.106 (as it covers FR1 and Repeater type 1-H, which is based on AAS architecture)

Related conformance testing specifications do not seem to require such clarifications.

Regarding releases of the above specifications to be impacted: as this topic is clearly related to be legacy releases, it is suggested to keep alignment among AAS and NR specifications, and update them from Rel-15 onwards. NR Repeater specification was triggered in Rel-17, and it is suggested to update it from Rel-17 onwards.

Therefore, the following is proposed:

Proposal 3: The following specifications to be updated with the clarification Note on the lower frequency boundary for AAS architecture: TS 37.105 Rel-15 onwards, TS 38.104 Rel-15 onwards, TS 38.106 Rel-17 onwards.

## Work-split

To be arranged during the meeting, following conclusion on proposals 1, 2, 3 above. Formal CRs to be provided for February 2025 meeting, as a package.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Do not define explicit limit for the lower frequency boundary in AAS BS specifications.

Proposal 2: Adopt the following Note in impacted specifications (in Operating bands clause):

“NOTE:	This specification does not define any practical limit for the lower frequency boundary of the AAS BS, as those aspects are implementation specific.”

Proposal 3: The following specifications to be updated with the clarification Note on the lower frequency boundary for AAS architecture: TS 37.105 Rel-15 onwards, TS 38.104 Rel-15 onwards, TS 38.106 Rel-17 onwards.

# References

[1] 	R4-2417198	Way Forward for [112bis][110] on low bands consideration for AAS BS, RAN4#112-bis

[2]		Huawei Launches 5G-AA Solutions for the Mobile AI Era, MBBR 2024, https://www.huawei.com/en/news/2024/10/mbbf-5gaa-solution-product

[3]		MIMO Evolution toward 6G: Modular Massive MIMO in Low-Frequency Bands, Jeongho Jeon; Gilwon Lee; Ahmad A.I. Ibrahim; Jin Yuan; Gary Xu; Joonyoung Cho, IEEE, https://ieeexplore.ieee.org/document/9665444