# 3GPP TSG-RAN WG4 Meeting # 113                                                         R4-2419695

# Orlando, USA, 18 – 22 November 2024

# Title:	n28 PC2 NS_18

# Agenda Item:	6.20.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents power amplifier (PA) measurement data to check the tentatively agreed upon power class 2 (PC2) A-MPR for Band n28 in Way Forward (WF) [1].

# Discussion

At the last RAN4 meeting, it was agreed in WF [1] that for the A-MPR proposals of  Table 1:

- To further check the A-MPR values for 2Tx

- To further confirm with measurements the yellow highlighted values,

- To confirm the values in [].

Table 1: Tentatively agreed NS_18 PC2 A-MPR [1]

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 8 | [3+0.5] | [8+2] | [3+2] | [0+2.0] |
|  | QPSK | ≤ 3.5 |  | ≤ 8 | [3+1] | [8+2] | [3+2] | [0+3] |
|  | 16 QAM | ≤ 4 |  | ≤ 9 | [3+1] | [8+3] | [3+2.5] | [1.0+3] |
|  | 64 QAM | ≤ 4.5 |  | ≤ 10 | [3+1] | [8+3] | [4.5+1.5] | [2.5+2] |
|  | 256 QAM | ≤ 6.5 |  | ≤ 11 | [3+1] | [8+3] | [5.5+0.5] | [4.5] |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | [4.5+1] | [9.5+2.5] | [5+2] | [1.5+3] |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | [4.5+1] | [9.5+2.5] | [5+2] | [2.0+3] |
|  | 64 QAM | ≤ 6 |  | ≤ 11.5 | [4.5+1] | [9.5+3] | [5.5+1.5] | [3.5+1.5] |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | [4.5+1] | [9.5+3] | [7.5+0.5] | [6.5] |

RAN4 also assumed that the basic rule that in order to preserve the benefits of HPUE operation, the PC2 A-MPR may not exceed the PC3 A-MPR by more than 3dB. In this contribution we bring further PA measurement data. Compared to our previous contribution [2] where a single, fixed Vcc was used, we evaluate the 1Tx performance with variable supply voltage when the measured output back-off (OBO) exceeds 5dB.

  - NS_18 PC2 evaluation results

Only fully allocated DFT-s-OFDM and CP-OFDM QPSK waveforms are evaluated for 25MHz, 30MHz and 40MHz channel bandwidth (CBW), hereby verifying region A4. The PA is initially calibrated 1Tx according to the “usual” PC2 RAN4 linearity assumptions, i.e. the initial supply voltage “Vcc” is adjusted to meet -31dB ACLR at 1dB PC2 OBO using a DFT-s-OFDM QPSK SCS15kHz Lcrb=100(RBstart=0) waveform. The supply voltage is re-evaluated when OBO exceeds 5dB. The post-PA insertion losses are set to 4dB. Table 2 summarizes the measured margins relative to the Region A4 values in [] of Table 1.

Table 2: Measured 1Tx NS_18 margin relative to Table 1 A-MPR in [] for Region A4.

| CBW for full RB allocation | DFT-s QPSK NS_18 1Tx margin | CP-OFDM QSPK NS_18 1Tx margin |
| --- | --- | --- |
| 25MHz | 0.5 | 0 |
| 30MHz | 5.5 | 0 |
| 40MHz | 0 | 1.0 |

Observation 1: The measured 1Tx performance is marginal for most fully allocated waveforms. Small additional degradation is expected due to 2Tx reverse-IMD.

  - NS_18 PC2 A-MPR proposals

For CBW less than 25MHz, we observe that the regions A1 and A2 currently specified for PC3 and proposed for PC2 are not clearly defined and that the applicable A-MPR for outer allocation is ambiguous. It is not clear for outer allocations which A-MPR applies: A1 or A2?

Observation 2: There are no equations defining the regions A1 and A2 regions for either PC3 or PC2. It is not clear which A-MPR requirement applies to outer allocations since both regions apply to “Outer”.

Proposal 1: Based on observation 2, clarify the applicable A-MPR for outer allocations for CBW< 25MHz.

Based on observation 1, and to account for 2Tx additional reverse-IMD contribution, we propose to increase the Region 4 A-MPR as highlighted in yellow in proposal 2, Table 3.

Proposal 2: Based on observation 1, adopt the PC2 NS_18 A-MPR of Table 3.

Table 3: Proposed NS_18 PC2 A-MPR

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 8 | [3+0.5] | [8+2] | [3+2] | [0+2.0] |
|  | QPSK | ≤ 3.5 |  | ≤ 8 | [3+1] | [8+3] | [3+2] | [0+3] |
|  | 16 QAM | ≤ 4 |  | ≤ 9 | [3+1] | [8+3] | [3+2.5] | [1.0+3] |
|  | 64 QAM | ≤ 4.5 |  | ≤ 10 | [3+1] | [8+3] | [4.5+1.5] | [2.5+2] |
|  | 256 QAM | ≤ 6.5 |  | ≤ 11 | [3+1] | [8+3] | [5.5+0.5] | [4.5] |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | [4.5+1] | [9.5+3] | [5+2] | [1.5+3] |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | [4.5+1] | [9.5+3] | [5+2] | [2.0+3] |
|  | 64 QAM | ≤ 6 |  | ≤ 11.5 | [4.5+1] | [9.5+3] | [5.5+1.5] | [3.5+1.5] |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | [4.5+1] | [9.5+3] | [7.5+0.5] | [6.5] |

# Conclusion

This paper captures our views on the Band n28 PC2 A-MPR for NS_18 and channel bandwidth up to 40MHz.

Observation 1: The measured 1Tx performance is marginal for most fully allocated waveforms. Small additional degradation is expected due to 2Tx reverse-IMD.

Observation 2: There are no equations defining the regions A1 and A2 regions for either PC3 or PC2. It is not clear which A-MPR requirement applies to outer allocations since both regions apply to “Outer”.

Proposal 1: Based on observation 2, clarify the applicable A-MPR for outer allocations for CBW< 25MHz.

Proposal 2: Based on observation 1, adopt the PC2 NS_18 A-MPR of Table 3.

Table 3: Proposed NS_18 PC2 A-MPR

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 8 | [3+0.5] | [8+2] | [3+2] | [0+2.0] |
|  | QPSK | ≤ 3.5 |  | ≤ 8 | [3+1] | [8+3] | [3+2] | [0+3] |
|  | 16 QAM | ≤ 4 |  | ≤ 9 | [3+1] | [8+3] | [3+2.5] | [1.0+3] |
|  | 64 QAM | ≤ 4.5 |  | ≤ 10 | [3+1] | [8+3] | [4.5+1.5] | [2.5+2] |
|  | 256 QAM | ≤ 6.5 |  | ≤ 11 | [3+1] | [8+3] | [5.5+0.5] | [4.5] |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | [4.5+1] | [9.5+3] | [5+2] | [1.5+3] |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | [4.5+1] | [9.5+3] | [5+2] | [2.0+3] |
|  | 64 QAM | ≤ 6 |  | ≤ 11.5 | [4.5+1] | [9.5+3] | [5.5+1.5] | [3.5+1.5] |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | [4.5+1] | [9.5+3] | [7.5+0.5] | [6.5] |

# References

- R4-2417082 WF on PC2 and 40MHz CBW for n28, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, CMCC.

- R4-2416227 Requirements for 40MHz CBW in Band n28, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks Solutions, Inc.