# 3GPP TSG-RAN WG4 Meeting # 113                                                         R4-2419697

# Orlando, USA, 18 – 22 November 2024

# Title:	CA_n40-n41 Simultaneous RxTx

# Agenda Item:	6.11.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on the challenges for CA_n40-n41 simultaneous RxTx. This contribution serves as a follow up to the latest agreed upon Way Forward (WF) [1] and our previous contribution [2].

# Discussion

The agreed WF [1] invites companies to evaluate if a new MSD test point for CA_n40-n41 is needed, and to consider whether a new signalling is necessary to support 2-layer MIMO with 2Rx antenna ports for Band n41. In this contribution, we focus our proposals on the discussion related to 2-layer MIMO with 2Rx antenna ports. Our views on MSD were previously captured in [2] and we believe there is no need to introduce an additional test point for CA_n40-n41.

Regarding CA_n40-n41 with simultaneous RxTx operation and based on observations 2 and 3 from [2], our views are:

-	An UE that supports 4Rx antenna for single Band n41 operation should be allowed to operate in Band n41 when configured for CA_n40-n41 with 2 Rx antenna ports; and

-	An UE that supports 6Rx antenna for single Band n41 operation should be allowed to operate in Band n41 when configured for CA_n40-n41 with 4 Rx antenna ports.

Regarding the impact of such waiver onto RAN 2 signalling:

RAN4 previously allowed special types of UEs to waive the mandatory 4Rx requirement: vehicle UEs were first, recently followed by XR devices. We are unsure if signalling is already in place to support such “exception” UEs in commercial networks. We believe that RAN2 should be informed about these restrictions and that RAN4 should ask if any impact on existing signalling is expected for such waiver.

Proposal: For CA_n40-n41 simultaneous RxTx operation, RAN4 to allow the following waivers:

- An UE that supports 4Rx antenna ports for single Band n41 operation should be allowed to operate in Band n41 with 2 Rx antenna ports when configured for CA_n40-n41simultaneous RxTx; and

- An UE that supports 6Rx antenna ports for single Band n41 operation should be allowed to operate in Band n41 with 4 Rx antenna ports when configured for CA_n40-n41simultaneous RxTx.It is recommended to inform RAN2 about these restrictions and to ask whether any impact on existing signalling is expected.

# Conclusion

This paper captures our views on supporting CA_n40-n41 with simultaneous RxTx.  Regarding the CA_n40-n41 MSD, based on previous contribution [2] we do not see the need to introduce a new test point and make the following proposal.

Proposal: For CA_n40-n41 simultaneous RxTx operation, RAN4 to allow the following waivers:

- An UE that supports 4Rx antenna ports for single Band n41 operation should be allowed to operate in Band n41 with 2 Rx antenna ports when configured for CA_n40-n41simultaneous RxTx;

- An UE that supports 6Rx antenna ports for single Band n41 operation should be allowed to operate in band n41 with 4 Rx antenna ports when configured for CA_n40-n41simultaneous RxTx,

It is recommended to inform RAN2 about these restrictions and to ask whether any impact on existing signalling is expected.

# References

- R4-2417058 WF on simultaneous Rx-Tx for CA_n40A-n41A, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Huawei.

- R4-2416262 CA_n40-n41 Simultaneous RxTx, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks Solutions, Inc.