3GPP TSG-RAN WG4 Meeting #113 															R4-2419700

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	5.22.1

Source: 	Ericsson

Title:	On remaining core issues for NR positioning

Document for:	Discussion

# Introduction

The latest agreed WF on SL positioning and CPP is in [21].

In this contribution, we discuss issues related to SL positioning core part.

# Discussion on SL positioning core issues

## On SL RSTD measurement period

In the previous meetings, there was a discussion on whether the SL RSTD measurement period needs further update.

The related agreements are:

RAN4#112-bis [21]:

Agreement:

- Reflect in TS 38.133 the SL PRS measurement of the reference UE link in the SL RSTD core requirement.

RAN4#112 [20]:

Conclusion:

    - No common understanding on RAN2 procedure for reporting behaviour. Companies check offline.

    - No consensus at the moment to revisit previous agreement

It is our understanding that the current SL RSTD measurement period definition is correct, but a clarification on that SL-PRS from the reference UE is available during the RSTD measurement period TSL RSTD,total may be added, if this helps to interpret the requirement correctly. Firstly, the reference link has a better side condition, according to the requirements, and therefore does not require 4 samples. Secondly, the reference link typically is not measured multiple times for multiple anchor links. A CR with an example clarification is provided in R4-2419701.

- Proposal 1 (SL positioning): A clarification may be in the SL RSTD measurement period that the requirement applies, provided that the SL-PRS from the reference UE is available within TSL RSTD,total.

# Summary

The following have been proposed in the current contribution.

- Proposal 1 (SL positioning): A clarification may be in the SL RSTD measurement period that the requirement applies, provided that the SL-PRS from the reference UE is available within TSL RSTD,total.

# References

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corp, March, 2023.

[2] R4-2303190, WF on Expanded and Improved NR Positioning RRM requirements, Ericsson, March, 2023.

[3] R1-2301950, Summary #1 on Measurements and reporting for SL positioning, vivo, March 2023.

[4] R1-2301951, Summary #2 on Measurements and reporting for SL positioning, vivo, March 2023.

[5] R1-2301952	Summary #3 on Measurements and reporting for SL positioning, vivo, March 2023.

[6] R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, April 2023.

[7] R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT, May 2023.

[8] R4-2310077, Simulation assumptions for SL positioning, CATT, May 2023.

[9] R4-2314343, Ad-hoc minutes on R18 NR Positioning RRM requirements, Ericsson, Aug. 2023.

[10] R4-2314354, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Aug. 2023.

[11] R4-2314358, LS on SL positioning and CPP measurements report mapping, Aug. 2023.

[12] R4-2312724, On SL positioning, Ericsson, Aug. 2023.

[13] RP-232670, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, MediaTek, Sep. 2023.

[14] R4-2317380, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Oct. 2023.

[15] R4-2321524, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Nov. 2023.

[16] R4-2403368, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, March 2024.

[17] R4-2406384, WF on SL positioning and carrier phase positioning, CATT, Apr. 2024.

[18] R4-2410193, WF on SL positioning and carrier phase positioning, CATT, May 2024.

[19] R4-2411003, Reply LS on SL positioning measurement, RAN1, Aug. 2024.

[20] R4-2414033, WF on SL positioning and carrier phase positioning, Ericsson, Oct. 2024.

[21] R4-2416870, WF on RRM requirements for NR_pos_enh2_Part2, CATT, Oct. 2024.