3GPP TSG-RAN4 Meeting #113	   											  R4-2419716

# Orlando, USA, 18th of November 2024 – 22nd of November 2024

Title: 	Discussion on RF requirements for NR-NTN S-band

Release: 	Rel-19

Agenda Item:	6.17.2 - System parameters and UE RF requirements

Source: 	THALES, Magister Solutions, ICS (Institute for Communication Systems - University of Surrey)

Document for:	Discussion

# For information

This document presents some critical issues with respect to the discussion of RF requirements for new NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) and RF core requirements resulted from the coexistence with/without 1.5 km between NTN UE and TN network.

As a matter of fact, if isolation distance is removed for coexistence scenario, the simulations show significant degradation in NTN DL performance in the coverage of area of the TN cluster. This aligns with prior assessment that NTN is complimentary to TN and likely to be used only in geographic areas when there is no TN coverage. Therefore, the probability of NTN UE UL causing interference to TN DL performance is extremely low in the coverage area of the TN cluster.

# Recall coexistence scenarios from Rel-17

The coexistence scenarios from Rel-17 could be found in the Table and Figures below (see also TR 38.863).

Table 1: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | NTN with NTN | NTN DL | NTN DL | HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | HAPS-HAPS | Phase 2 |

For further visualisation, the interference charts with respect to scenarios 1 to 6 are further represented with respect to NTN-TN coexistence scenarios in Figure 1 (scenarios 1 to 4) and Figure 2 (scenarios 5 and 6).

Figure 1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

Figure 2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

# Discussion with respect to simulation results with and without isolation distance for Scenario 1

The coexistence scenarios from Rel-17 with and without isolation distance could be found in the Figures and Tables below. As a matter of fact, 3 alternatives have been considered for the isolation distance from Scenario 1:

1/ Alternative 1: 2xISD NTN UE isolation distance, which was the default FR1 Rel-17 coexistence assumption (1500 m from NTN UE ring to TN cluster edge);

2/ Alternative 2: No NTN UE isolation distance (0 m from NTN ring to TN cluster edge);

3/ Alternative 3: NTN UEs in TN cluster (“InCluster”), with 250 m or 30 m isolation distance between NTN UEs and gNBs.

Figure 3: Examples of Alternative 1, 2 and 3

The simulations for GEO 90° elevation angle with urban scenario case indicate à 38% throughput loss for NTN DL, corresponding to 32.9dB ACIR (as a result of TN BS ACLR of 50dB and NTN UE ACS of 33dB). This would be a typical example of TN DL interference (e.g. n66) with NTN UE (e.g. n252/n256).

Figure 4: Scenario 1 – GEO, Average Throughput Loss Results

The blue curve (corresponding to Scenario 1 – isolation distance Alternative 1) is with the default 1500 m isolation distance between NTN UEs and TN cluster edge. ~17 dB ACIR is required for 5% TP loss.

The red curve (corresponding to Scenario 1 – isolation distance Alternative 2) is without isolation distance where the NTN users are at the outer edge of the TN cluster. ~35 dB ACIR is required for 5% TP loss.

In orange and green cases (corresponding to Scenario 1 – isolation distance Alternative 3) the NTN UEs are within the TN cluster.

- In the "InCluster" case the minimum distance between NTN UE and gNB is 250 m.

- In the "InCluster30" case the minimum distance between NTN UE and gNB is 30 m.

Observation1: In-Cluster deployment is not going to work from NTN DL perspective, the required ACIR is clearly above 40 dB for a throughput loss lower than 5%. LEO600 is slightly better, but the required ACIR for 5% throughput loss is still ~ 40 dB.

The simulations for LEO@600km with urban scenario case indicate à 20% throughput loss for NTN DL, corresponding to 32.9dB ACIR (as a result of TN BS ACLR of 50dB and NTN UE ACS of 33dB). This would be a typical example of TN DL interference (e.g. n66) with NTN UE (e.g. n252/n256).

Figure 5: Scenario 1 – LEO@600km, Average Throughput Loss Results

Observation 2: In-Cluster/No-Isolation deployment is not going to work from NTN DL perspective, the required ACIR is clearly above 40 dB.

Simulations with 5%-tile throughput loss indicate even worst results, as shown below.

Figure 6: Scenario 1 – LEO@600km and GEO, 5%-tile Throughput Loss Results

Observation 3: In-Cluster deployment for LEO@600km with TN BS - NTN UE isolation distance of 30 m indicates a 5%-tile throughput loss of more than 90% for the NTN UE.

Observation 4: In-Cluster deployment for LEO@600km with TN BS - NTN UE isolation distance of 250 m indicates a 5%-tile throughput loss of more than 60% for the NTN UE.

Observation 5: In-Cluster deployment for GEO with TN BS - NTN UE isolation distance of 30 m indicates a 5%-tile throughput loss of 100% for the NTN UE.

Observation 6: In-Cluster deployment for GEO with TN BS - NTN UE isolation distance of 250 m indicates a 5%-tile throughput loss of more than 80% for the NTN UE.

Table 1 - Numerical results for LEO@600km with different isolation distances

| LEO600 - Default_Urban |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR [dB] |  | 0 | 10 | 20 | 30 | 40 | 50 |
| NTN DL Throughput loss [%] | Average | 15,73 | 3,63 | 0,7 | 0,09 | 0,01 | 0 |
|  | 5th percentile | 56,43 | 14,89 | 1,65 | 0,55 | 0 | 0 |
| NTN DL SINR attenuation [dB] | Average | 2,02 | 0,44 | 0,08 | 0,01 | 0 | 0 |
|  | 5th percentile | 6,02 | 1,38 | 0,15 | 0,05 | 0 | 0 |
| LEO600 - InCluster30_Urban |  |  |  |  |  |  |  |
| ACIR [dB] |  | 0 | 10 | 20 | 30 | 40 | 50 |
| NTN DL Throughput loss [%] | Average | 95,5 | 79,27 | 50,12 | 23,89 | 10,31 | 4,03 |
|  | 5th percentile | 100 | 100 | 100 | 100 | 78,9 | 25,87 |
| NTN DL SINR attenuation [dB] | Average | 26,41 | 16,92 | 9,01 | 4 | 1,6 | 0,56 |
|  | 5th percentile | 49,53 | 39,53 | 29,52 | 19,55 | 10,03 | 2,45 |
| LEO600 - InCluster_Urban |  |  |  |  |  |  |  |
| ACIR [dB] |  | 0 | 10 | 20 | 30 | 40 | 50 |
| NTN DL Throughput loss [%] | Average | 94,4 | 74,05 | 40,42 | 15,08 | 4,78 | 1,27 |
|  | 5th percentile | 100 | 100 | 100 | 75,69 | 28,15 | 5,67 |
| NTN DL SINR attenuation [dB] | Average | 22,95 | 13,58 | 6,19 | 2,14 | 0,63 | 0,15 |
|  | 5th percentile | 38,98 | 28,98 | 19 | 9,32 | 2,7 | 0,5 |
| LEO600 - NoIsolation_Urban |  |  |  |  |  |  |  |
| ACIR [dB] |  | 0 | 10 | 20 | 30 | 40 | 50 |
| NTN DL Throughput loss [%] | Average | 74,88 | 39,14 | 13,29 | 3,58 | 0,75 | 0,12 |
|  | 5th percentile | 100 | 100 | 62,94 | 16,33 | 1,47 | 0,73 |
| NTN DL SINR attenuation [dB] | Average | 13,16 | 5,68 | 1,76 | 0,44 | 0,09 | 0,01 |
|  | 5th percentile | 26,32 | 16,38 | 6,98 | 1,53 | 0,12 | 0,08 |

Table 2 - Numerical results for GEO with different isolation distances

| GEO - Default_Urban |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR [dB] |  | 0 | 10 | 20 | 30 | 40 | 50 |
| NTN DL Throughput loss [%] | Average | 32,96 | 8,68 | 1,88 | 0,32 | 0,04 | 0 |
|  | 5th percentile | 100 | 31,76 | 4,05 | 0 | 0 | 0 |
| NTN DL SINR attenuation [dB] | Average | 3,55 | 0,85 | 0,17 | 0,03 | 0 | 0 |
|  | 5th percentile | 7,8 | 2 | 0,25 | 0 | 0 | 0 |
| GEO - InCluster30_Urban |  |  |  |  |  |  |  |
| ACIR [dB] |  | 0 | 10 | 20 | 30 | 40 | 50 |
| NTN DL Throughput loss [%] | Average | 99,42 | 93,32 | 71,68 | 40,76 | 19,72 | 9,28 |
|  | 5th percentile | 100 | 100 | 100 | 100 | 100 | 68,24 |
| NTN DL SINR attenuation [dB] | Average | 31,79 | 21,97 | 13,03 | 6,49 | 2,92 | 1,19 |
|  | 5th percentile | 54,38 | 44,38 | 34,38 | 24,4 | 14,7 | 5,65 |
| GEO - InCluster_Urban |  |  |  |  |  |  |  |
| ACIR [dB] |  | 0 | 10 | 20 | 30 | 40 | 50 |
| NTN DL Throughput loss [%] | Average | 99,24 | 91,53 | 64,88 | 30,1 | 10,69 | 3,25 |
|  | 5th percentile | 100 | 100 | 100 | 100 | 53,79 | 12,41 |
| NTN DL SINR attenuation [dB] | Average | 27,87 | 18,11 | 9,45 | 3,71 | 1,19 | 0,32 |
|  | 5th percentile | 42,3 | 32,3 | 22,38 | 12,65 | 3,88 | 0,7 |
| GEO - NoIsolation_Urban |  |  |  |  |  |  |  |
| ACIR [dB] |  | 0 | 10 | 20 | 30 | 40 | 50 |
| NTN DL Throughput loss [%] | Average | 92,75 | 64,25 | 26,83 | 7,77 | 2,05 | 0,39 |
|  | 5th percentile | 100 | 100 | 77,24 | 28,97 | 6,9 | 0,69 |
| NTN DL SINR attenuation [dB] | Average | 17,45 | 8,69 | 3,03 | 0,82 | 0,2 | 0,03 |
|  | 5th percentile | 26,32 | 16,43 | 7,2 | 1,8 | 0,38 | 0,05 |

Observation 7: Isolation distance lower than 1.5 km introduces an average throughput loss higher than 30-40% in the case of Scenario1.

Observation 8: Isolation distance lower than 1.5 km introduces a 5%-tile throughput loss higher than 60-90% in the case of Scenario1.

Observation 9: Given the wide coverage area of a spot beam, it is more judicious to leverage the TN network for smartphone connectivity in geographic areas where coverage from TN is present, and utilize the limited NTN DL capacity for areas that do not TN coverage within the spot beam of the satellite or when the TN is unavailable (e.g. during emergencies, disaster recover, etc.).

Observation 10: Based on probabilistic assessments provided in the previous meetings and considering the above observations, the list of coexistence impact to the TN DL from an adjacent NTN UL operation is negligible.

Observation 11: Relaxing the isolation distance to justify -50 dBm/MHz UE coexistence emission limit for protection of TN DL from NTN UL (Scenarios 4 and 5) will have implications on TN operation when it will be required to protect existing NTN spectrum bands in the coverage area of the TN (Scenario 1).

Proposal 1: Given that 3GPP is a standard body and not a regulatory body, RAN4 shall not place excessive burden that limit NTN spectrum use through coexistence emission limits, especially when the risk of coexistence issue is extremely low, and allow satellite operators to fully utilize the entire authorized spectrum.

Proposal 2: 3GPP should consider some reasonable isolation distance between TN and NTN UEs for in-cluster operation, otherwise NTN performance is severely deteriorated.

# Conclusions

Observation1: In-Cluster deployment is not going to work from NTN DL perspective, the required ACIR is clearly above 40 dB for a throughput loss lower than 5%. LEO600 is slightly better, but the required ACIR for 5% throughput loss is still ~ 40 dB.

Observation 2: In-Cluster/No-Isolation deployment is not going to work from NTN DL perspective, the required ACIR is clearly above 40 dB.

Observation 3: In-Cluster deployment for LEO@600km with TN BS - NTN UE isolation distance of 30 m indicates a 5%-tile throughput loss of more than 90% for the NTN UE.

Observation 4: In-Cluster deployment for LEO@600km with TN BS - NTN UE isolation distance of 250 m indicates a 5%-tile throughput loss of more than 60% for the NTN UE.

Observation 5: In-Cluster deployment for GEO with TN BS - NTN UE isolation distance of 30 m indicates a 5%-tile throughput loss of 100% for the NTN UE.

Observation 6: In-Cluster deployment for GEO with TN BS - NTN UE isolation distance of 250 m indicates a 5%-tile throughput loss of more than 80% for the NTN UE.

Observation 7: Isolation distance lower than 1.5 km introduces an average throughput loss higher than 30-40% in the case of Scenario1.

Observation 8: Isolation distance lower than 1.5 km introduces a 5%-tile throughput loss higher than 60-90% in the case of Scenario1.

Observation 9: Given the wide coverage area of a spot beam, it is more judicious to leverage the TN network for smartphone connectivity in geographic areas where coverage from TN is present, and utilize the limited NTN DL capacity for areas that do not TN coverage within the spot beam of the satellite or when the TN is unavailable (e.g. during emergencies, disaster recover, etc.).

Observation 10: Based on probabilistic assessments provided in the previous meetings and considering the above observations, the list of coexistence impact to the TN DL from an adjacent NTN UL operation is negligible.

Observation 11: Relaxing the isolation distance to justify -50 dBm/MHz UE coexistence emission limit for protection of TN DL from NTN UL (Scenarios 4 and 5) will have implications on TN operation when it will be required to protect existing NTN spectrum bands in the coverage area of the TN (Scenario 1).

Proposal 1: Given that 3GPP is a standard body and not a regulatory body, RAN4 shall not place excessive burden that limit NTN spectrum use through coexistence emission limits, especially when the risk of coexistence issue is extremely low, and allow satellite operators to fully utilize the entire authorized spectrum.

Proposal 2: 3GPP should consider some reasonable isolation distance between TN and NTN UEs for in-cluster operation, otherwise NTN performance is severely deteriorated.

# Annex - Simulation results presented in R4-2413352

Under geographical separation assumption of 1.5km between NTN UE and Terrestrial Networks (TN), NTN-TN coexistence simulations have been performed even for HPUE with 31dBm transmission power corresponding to Power Class 1 (which is the worst case compared to 26dBm and 29dBm transmission power corresponding to Power Class 2 and Power Class 1.5). The results do not indicate any degradation, so the current TN UE requirements can be reused for NTN UE.

The basic simulation assumptions show:

- UE Tx Power: 23, 26, 29, 31 dBm

- Frequency band : S-band with 20 MHz DL and UL bands for both networks

- TN network of 7 sites with 3 cells per site

- 10 TN UEs deployed per cell in the cell range

- 50 NTN UEs deployed on the edge of TN; isolation distance of 2*ISD in Urban scenarios

- NTN LEO@600km/LEO@1200km/GEO satellite on top of TN

- Full buffer traffic model

- Scenarios: Urban or Rural

Simulations have been further performed only based on Scenario 4 and 5 with the following conclusions:

1/ Scenario 4 – 23dBm vs 31dBm UE Tx power

LEO@600km

LEO@1200km

GEO

For Scenario 4, throughput loss increases from 0.035% to 0.20% with 12 dB ACIR from 23 dBm to 31 dBm UE.

For Scenario 4, throughput loss is 0 at 30 dB ACIR for 31 dBm UE.

Increased UE Transmission Power (TxP) has no co-existence impact in Scenario 4 with studied system parameters as per WID description.

2/ Scenario 5 – 23dBm vs 31dBm UE Tx power

LEO@600km

LEO@1200km

GEO

For Scenario 5, throughput loss increases from 0.07% to 0.14% with 12 dB ACIR from 23 dBm to 31 dBm UE.

For Scenario 5, throughput loss is 0.02% at 30 dB ACIR for 31 dBm UE.

Increased UE Transmission Power (TxP) has no co-existence impact in Scenario 5 with studied system parameters as per WID description.

From adjacent channel interference impact point of view for HPUE, under WID assumption, there are no problems identified even with 31dBm NTN HPUE (31dBm NTN UL Transmission Power).

Rel-19 NTN HPUE to reuse Rel-17 NTN UE ACLR for all power classes up to 31dBm (PC1).

Since no impact on Rx side with respect to Rel-17 according to coexistence analysis, Rel-19 NTN HPUE to reuse Rel-17 NTN UE ACS.