3GPP TSG-RAN WG4 Meeting #113                                            R4-2419724

# Orlando, USA, 18th – 22nd of November 2024

Title: 	Updates on the VSAT and Satellite Parameters for Ku-band Coexistence

Agenda Item:	7.9.2 - Coexistence study based on ITU regulations

Release:	Rel-19

Source: 	THALES

Document for:	Discussion

# For information

As described in the approved Way Forward for [112][314] NR_NTN_Ku_Band_General (R4-2413519, Eutelsat Group), the following agreements have been made at the previous RAN4#112 meeting:

#### Issue 1-1: Work plan

#### The work plan in R4-2411506 is approved

#### Issue 1-2: Band definitions

Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S).

Additional band definitions are expected once the applicable regulations have been identified and studied.

#### Issue 2-2: Coexistence scenarios

Adopt the interference scenarios in [Intelsat] R4-2411507 for the coexistence studies

#### Issue 2-4: Coexistence frequencies

#### Use 11GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band.

#### Issue 2-8: Coexistence TN channel model

For TN channel model, reuse existing version of TR 38.901.

#### Issue 2-9: Coexistence TN channel model

For NTN channel model, reuse parameters from TR 38.811 and collect the following missing information:

- Shadow fading and clutter loss tables for the reference frequencies

- Tropospheric scintillation.

#### Issue 2-10: TN Parameters

As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from:

- The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz

- The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

Then, the following decisions have been taken at RAN4#112-bis (see Way Forward for [112bis][313] NR_NTN_Ku_Band_General, R4-2416593, Eutelsat Group), but some decisions are still missing.

#### Sub-topic 2-1: SAN equivalent antenna aperture

Use 9.1m DL, 7.14m UL (GEO ), 0.91m DL, 0.714m UL (LEO DL 600 & 1200)

#### Issue 2-8: Coexistence TN channel model

For TN channel model, reuse existing version of TR 38.901.

#### Sub-topic 2-2: SAN antenna pattern

Use section 6.4.1 from 38.811

#### Sub-topic 2-3: SAN Tx antenna maximum gain

Use 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200)

#### Sub-topic 2-4: SAN Rx antenna maximum gain

Use 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200)

#### Sub-topic 2-5: SAN antenna beamwidth

Use 0.177 (GEO), 1.768 (LEO 600 & 1200)

#### Sub-topic 2-7: SAN max EIRP

43 dBw/MHz (GEO), 13 dBw/MHz (LEO 1200), 7 dBW/MHz (LEO 600) using 100 MHz bandwidth

#### Sub-topic 2-9: VSAT equivalent antenna aperture

Use 60 cm

#### Sub-topic 2-10: VSAT polarization

Assume circular first and if necessary derive the linear result

#### Sub-topic 2-11: VSAT max Tx antenna gain

Use 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W)

#### Sub-topic 2-12: VSAT max Rx antenna gain

34.3 dBi

#### Sub-topic 2-15: VSAT noise figure

Type 1 2.5 dB for VSAT LEO and GEO

Type 2 6 dB for VSAT to LEO

Other options are not precluded

#### Sub-topic 2-16: VSAT max Tx power

40 dBm (10 W)

#### Sub-topic 2-21: Channel model

38.811 is the channel model  for satellite used for Ka. It can be scaled for Ku.

For TN use 38.863 and change the frequency to 11 & 14 GHz

#### Sub-topic 2-22: TN parameters

NTN to NTN won’t be considered.

Continue with the TN - NTN coexistence study noting that the applicability of the VSAT ACS simulation result may have to be relaxed for the core requirement derivation.

Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

# Proposals from R4-2416503

For the time being, the following NTN Ku-band has been agreed in WF R4-2413519: 10.7 - 12.75 GHz (DL); 14.0 - 14.5 GHz (UL). Moreover, it seems that SAN/VSAT UE parameters for Ku-band discussed at previous meeting were incomplete (see e.g. R4-2411507, Intelsat; R4-2320973, THALES).

Based on current Ku-band UL/DL assumption, with the use of 11 GHz for DL coexistence evaluation and 14 GHz for UL coexistence evaluation, in this contribution we propose the Ku-band VSAT and Satellite parameters to be considered for the coexistence simulation/link budget evaluation as follows:

1/ RAN4 to use following Satellite parameters for Ku-band work:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 9.1 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.177 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 0.714 m | 0.714 m |
| G/T |  | 31.4 dB K-1 | 11.4 dB K-1 | 11.4 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

2/ RAN4 to use following VSAT UE parameters for Ku-band work:

- Option 1/2

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB -> updated to 2.5 and 6 dB as per RAN4#112-bis meeting |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 36.6 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

- Option 2/2 (for EIRPmax=76.2dBm equivalent to Ka-band) – with increase of the transmission power up to 10W instead of 2W.

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB -> updated to 2.5 and 6 dB as per RAN4#112-bis meeting |
| Tx transmit power | 10 W (40 dBm) |
| Tx antenna gain | 36.2 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

# Updated proposals to take into account RAN4#112-bis discussions

Proposal 1: Keep Bessel function for the VSAT UE antenna model (see TR 38.863) as basic option/model for coexistence scenarios.

Proposal 2: Other UE VSAT antenna models can be driven by (calibration and/or coexistence) contributions, but shall not be considered as mandatory.

Proposal 3: Consider G/T for GEO of 31.4 dB K-1 and G/T for LEO of 11.4 dB K-1.

# Conclusion

Proposal 1: Keep Bessel function for the VSAT UE antenna model (see TR 38.863) as basic option/model for coexistence scenarios.

Proposal 2: Other UE VSAT antenna models can be driven by (calibration and/or coexistence) contributions, but shall not be considered as mandatory.

Proposal 3: Consider G/T for GEO of 31.4 dB K-1 and G/T for LEO of 11.4 dB K-1.

# Annex

Please recall the Ka-band values (they are also in the TR 38.821 from the study phase):

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 6.1.1.1-3: UE characteristics for system level simulations

| Characteristics | VSAT (Note 2) | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | TBD dBi per element |
| Antenna temperature | 150 K | 290 K | TBD K |
| Noise figure | 1.2 dB | 7 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | TBD dBi per element |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |