3GPP TSG-RAN WG4 Meeting #113	R4-2419730

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Discussion on Fast SCell activation with EMR

Agenda item:	7.15.3

Document for:	Discussion

Introduction

RAN4 Rel-19 work item on fast SCell activation for UE supporting Rel-18 EMR officially trigered in RAN4#112 and continued in RAN4#112bis. During RAN2#112bis meeting, there was a good process on some of the issues, while others still remains.

As per the outcomes of this meeting ‎[1], still following  points are open further discussion:

- Feasibility of fast SCell activation requirements,

- Target scenarios of fast SCell activation,

- How to define the fast SCell activation delay requirements with valid eEMR reporting, and how to update the known condition with consideration of valid eEMR reporting,

- Whether network indication is needed or not

In this paper, we address the above open points and provide our views on reduce the SCell activation delay with valid Rel-18 EMR reporting.

Discussion

Work schedule and closing of the WI

4.1 Objective of SI or Core part WI or Testing part WI Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

Currently there are still many open issues in this topic, and the main open issue that currently blocks the completion of this work item is the issue of whether to define requirements for the scenario where the validityDuration-r18 is configured. This, relatively non-technical, issue was extensively discussed in the rel-18 and there was a compromise to include validityDuration-r18 with configurability. If the “not configured” scenario is not included, it may not be feasible to define requirements for either of the cases. Therefore, it would make the feasibility of the whole feature questionable. The feature in this case would not bring any improvements over Rel-18 baseline.

Work item cannot be closed with specification impact without including the case where  valdityDuration is not configured.

RAN4 to conclude that it is not feasible to define requirements only for the case where validityDuration is configured.

The rest of the contribution assumes that the case is included, and we can proceed with the technical work.

Applicability of fast SCell activation delay requirements

Regarding applicability of fast SCell activation, following agreement is made during the previous discussion

Issue 1-1-1: Applicability of fast SCell activation delay requirements Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

This issue has been extensively discussed, and our proposal is still the same:

RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

Known condition

In this work item, we can define a new known condition when the EMR has been reported. The current condition does accommodate the EMR reporting, or it does not consider the idle-mode detection.

Define new known and unknown condition for Scell with EMR reporting

- SCell in FR1 or FR2 is known if it has been meeting the following conditions:

    - The UE has sent a measurement report for the SCell being activated according to reporting requirements in 4.7  or 5.8, and

    - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state, and

    - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

- Otherwise SCell in FR1 or FR2 is unknown.

Additional conditions when UE is not configured with ValidityDuration-r18

Some companies have raised concerns on whether the UE should report the measurement results when the validityDuration-r18 is not configured. We would like to still highlight that a time condition, such as validityDuration-r18 does not have any relation to how good radio conditions UE has, or is observing. For instance, when UE is bolted to a wall, it makes very little difference whether the measurements were made 5s, or 101s ago.

To address companies concerns on validityDuration-r18, we are ok to consider additional conditions to safeguard the UEs when the validityDuration-r18 is not configured:

- SSB remains detectable from idle/inactive mode to the connected at least until Scell activation

- UE is either in low-mobility or UE is stationary

- UE is not at the cell edge

The above should be enough to make sure that the measurements are usable regardless of the configuration of Validityduration-r18.

ValidityDuration-r18 has still no relevance to UE radio conditions, and it is not feasible to have this as the only solution in rel18.

When the validityDuration is not configured, if needed, to make the cell known, consider conditions that relate to the radio conditions:  mobility condition(s) and cell edge condition(s).

Target scenarios of fast SCell activation

#### Issue 1-1-3: Target scenarios of fast SCell activation Proposals - Proposal 1: (CMCC, CTC) - it is proposed to reduce SCell activation delay for unknown case in FR1 and FR2-1. - Proposal 2: (Nokia, Ericsson) - RAN4 to study both known and unknown conditions for fast SCell activation with Rel-18 EMR. - Proposal 3: (Qualcomm, Samsung) - RAN4 to not consider the following SCells for the eEMR based fast SCell activation requirements: - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA) - SCell being in the same band as PCell in FR2 (i.e. intra-band CA) - Proposal 4: (Nokia) - RAN4 to consider all possible FR1 and FR2 scenarios for SCell activation, and the work can be initiated for FR1 Pcell + FR2 Pscell/SCell. - Proposal 5: (Nokia) - RAN4 should specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay). - Proposal 6: (Samsung) - RAN4 to consider the following scenarios as the starting point: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1

Both FR1 and FR2 needs to be considered to determine the targeting scenarios. First, in Rel-18 EMR, both FR1 and FR2 are mainly considered to specify the related RAN4 requirements, e.g., SCell delay enhancements. The delay reduction in SCell activation is also required to both FR1 and FR2 cases. Activating SCell enables the UE to increase a throughput by adding a transmission stream using SCell. When transmitting a given size of data, the UE can reduce the transmission delay by activating and using SCell. Thus, RAN4 should study and analyze the potential activation delay reduction in FR1 and FR2, respectively.

The gain on the total transmission delay reduction can highly depend on the initial throughput of the UE associated with PCell. If the transmission over PCell has a low throughput, the additional throughput provided by SCell can further reduce the total transmission delay. Comparing PCells in FR1 and FR2, the UE associated with PCell in FR1 has a low throughput. In that case, activating SCell in FR2 can effectively increase the performance gain due to the wider bandwidth part in FR2 than that of FR1. Hence, RAN4 can prioritize the scenario of FR1 Pcell FR2 Pscell while studying other combinations of FR1 and FR2 scenarios.

- RAN4 to consider all possible FR1 and FR2 scenarios for SCell activation, and the work can be initiated for FR1 Pcell FR2 Pscell/SCell.

SCell activation delay requirements

Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case. Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test Other proposals: - Proposal 6: For a UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements, the legacy known conditions with deactivated SCell L3 measurement still apply. - Proposal 7: - In case the SCell has more than one actually transmitted SSB, the report should be based on beam level measurement result, i.e. MeasIdleConfig > MeasIdleCarrierNR > BeamMeasConfigIdle-NR > includeBeamMeasurements = true. - Proposal 8: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication - The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation - [Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

In the previous meetings, companies have proposed to override the ValidityDuration with known condtion and introduce an additional “X” value that limits the network configuration. To consider the technical side of this issue, first it is required that the network configures a validityDuration for the UE. Currently we don’t have a rel-18 EMR test case for this validityDuration. Now we are overriding the validityDuration with X value. If we override the validityDuration with X value, we can then send an LS to RAN2 and ask them to remove the parameter as it has practically no impact. Better approach to this is to support known and unknown scenarios with short and long validityDuration values. We would like to highlight that in EMR framework, the network always request UE to provide results, and the UE always indicates if it has results. Hence, if the UE does not have confidence on the results due to changed radio conditions, the UE may always decide not to indicate measurement availability.

Not to introduce side condition X that limits the validityDuration

Discuss existing SSB based known SCell activation delay requirement with [2] SSB samples independently of the validityDuration configuration

Measurement period condition of FR1 known SCell activation

Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation Proposals - Option 1: (CATT) - No need to update the measurement period condition for FR1 known SCell activation. - Option 2a: (Apple) - For fast SCell activation based on eEMR, time for AGC is always needed since measurement period in IDLE/INACTIVE mode is longer than 2400ms in most cases. - Option 2b: (Qualcomm, Nokia) - In any case, the SCell activation requirement should allow UE to receive at least one SSB sample based on [SSB period] or [configured SMTC periodicity]. - Option 3: (Samsung) - If the SCell is known based on the eEMR based known condition and belongs to FR1, 2400 ms can be modified to Y ms The basic components to define Y and the subsequent impact on AGC training gate for activation shall be discussed

This issue was discussed in the meeting, and we are ok for UE to have at least one SSB sample in the Scell activation.

In any case, the SCell activation requirement should allow UE to receive at least one SSB sample based on [SSB period] or [configured SMTC periodicity].

Network Indication

Issue 1-2-4: Whether the indication to network is needed Proposals - Option 1: (CATT, Apple, Samsung) - Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation. - Option 2: (Ericsson, Nokia) - RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

Our understanding of this indication is that there are at least two cases where indication could be needed. First being that the known condition is not known to the network, hence the network needs always to receive a measurement report prior to activating the EMR result (if not blind activation). On the other hand, the SSB index is not always incdlued in the EMR results. RAN4 can discuss this indication further, and ask input from RAN2 if there are any problems of using Scell activation with the supported scenarios and EMR reporting. We have provided a draft LS

Send an LS to RAN2 and check whether additional information / indication (e.g. SSB index) is needed for the supported scenarios.

Conclusion

The following Observations and Proposals were made:

Observation 1: Work item cannot be closed with specification impact without including the case where  valdityDuration is not configured.

Proposal 1: RAN4 to conclude that it is not feasible to define requirements only for the case where validityDuration is configured.

Proposal 2: RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

Proposal 3: Define new known and unknown condition for Scell with EMR reporting

SCell in FR1 or FR2 is known if it has been meeting the following conditions:

	The UE has sent a measurement report for the SCell being activated according to reporting requirements in 4.7  or 5.8, and

	the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state, and

	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

Otherwise SCell in FR1 or FR2 is unknown.

Observation 2: ValidityDuration-r18 has still no relevance to UE radio conditions, and it is not feasible to have this as the only solution in rel18.

Proposal 4: When the validityDuration is not configured, if needed, to make the cell known, consider conditions that relate to the radio conditions:  mobility condition(s) and cell edge condition(s).

Proposal 6: Not to introduce side condition X that limits the validityDuration

Proposal 7: Discuss existing SSB based known SCell activation delay requirement with [2] SSB samples independently of the validityDuration configuration

Proposal 8: In any case, the SCell activation requirement should allow UE to receive at least one SSB sample based on [SSB period] or [configured SMTC periodicity].

Proposal 9: Send an LS to RAN2 and check whether additional information / indication (e.g. SSB index) is needed for the supported scenarios.