3GPP TSG-RAN WG4 Meeting #113	R4-2419733

Orlando, US, Nov 18 – Nov 22, 2024

Source:	Nokia

Title:	Discussion on LP-WUS RRM core requirements

Agenda item:	7.23.4.2

Document for:	Discussion

Introduction

In this contribution we discuss the remaining open issues of Rel-19 NR LP-WUS RRM core requirements.

Discussion

General about LP-WUS and existing requirements

LP-WUS has the potential to significantly benefit UE experiences, but it's important to design it carefully to minimize any negative impact on network performance Therefore, we should minimize impact on existing procedures. We should also agree that we avoid touching the existing MR requirements in this work item. When LP relaxations are not allowed, the existing legacy measurement delays and measurement accuracy requirements remain unchanged.

Existing MR measurement delays and accuracy requirements are kept unchanged.

Accuracy and number of samples

Issue 2-1-4: Accuracy baseline for simulation Agreement: Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only: - For FR1: - For OOK based LP-WUR, {±3.5, ±2.5} dB for LP-RSRP and {±3.5, ±2.5} dB for LP-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - For OFDM based LP-WUR, {±3.5, ±2.5} dB for SS-RSRP and {±3.5, ±2.5} dB for SS-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session - Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

In our simulation paper, we provide example accuracy values with number of samples, and we think it’s a good way to progress the situation. Currently it seems that 4 samples is needed for LP-SS measurements regardless of SNR and this also provide a decent accuracy in AWGN channel. We did also provide results in TDL-C for a reference.

Regarding the agreements in the last meeting. We think it is possible that OOK based accuracy needs to be revisited after a simulation campaign. The number of samples may not necessarily increase the accuracy dramatically, hence to avoid specifying too many samples, and to create realistic requirements, we can revisit the requirements.

Simulate LP-SS accuracy and decide number of samples based on accuracy.

Conclude final accuracy & sample number after simulations have been collected and compared

MR RRM Relaxation

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation Agreement RAN4 specifies relaxed UE MR measurement requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting LP-WUR

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell Aspect 1: Agree on relaxation/scaling factor(s) for MR serving/neighbour cell relaxation as the starting point - Support: vivo, ZTE, MTK, Samsung, Apple, HW, China Telecom, QC - Discuss the number later: Nokia, E///, OPPO, Xiaomi, LGE, CATT Aspect 2: The same or different relaxation/scaling factors for MR serving and neighbour cell relaxation - Same: MTK, vivo, QC, China Telecom, HW, OPPO, Xiaomi, CMCC, ZTE, Samsung, Apple - Different: CATT, Nokia - FFS Smaller factor for serving cell or neighboring cell Exact relaxation factor as the starting point - Option 1: Only consider the scaling factor(s) larger than that of legacy Rel-16 UE power saving WI (i.e., 3) (QC, CMCC, LGE, ZTE, HW, China Telecom, Apple, vivo, MTK, Xiaomi, Samsung) - Option 2: 3 is included as one of the factors (E///, Nokia) For the scaling factor(s) larger than 3, which number can be used as starting point. - 8: CMCC, China Telecom, vivo, Samsung, Ericsson, MTK, QC, Nokia - >=16: vivo, QC, HW, Apple, Samsung, MTK

From WID: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

In the last RAN4 meeting, relaxation factor was discussed for serving and neighbouring cell measurements. In the current discussion it is not clear whether the same relaxation factor applies for both serving and neighbouring cell measurements, and whether the relaxation factor is same regardless of the conditions where the UE is in. For instance, when the UE is configured with long DRX cycle, the impact of relaxation factor is much more significant than with short DRX cycle. Historically, the relaxation factors have been static values. However, currently LP-WUS is having multiple scenarios with various thresholds and conditions to consider. For instance, currently it is not clear if the mobility condition is used together with the cell edge criteria and how these impact the neighbouring cell measurements. Due to large number of scenarios, the relaxation factor should be configurable. This means that in specific deployments where the network can guarantee good cell centre conditions, UE would get larger power saving gains (larger relaxation factor. This allows also the network to consider the relaxation factor together with DRX / eDRX configuration.

If relaxation factor is not configurable, then the static relaxation factor is conservative,  for example 3.

Relaxation factor is configurable per serving and neighbouring cell measurements

Discuss the range of relaxation factor, once simulation work has progressed.

Different companies are proposing different MR relaxation factors based on the TR study and legacy Rel-16 UE power saving feature as a starting point. Some companies suggested to fix the relaxation factor first and evaluate other aspects. Our view is that it’s too early to agree/fix relaxation factor. And it’s still not clear to use the study TR relxation numbers as starting point for evaluation.

RAN4 to further study the MR Relaxation scaling factor before fixing the relaxation factor first and evaluate other aspects with simulations

Also it’s still not clear if both neighbor cell measurement and serving cell measurement shall share the same relaxation criteria or relaxation factor.

RAN4 needs to check if both neighbor cell measurement and serving cell measurement shall share the same relaxation criteria or relaxation factor.

LP-SS periodicity and PSS/SSS periodicity

Issue X: Measurement delay requirements for LP-WUR based on SSB in IDLE/Inactive mode shall be based on Option 1: Based on DRX configured for MR (vivo) Option 2: follow the same periodicity of LP-SS if LP-SS periodicity is configured (CMCC, Xiaomi, vivo, HW, Apple, QC, MTK) Option 3: A fix value (like 320ms), FFS on the value (Apple - if LP-SS periodicity is not configured, HW- if LP-SS periodicity is not configured, LGE - if LP-SS periodicity is not configured, ) Option 4: max of option 1 and 2 (QC) Option 5: max of option 2 and 3 (OPPO) Other options are not precluded. Nokia: further discuss the issue when SSB Periodicity and LP-SS periodicity are both configured, but network does not know which periodicity is followed. Further discuss in the next meeting: Case 1: If LP-SS periodicity is configured, Option A: follow the same periodicity of LP-SS. (Apple, vivo, QC, HW, LGE, MTK, Xiaomi, CMCC) Option B: Based on DRX configured for MR (Samsung) Option C: based on LP-SS periodicity (E////) Other options are not precluded. Case 2: If LP-SS periodicity is not configured Option A: Based on DRX configured for MR (vivo) Option B: A fix value (like 320ms), FFS on the value (Apple, HW, LGE, vivo) Other options are not precluded.

In the last meeting it was discussed whether LP-SS and PSS/SSS should have dependency based on LP-SS configuration. There should not be dependency between LP-SS configuration and PSS/SSS configuration to allow LR to operate independently from MR.

RAN1 is currently still deciding whether to support only one periodicity for LP-SS or multiple periodicities:

Agreement from RAN1#118-bis For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

There is no dependency between LR LP-SS periodicity configuration and PSS/SSS periodicity, these are independent.  In case companies want to have this, LS to RAN1 and 2 is triggered to clarify the scenario.

When UE is only supporting PSS/SSS, LR should follow configured PSS/SSS periodicity configuration, which may be different from MR PSS/SSS configuration. If RAN4 wants to progress more on this issue,

Define requirements with the assumption that UE performing LR LP-SS measurements only with LP-SS configured periodicity.

Define requirements for UE performing LR PSS/SSS measurements only with LR specific PSS/SSS periodicity only.

LP-SS periodicity is independent from PSS/SSS periodicity, i.e., one does not impact the other in RAN4 requirements

Threshold

Currently, RAN2 has made the following agreements:

### Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-SS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates. ### RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

RAN4 should consider the monitoring of LP-WUS and measurement relaxation and offloading separately. From the network point of view, it is important to be able to set meaningful thresholds for LP-WUS monitoring and measurement relaxation and offloading thresholds for both LP-SS and PSS/SSS.

The purpose of the LP-WUS monitoring threshold is to allow UE to stop legacy PO monitoring. If the UE is required to monitor paging occasions, MR is required to read an SSB before every paging occasion, and with this effort the MR would be able to perform and evaluate serving cell measurements. To summarise, the main point of LP-WUS entry and exit thresholds is to control paging monitoring. Relaxation / offloading is independently controlled (but may be simultaneously triggered).

For MR to receive paging, at least one SSB needs to be measured

The purpose of LP-WUS monitoring thresholds is stopping MR paging monitoring

The purpose of MR offloading / relaxation thresholds is to control when the MR serving / neighbouring cell measurements can be relaxed

From the procedure point of view, LP-WUS monitoring may start already before relaxations / offloading occur, hence the requirements should consider LP-WUS monitoring entry/exit independently. Entering LP-WUS monitoring may occur at different time to MR relaxation / offloading entry.

UE may stop paging monitoring once the LP-WUS monitoring conditions are fulfilled. During this time, LP-WUS monitoring requirements apply.

As agreed by RAN2, LP-WUS monitoring thresholds, which allow UE to stop monitoring legacy PO, are configured independently from MR offloading and relaxation thresholds. RAN4 shall define LP-WUS monitoring requirements independently from the MR measurement relaxation and offloading.

Criteria (entry/exit conditions)

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

Agreement from RAN2#127-bis:

Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

RAN2 is currently discussing the criteria, hence RAN4 can wait for progress.

RAN2 is currently discussing about the relaxation criteria and the discussion in RAN4 can wait until RAN2 outcome.

Measurement in idle mode and LP-WUS measurement relaxation

Issue 1-5-2: LR based RRM with EMR in IDLE/inactive mode - Proposals - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple)

LP-WUS is intended to be used for the benefit of UE power saving. One approach for UE power saving is to offload some or all current measurements performed by the MR to the LR. At least in the rel-18, only serving cell measurements are performed by LR. There are currently existing idle-mode measurements including reselection measurements, which need to be considered when UE is / has been in LP-WUS mode as the idle-mode measurement reporting is impacted if the UE has relaxed the measurements due to LP-WUS. When considering idle-mode inter-frequency measurements, CA/DC measurements should be also considered when UE is in LP-WUS mode and these have been relaxed.

The rel-18 work on fast CA/DC setup also expanded the idle-mode procedures to cover cell reselection measurements.  By using LP-WUS, the benefits of these features would be diminishing as there would be no measurements reports to provide to the network, since some or all measurements can be offloaded to LR and LR may not be able perform neighbour cell measurements. In some case even all MR measurements can be omitted when LP-WUS is used.

Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements

In the last meeting, some companies were proposing that when T331 is running, the UE will keep the EMR measurements running on configured carriers, this would mean that for the first 300 seconds in the idle-mode (T331 timer value), the UE should not enter LP-WUS mode when UE goes into idle-mode, in other words, the T331 condition overrides the MR relaxation condition. From the network point of view this is ok, as the performance in the idle-mode with shorter cycles can be guaranteed.

When T331 timer is configured, the UE will not go to idle-mode while the timer is running.

If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode)

RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Existing MR measurement delays and accuracy requirements are kept unchanged.

Proposal 2: Simulate LP-SS accuracy and decide number of samples based on accuracy.

Proposal 3: Conclude final accuracy & sample number after simulations have been collected and compared

Observation 1: If relaxation factor is not configurable, then the static relaxation factor is conservative,  for example 3.

Proposal 4: Relaxation factor is configurable per serving and neighbouring cell measurements

Proposal 5: Discuss the range of relaxation factor, once simulation work has progressed.

Proposal 6: RAN4 to further study the MR Relaxation scaling factor before fixing the relaxation factor first and evaluate other aspects with simulations

Proposal 7: RAN4 needs to check if both neighbor cell measurement and serving cell measurement shall share the same relaxation criteria or relaxation factor.

Proposal 8: There is no dependency between LR LP-SS periodicity configuration and PSS/SSS periodicity, these are independent.  In case companies want to have this, LS to RAN1 and 2 is triggered to clarify the scenario.

Proposal 9: Define requirements with the assumption that UE performing LR LP-SS measurements only with LP-SS configured periodicity.

Proposal 10: Define requirements for UE performing LR PSS/SSS measurements only with LR specific PSS/SSS periodicity only.

Proposal 11: LP-SS periodicity is independent from PSS/SSS periodicity, i.e., one does not impact the other in RAN4 requirements

Observation 2: For MR to receive paging, at least one SSB needs to be measured

Observation 3: The purpose of LP-WUS monitoring thresholds is stopping MR paging monitoring

Observation 4: The purpose of MR offloading / relaxation thresholds is to control when the MR serving / neighbouring cell measurements can be relaxed

Proposal 12: UE may stop paging monitoring once the LP-WUS monitoring conditions are fulfilled. During this time, LP-WUS monitoring requirements apply.

Proposal 13: As agreed by RAN2, LP-WUS monitoring thresholds, which allow UE to stop monitoring legacy PO, are configured independently from MR offloading and relaxation thresholds. RAN4 shall define LP-WUS monitoring requirements independently from the MR measuremen relaxation and offloading.

Proposal 14: RAN2 is currently discussing about the relaxation criteria and the discussion in RAN4 can wait until RAN2 outcome.

Proposal 15: Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements

Observation 5: When T331 timer is configured, the UE will not go to idle-mode while the timer is running.

Proposal 16: If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode)

Proposal 17: RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.