3GPP TSG-RAN WG4 Meeting 113		R4-2419734

Orlando, US, Nov 18th – Nov 22nd, 2024

Title:	LS reply on Fast RRC processing for rel-18 LTM

Response to:	LS R2-2409377 on LS on the fast RRC processing for LTM from RAN2

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4 #113

To:	RAN2

Cc:

Contact person:	Jani-Pekka Kainulainen

jani-pekka.kainulainen@nokia.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN2 sent an LS to RAN4 about fast RRC processing capability R2-2409377.

According to the RAN4 feature list, two components are included in the LTM fast RRC processing capability ltm-FastProcessingConfig-r18: - maxNumberStoredConfigCells, indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18, represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133. Currently, only the number of LTM candidate configurations is visible in LTM-config, while the number of SpCell/SCell(s) in LTM candidate configurations is not visible until the UE decodes LTM candidate configurations. RAN2 would like to know the answers for the following three questions: - As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations? - Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration? - Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

RAN4 has considered the issue and questions listed in RAN2 LS. In RAN4, the discussion related to the issue been about the number of candidate configurations, and the number of cells in the candidate configuration has not been discussed as an impacting factor. In our understanding, the UE capability is about the number of candidate configurations regardless of whether the configuration includes SCells or not. Furthermore, there are no requirements for Scell activation in LTM in rel-18.

Question 1 reply: Yes, this is the correct understanding.

Question 2 reply: No, the UE does need to know. RAN4 capability is about the number of candidate configurations, not about the cells inside the configuration.

Question 3 reply: Yes, it is up to the network to decide how many serving and candidate cells to configure. As long as the number of LTM candidate configurations and serving cells does not exceed maxNumberStoredConfigCells and the number of LTM candidate configurations does not exceed maxNumberConfigs, the UE shall be able to decode all LTM candidate configurations after receiving the RRC configuration

# 2	Actions

To RAN1 and RAN2

ACTION: 	RAN4 requests RAN2 to take the above information into consideration in their design for Rel-18 LTM.

# 3	Dates of next TSG RAN WG 2 meetings

TSG RAN WG4 Meeting #114 		17 – 21 February 2025	Athens, Greece

TSG RAN WG4 Meeting #114bis 	07 – 11 April 2024 Wuhan, China