3GPP TSG-RAN4 Meeting #113	R4-2419754

# Orlando USA, 18th – 22nd November, 2024

Source: 	Qualcomm Incorporated

Title: 	Post Deployment Testing of AI-ML

Agenda item:	7.17.1

Document for:	Discussion

# Introduction

In this paper, we focus on post deployment testing of AI-ML.

# Prior Agreements

RAN4 made the following agreement during the last meeting [1].

Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

# Discussion

## Notion of “Functionality” in one sided AI-ML

RAN2 sent the following LS to RAN1 recently [2]:

For functionality-based LCM for UE-sided model for beam management use case, RAN2 has studied and worked on the signalling procedure of applicable functionality reporting. RAN2 agreed the following understandings on terminologies: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference. RAN2 further has made following agreements and signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

RAN2’s LS to RAN1 suggests that network may send UE a set of functionalities for one sided AI-ML based beam management. Thereafter, based on UE’s reported applicable functionality, network sends inference configuration. Later, network can activate/deactivate inference configuration and functionality.

This LS illustrates that network and UE has a common understanding regarding functionality in one sided AI-ML. On the other hand, in one sided AI-ML, UE’s AI-ML model might be transparent to network. Hence, network will not know even if UE switches its model.

Observation 1: RAN2’s recent LS to RAN1 suggests that network and UE will have a common understanding regarding functionality in one sided AI-ML. On the other hand, in one sided AI-ML, UE’s AI-ML model might be transparent to network.

Hence, at least in the context of one-sided AI-ML, RAN4’s considered options for post deployment testing should focus on functionality, instead of model.

Proposal 1: At least, in the context of Rel-19 one-sided AI-ML model, RAN4 adopts the following changes in last meeting’s agreement (changes shown in blue color)

“For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive:

- Option 1: Conduct the validation of a change in functionality before its deployment/activation in already deployed UEs

  - Validation takes into account the UE hardware in which the model is to be deployed/activated.

  - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model)

  - FFS whether validation can be done at the device along with inference for another active functionality(ies)

  - FFS on the feasibility

  - FFS on one possibility for consideration for option 1 is to capture model fuctionality input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models.

  - Other aspects not precluded.

- Option 2: Using performance monitoring and LCM procedures

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - FFS on monitoring can be used for managing fallback, changes in functionality, model functionality update/model functionality switching/model functionality transfer, if applicable

FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device.

For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed.

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - Monitoring can be used for managing fallback, change in functionality, model functionality update/functionality model switching/functionality model transfer, if applicable

## Pre-deployment Testing vs LCM

Option 1 of last meeting’s agreement forces every AI-ML functionality to be tested before it gets deployed or activated in the field. In a real deployment, network operators and infra vendors may have to deploy many different functionalities to tackle different scenarios in the field. Testing every functionality before activation will delay the deployment of the corresponding functionality. Hence, RAN4 should use performance monitoring and LCM procedures to deal with potential changes in the performance of functionalities.

Observation 2: In a real deployment, network operators and infra vendors may have to deploy many different functionalities to tackle different scenarios in the field. Testing every functionality before activation will delay the deployment of the corresponding functionality.

Proposal 2: RAN4 supports option 2 of post-deployment testing, i.e., RAN4 uses performance monitoring and LCM procedures to deal with potential changes in the performance of functionalities.

# Conclusions

Observation 1: RAN2’s recent LS to RAN1 suggests that network and UE will have a common understanding regarding functionality in one sided AI-ML. On the other hand, in one sided AI-ML, UE’s AI-ML model might be transparent to network.

Proposal 1: At least, in the context of Rel-19 one-sided AI-ML model, RAN4 adopts the following changes in last meeting’s agreement (changes shown in blue color)

“For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive:

  - Option 1: Conduct the validation of a change in functionality before its deployment/activation in already deployed UEs

    - Validation takes into account the UE hardware in which the model is to be deployed/activated.

    - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model)

    - FFS whether validation can be done at the device along with inference for another active functionality(ies)

    - FFS on the feasibility

    - FFS on one possibility for consideration for option 1 is to capture model fuctionality input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models.

    - Other aspects not precluded.

  - Option 2: Using performance monitoring and LCM procedures

    - Performance monitoring will be designed in other groups

    - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

    - FFS on monitoring can be used for managing fallback, changes in functionality, model functionality update/model functionality switching/model functionality transfer, if applicable

FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device.

For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed.

    - Performance monitoring will be designed in other groups

    - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

    - Monitoring can be used for managing fallback, change in functionality, model functionality update/functionality model switching/functionality model transfer, if applicable

Observation 2: In a real deployment, network operators and infra vendors may have to deploy many different functionalities to tackle different scenarios in the field. Testing every functionality before activation will delay the deployment of the corresponding functionality.

Proposal 2: RAN4 supports option 2 of post-deployment testing, i.e., RAN4 uses performance monitoring and LCM procedures to deal with potential changes in the performance of functionalities.

# References

[1] R4-2417212, “WF on requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting P

[2] R2-2407848, “LS on applicable functionality reporting for beam management UE-sided model”,

3GPP TSG RAG WG2#127