3GPP TSG-RAN WG4 Meeting#113							         		R4-2419755

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	Input for R19 RedCap WI revision to enable PC2 FDD for RedCap UEs

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.7.1	UE RF requirements	[NR_PC2_RedCap_UE-Core]

Document for:	Approval

# 1	Background

In RAN#112, we discussed which requirements are relevant for both TDD, HD-FDD and FD-FDD 1Tx PC2 RedCap UEs in [1] in support to enabling PC2 FDD support for RedCap UEs. At the time it was suggested that this would not be possible before theDecember RAN meeting, but that technical justifications may still be discussed in RAN4. Since the December RAN meeting is just after this RAN4 meeting, in this contribution we summarize the input in [1] on the PC2 FDD aspects in view of a possible revision of the R19 RedCap HPUE WI by adding the support of FDD bands for RedCap HPUE.

# Discussion

## 2.1 Requirements for 1Tx FDD PC2 RedCap

In the current R18 specification, PC2 is defined for the following FDD Bands: n1/n84, n2, n3/n80, n7, n8, n13, n14, n70, n71 and n85.

For 1Tx PC2 RedCap FDD (both HD and FD) UEs the relevant aspects are listed here:

- RSD does not apply for HD-FDD bands, thus the PC3 RedCap REFSENS in 38.101-1 clause 7.3I.2 can be reused as-is.

- For FD-FDD bands, the PC2 1Tx or 2Tx RSD cannot be applied as is for 1Tx FD-FDD RedCap UEs with 1Rx. However, for 2Rx RedCap UEs, the PC2 1Tx RSD can be used as-is. Further study is needed to derive a proper FD-FDD RSD for 1Rx/1Tx RedCap UEs. This is discussed further after Table 2.

- The 1Tx PC2 MPR in Table 6.2.2-2 Maximum power reduction (MPR) for power class 2 applies.

- A-MPR: Note that NS_100 used for UTRA coexistence does not depend on RF filter rejection. Thus, the PC2 NS_100 A-MPR can be reused as-is.

  - For other band-specific NS requirements, one important aspect is whether the RF filter rejection is needed to meet the PC2 A-MPR. In Table 2 we collected the existing non-NS_01 requirements for PC2 FDD bands,

  - Observation: for some FDD bands, including Bands n1/n84 (NS_48/49), n7 (NS_46) and n13 (NS_07), the RF filter rejection is needed to meet the A-MPR requirement. Thus, RedCap UEs shall use an RF filter or use PAs that are significantly more linear than a 1Tx PC2 PA which is not available. However, this is also the case for PC3 that dedicated RF filters are needed. Thus, there is no additional issue for 1Tx PC2 FDD RedCap UE compared to PC3 FDD RedCap UEs.

Table 2: NS requirements for PC2 FDD bands

| Bands | NS | Emission Requirement | PC2 A-MPR (DFT-s-OFDM QPSK) | RF Filter assumption |
| --- | --- | --- | --- | --- |
| n1/n84 | NS_05(U) | -41dBm/300kHz in 1884.5-1915.7MHz. 5/10/15/20MHz CBW | 5/10/15/20MHz up to 13dB only lower edge channels | No rejection assumed? |
|  | NS_48 | +1.6dBm/5MHz 1915-1920MHz -15.5dBm/5MHz 1900-1915MHz -50dBm/MHz in band 34 | 10/15/20MHz up to 4.5dB only lower channels | Filter rejection needed, at least to meet n34 band protection and emissions <1915MHz |
|  | NS_49 | +1.6dBm/5MHz 1915-1920MHz -15.5dBm/5MHz 1895-1915MHz -40dBm/MHz in 1880-1895MHz -50dBm/MHz in band 34 | 10/15/20MHz up to 5.5dB only lower channels. filter for n34 and <1915MHz |  |
| n2/n25 n66/n70 | NS_03(U) | Only specific SEM and UTRA requirement | Any CBW only for outer. PC2=PC3? | No filter rejection needed |
| n7 | NS_46 | -40dBm/MHz in 2595-2620MHz -15.5dBm/5MHz in 2575-2595MHz +1.6dBm/MHz 2570-2575MHz | 10/15/20MHz up to 6.5dB | Duplex filter rejection assumed for offsets >10MHz |
| n8 | NS_43(U) | '-40dBm/MHz in 860-890MHz (in-band) for channels in 900-915MHz | 5/10/15MHz up to 12dB NS_43 + 9dB outer for NS_43U | No Filter rejection possible |
| n13/n14/n85 | NS_06 | Only specific SEM requirement | n-a | No filter rejection needed |
| n13 | NS_07 | -57dBm/6.25kHz in 769-775MHz. 5/10MHz CBW | 5/10MHz lower edge channels up to 14.5dB | Duplex filter rejection assumed |
| n71 | NS_35 | Only specific SEM requirement | Only edge RBs | No filter rejection needed |

On 1Rx/1Tx PC2 RSD for 1Rx FD-FDD RedCap UEs:

- For FD-FDD UE, the PC3 REFSENS is different to the “normal” UE REFSENS. Therefore, with RSD being a delta, it cannot be reused.

- The 1Tx RSD for “normal” UEs is improved due to the lower interference in the diversity path and the MRC gain, which is obviously not applicable for 1Rx case.

- The 2Tx RSD for “normal” UEs does not see the improvement since both antennas see the same interference. This case could be a good starting point accounting for the 3dB higher interference from a PC2 1Tx PA and the 1Rx REFSENS.

- Even if additional work is needed, the “normal” UE RSD framework can be reused.

Proposal for 1Tx PC2 FDD RedCap UEs:

- PC3 RedCap REFSENS in 38.101-1 clause 7.3I.2 can be reused as-is for 1Tx PC2 HD-FDD RedCap UE

- PC2 1Tx RSD can be reused as-is for 2Rx/1Tx RedCap FD-FDD UEs.

- For 1Rx/1Tx FD-FDD RedCap UEs:

  - 1Tx PC2 MPR in 38.101-1 Table 6.2.2-2 Maximum power reduction (MPR) for power class 2 can be reused as-is for 1Tx PC2 FDD RedCap UE

  - Like the corresponding PC3 case, to meet non-RedCap UE PC2 A-MPR for NS_07, NS_46 and NS_48/49, RF filter rejection is needed.

- Hence, there is only additional work for 1Tx FDD PC2 RedCap UEs compared to 1Tx PC3 FDD RedCap UEs is to develop RSD for 1Rx/1Tx FD-FDD case.

## 2.2 Proposal for PC2 RedCap UE scope

As discussed in Chapter 2.1, the work needed to develop the requirements for 1Tx PC2 TDD RedCap UE is minimal and can reuse all of the existing PC3 RedCap REFSENS and the “normal” UE 1Tx PC2 MPR/A-MPR.

As discussed in Chapter 22, the work needed to develop the requirements for 1Tx PC2 FDD RedCap UE is very similar to the TDD case, with only one additional aspect related to further developing the RSD framework to cover 1Tx/1Rx HD-FDD case.

Proposal for the WI scope:

- Given that 1Tx PC2 HD-FDD case is equivalent to the TDD case, this should be added as an objective without additional work. One of the TDD example bands could be changed to an HD-FDD Band.

- Given that the TDD and HD-FDD cases do not require the development of a generic specification framework, part of the effort could focus on further developing the RSD specification framework to encompass the 1Rx FD-FDD case to be added as an objective using the same band example than for HD-FDD.

- The FDD band example should be chosen such that Tx interference affects REFSENS (n1 and n66 should be avoided).

# Conclusion

In this contribution, we discussed the need for developing new generic requirement to handle 1Tx PC2 RedCap UEs for TDD, HD-FDD and FD-FDD cases. Based on our analysis, we formulate the following proposals.

Proposal for 1Tx PC2 FDD RedCap UEs:

- PC3 RedCap REFSENS in 38.101-1 clause 7.3I.2 can be reused as-is for 1Tx PC2 HD-FDD RedCap UE

- PC2 1Tx RSD can be reused as-is for 2Rx/1Tx RedCap FD-FDD UEs.

- For 1Rx/1Tx FD-FDD RedCap UEs:

  - 1Tx PC2 MPR in 38.101-1 Table 6.2.2-2 Maximum power reduction (MPR) for power class 2 can be reused as-is for 1Tx PC2 FDD RedCap UE.

  - Like the corresponding PC3 case, to meet non-RedCap UE PC2 A-MPR for NS_07, NS_46 and NS_48/49, RF filter rejection is needed.

- Hence, there is only additional work for 1Tx FDD PC2 RedCap UEs compared to 1Tx PC3 FDD RedCap UEs is to develop RSD for 1Rx/1Tx FD-FDD case.

Proposal for the WI scope:

- Given that 1Tx PC2 HD-FDD case is equivalent to the TDD case, this should be added as an objective without additional work. One of the TDD example bands could be changed to an HD-FDD Band.

- Given that the TDD and HD-FDD cases do not require the development of a generic specification framework, part of the effort could focus on further developing the RSD specification framework to encompass the 1Rx FD-FDD case to be added as an objective using the same band example than for HD-FDD.

- The FDD band example should be chosen such that Tx interference affects REFSENS (n1 and n66 should be avoided).

# References

[1] R4-2411242 Enabling PC2 RedCap UEs in TDD and FDD bands, Skyworks Solutions Inc., RAN4#112

[2] R4-2414286 WF on RedCap PC2 requirements China Telecom, RAN4#112bis