3GPP TSG-RAN WG4 Meeting #113	R4-2419764

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.23.4.2

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for LP-WUS/WUR

Document for:	Discussion

# Introduction

RAN4 has been discussing various scenarios and issues related to RRM requirements over the past few meetings also achieved few agreements [2]-[5] on the scope of the RRM requirements. In this paper, we further discuss some aspects to be considered for RRM core requirements.

# RRM measurements relaxation/offloading

- Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

- Measurement delay requirements for LP-WUR based on SSB in IDLE/Inactive mode shall be based on

  - Case 1: If LP-SS periodicity is configured,

    - Option A: follow the same periodicity of LP-SS. (Apple, vivo, QC, HW, LGE, MTK, Xiaomi, CMCC)

    - Option B: Based on DRX configured for MR (Samsung)

    - Option C: based on LP-SS periodicity (E////)

    - Other options are not precluded.

  - Case 2: If LP-SS periodicity is not configured

    - Option A: Based on DRX configured for MR (vivo)

    - Option B: A fix value (like 320ms), FFS on the value (Apple, HW, LGE, vivo)

    - Other options are not precluded.

The first issue that we want to address is related to RRM requirements specification considering the two types of WURs – OFDM based and OOK based. While OOK based WUR always processes LP-SS, the OFDM based WUR may process either LP-SS or SSBs. RAN4 made the following agreements on this issue during the last meeting:

First, we would like to understand if it’s possible that LP-SS is not configured to the UE. Based on the WID, we think LP-SS is always transmitted and there is no reason that the LP-SS periodicity is not configured to the UE. If that’s the case, then RAN4 doesn’t need to discuss case 2.

Observation 1: Based on the WID, LP-SS is always transmitted by the network.

Proposal 1: Specify RAN4 requirements under the assumption that the LP-SS configuration (resources, periodicity etc.) is always provided by the network to the UE.

Secondly, to decide the measurement periodicity by the WUR, the power savings gains should be carefully assessed. Frequent measurements by the WUR as compared to the legacy MR based measurements, will lead to diminishing power savings and may provide little incentive for the WUR deployment. For instance, performing measurements according to, let’s say, an LP-SS periodicity of 320ms may lead to 4 and 8 times more measurements as compared to the MR based measurements with 1.28s and 2.56s DRX periodicity, respectively. This significantly limits the power savings that can be achieved with the LP-WUR. The situation may get even worse with eDRX, where we may even see negative power savings (i.e., the UE consumes more power with WUR operation). So, we think, for both type of WUR, the baseline periodicity of measuring LP-SS or SSB should be a multiple of LP-SS that is decided based on the corresponding DRX configuration.

Observation 2: Specifying the measurement periodicity of the WUR based on LP-SS periodicity may limit the power savings as compared to DRX and may even lead to negative power savings with eDRX.

Proposal 2: The baseline periodicity of measuring LP-SS or SSB should be a multiple of LP-SS periodicity that is decided based on the corresponding DRX configuration.

The next issue is related to WUR implementation. Although 3GPP may assume that the WUR is completely detached from the MR, it’s highly likely that UE may share some RF components between the two radios. So, for CONNECTED mode, when the UE uses measurement gaps for neighbour cell measurements, it may tune away from the serving cell. This typically causes interruptions on the UE where the UE cannot transmit or receive any signal other than those intended for RRM measurements. We think such interruptions may also be needed for WUS monitoring when the WUS MO collides with the MG.

Observation 3: In the Connected mode, the UE may not be able to monitor WUS during MGs as it may retune to a different carrier frequency for measurements.

Proposal 3: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs.

# Conclusion

Observation 1: Based on the WID, LP-SS is always transmitted by the network.

Proposal 1: Specify RAN4 requirements under the assumption that the LP-SS configuration (resources, periodicity etc.) is always provided by the network to the UE.

Observation 2: Specifying the measurement periodicity of the WUR based on LP-SS periodicity may limit the power savings as compared to DRX and may even lead to negative power savings with eDRX.

Proposal 2: The baseline periodicity of measuring LP-SS or SSB should be a multiple of LP-SS periodicity that is decided based on the corresponding DRX configuration.

Observation 3: In the Connected mode, the UE may not be able to monitor WUS during MGs as it may retune to a different carrier frequency for measurements.

Proposal 3: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs.

# References

[1] 	RP-240801, “Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR)”, vivo, RAN#103

[2] R4-2406367, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#110bis

[3] R4-2410296, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#111

[4] R4-2413899, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112

[5] R4-2416861, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112bis