3GPP TSG-RAN WG4 Meeting #113	R4-2419768

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Discussion on remaining issues for less than 5Mhz ph2

Agenda item:	7.4.3

Document for:	Discussion

Introduction

This contribution discusses on the remaining issues of less than 5MHz.

Discussion

Issue 1-1-3: Whether intra-band CA is in the scope: - Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations. - Option 3: No need to discuss since there is no impact to RRM requirements in Rel-19.

In the last meeting discussion on intra-band support was introduced. Currently only inter-band CA is considered in RAN4 RF for both contiguous non-contiguous case. Therefore, we don’t see the need for defining intra-band requirements.

Consider only inter-band CA for contiguous and non-contiguous case.

The main changes in the rel-18 were related to SSB index reading for L3 measurements due to PBCH puncturing and with PDCCH RLM and BFD requirements.

In the rel-18 WI, the following agreement was made on which component the extra delay is added.

Agreement: - For HO requirements, include the 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆ instead of Tsearch. - For less than 5MHz requirement, include the additional time for T∆ in the spec. Agreement: - Inter-frequency RRC connection re-establishment: - Side conditions for NR is target cell detection for RRC connection re-establishment is Es/Iot≥-4 dB. - Intra-frequency RRC connection re-establishment: - Side conditions for NR is target cell detection for RRC connection re-establishment is Es/Iot≥-6 dB. - RRC connection release with re-direction: - Side conditions for NR target cell detection for RRC connection release with re-direction is Es/Iot≥-4 dB. Side conditions for defining Tsearch in HO requirements for 12PRB SSB and Tsearch in HO requirements for 12PRB SSB. Agreement: - Unknown intra-frequency target cell: target cell Es/Iot≥-2 dB. - Unknown inter-frequency target cell: target cell Es/Iot≥-2 dB. - Unknown intra-frequency target cell: [3]*Trs ms (target cell Es/Iot≥-2 dB). - Unknown inter-frequency target cell: [5]*Trs ms (target cell Es/Iot≥-2 dB).

In rel-19, follow rel-18 baseline and add 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆, where applicable.

In rel-19, follow rel-18 baseline and add 2 additional SSB samples where SSB index reading is required.

As many of the issues are quite similar to each other, RAN4 can discuss them while drafting the Big CR.

The remaining issues can be discussed directly in the CRs

Conclusion

The following Observations and Proposals were made:

Proposal 1: Consider only inter-band CA for contiguous and non-contiguous case.

Proposal 2: In rel-19, follow rel-18 baseline and add 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆, where applicable.

Proposal 3: In rel-19, follow rel-18 baseline and add 2 additional SSB samples where SSB index reading is required.

Proposal 4: The remaining issues can be discussed directly in the CRs