3GPP TSG-RAN WG4 Meeting #113	R4-2419772

Orlando, Florida, USA, November 18 – 22, 2024

Source:	Nokia

Title:	RRM performance requirements for NR Carrier Phase Positioning

Agenda item:	5.22.2.6

Document for:	Discussion

Introduction

The RRM performance part of the NR_pos_enh2 WI [1] is foreseen to be closed at RAN4 #113. Nevertheless, some aspects for NR carrier phase positioning (NR CPP) still need to be finalized.

This contribution provides our view on open issues for RRM performance requirement for NR CPP [2].

Discussion

RAN4 agreements and open issues on performance requirements

RAN4 agreements and list of open issues from RAN4 #112bis are captured below [2]. More details are available in [3].

## Sub-topic 2-2 Carrier Phase Positioning Performance Requirements ### Issue 2-2-1: Applicable scenarios for CPP measurements Agreement: RAN4 to only define accuracy for the scenarios where CPP measurements are performed on PRS resources within the same sets of slots and sets of symbols (i.e. the agreed scenario), considering PRS repetitions. - Set of symbols is based on PRS configuration. ### Issue 2-2-2: Margins for accuracy requirements due to carrier phase offset/RSTD uncertainty Agreement: - Define accuracy requirements based on the available simulation results for: - FR1: - BW>24 PRBs: [] can be removed - 24 PRBs: keep [] until RAN4#113 - FR2: - 60 kHz - BW>24 PRBs: [] can be removed - 24 PRBs: keep [] until RAN4#113 - 120 kHz - BW>32 PRBs: [] can be removed - 32 PRBs: keep [] until RAN4#113

### Issue 2-2-3: Accuracy requirements for additional PRS configurations Agreement: For RSCP/RSCPD accuracy requirements: - CATT to update the simulation results summary, including Nokia’s results. The latest version can be found in R4-2416906. - Discuss the average numbers and include the agreeable numbers and configurations into CR (revision of R4-2416329) - The requirements may be updated in the next meeting (up to the discussion in the next meeting) Interested companies can provide simulation results for deriving the TBDs, e.g. in Table 10.1.43.2 (Measurement Accuracy Requirements for RSCPD).

Additional RF margin for frequency drift and RF calibration

Regarding issue 2-2-2, the impact of CFO has been discussed for a number of meetings. Yet, there is no evidence in our view that this impact is negligible for CPP.

For deriving the CFO impact to NR DL CPP, for measuring RSCPD or DL RSCP, it is assumed that the UE receives PRS resources from different TRPs on the same PFL within the same time window. Thus, no CFO is assumed at the UE side. However, for the TRPs used for the measurement, i.e. reference TRP and target TRP, different CFOs need to be assumed. As the CFO impairment is similar or even same at target UE and PRU, the LMF may compensate this impairment to some degree. However, for the reported measurement quantity, the CFO impairment considerably affects the CP measurement accuracy, as also shown in [7].

Based on TS 38.104, clause 6.5.1.2, that Wide Area BS has a frequency error of ±0.05 ppm, and Medium Range and Local Area BSs have a frequency error of ±0.1 ppm, the maximum CFO difference between reference TRP and target TRP is derived for the PRS resource composed of 4 symbols in a slot and repeated instances in adjacent slots in case of configured PRS repetition > 1, based on the simulation assumptions for CP measurement accuracy. The analysis is provided in Table 1 below.

It is observed that the CFO impairment makes the CP measurement useless in case of PRS repetition = 4, which extends the measurement period over more than 3 slots. Thus, it is proposed to remove these PRS configurations from the measurement accuracy requirements for CPP.

For FR1, for SCS=15kHz in 2GHz and SCS=30kHz/60 kHz in 4 GHz, the CFO margin is limited to 21 and 10 degrees, respectively, in case of Wide Area BS and to 41 and 21 degrees, respectively in case of Medium Range and Local Area BS.

For FR2, due to the much higher carrier frequency, the CFO impact is more severe and yields to too high degradations largely exceeding 100 degrees. Only for SCS=120kHz the CFO margin is 51 degrees for Wide Area BS and 103 degrees for Medium Range/Local Area BS. It is proposed to keep only these PRS configurations based on SCS=120kHz and PRS repetition = 1 in the CP measurement accuracy requirements.

RAN4 to remove following PRS configurations for RSCPD and DL RSCP measurement accuracy:   i) configurations with PRS repetition=4 and ii) configurations with SCS=60kHz in FR2 (40 GHz).

Hence for the green marked PRS configurations in Table 1, it is proposed to define CP measurement accuracy requirements including the CFO margin as proposed in Table 1.

Table 1: Derivation of CFO margin for RSCPD and DL RSCP for NR DL CPP.

In fact, the CFO identifies the dominant impairment for RSCPD and DL RSCP and hence can be used to define both the RSCPD and DL RSCP margins.

This yields to following proposal for the RSCPD and DL RSCP margins in TS 38.133 based on the distinction of PRS signals originated by TRPs belonging to Wide Area BS class or Medium Range/Local Area BS class.

RAN4 to adopt the following margins for RSCPD and DL RSCP in the big CR to TS 38.133:

Table 10.1.43.2-5: Margin for DL RSCPD measurement accuracy in FR1 for reference and target TRP belonging to Wide Area BS class [TS 38.104].

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 52 | ≥ 48 | N/A | [21] |
| N/A | N/A | ≥ 64 | [10] |

Table 10.1.43.2-6: Margin for DL RSCPD measurement accuracy in FR1 for reference and target TRP belonging to Medium Range or Local Area BS class [TS 38.104].

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 52 | ≥ 48 | N/A | [41] |
| N/A | N/A | ≥ 64 | [21] |

Table 10.1.43.2-7: Margin for DL RSCPD measurement accuracy in FR2 for reference and target TRP belonging to Wide Area BS class [TS 38.104].

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| N/A | ≥ 64 | [51] |

Table 10.1.43.2-8: Margin for DL RSCPD measurement accuracy in FR2 for reference and target TRP belonging to Medium Range or Local Area BS class [TS 38.104].

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| N/A | ≥ 64 | [103] |

Table 10.1.44.2-5: Margin for DL RSCP measurement accuracy in FR1 for reference and target TRP belonging to Wide Area BS class [TS 38.104].

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 52 | ≥ 48 | N/A | [21] |
| N/A | N/A | ≥ 64 | [10] |

Table 10.1.44.2-6: Margin for DL RSCP measurement accuracy in FR1 for reference and target TRP belonging to Medium Range or Local Area BS class [TS 38.104].

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 52 | ≥ 48 | N/A | [41] |
| N/A | N/A | ≥ 64 | [21] |

Table 10.1.44.2-7: Margin for DL RSCP measurement accuracy in FR2 for reference and target TRP belonging to Wide Area BS class [TS 38.104].

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| N/A | ≥ 64 | [51] |

Table 10.1.44.2-8: Margin for DL RSCP measurement accuracy in FR2 for reference and target TRP belonging to Medium Range or Local Area BS class [TS 38.104].

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| N/A | ≥ 64 | [103] |

Proposals 1 and 2 are implemented in the accompanying draft CR [8].

  - Accuracy performance for additional PRS configurations

It was observed at RAN4 #112 that accuracy requirements for CP measurements (RSCPD/DL RSCP) for some PRS configurations were missed out in the simulations. In particular, accuracy requirements for the following highlighted configurations in Table 1 need to be determined to update the draft big CR [4].

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 24 | 4 | 256 |
|  |  | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  |  | 132 | 1 | 32 |
|  | 60, FR1 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR2 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 32 | 4 | 32 |
|  |  | 64 | 1 | 16 |
|  |  | 128 | 1 | 8 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 1 |  |  |  |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13)dB for NSample = 1; (-3, -6, -6)dB for NSample = 1. (-3, -13, -13)dB for NSample = 1; (0, -6, -6)dB for NSample = 1. |  |  |  |
| PRS periodicity | 40 ms |  |  |  |
| Network synchronization | Synchronous with time shifts <0, 0, 3 us> |  |  |  |

Table 1: PRS configurations used for NR RSCPD study for SCS = 15kHz, 30kHz, 60kHz and 120 kHz (FR1 and FR2).

For these additional PRS configurations, we provided our simulation results in [5], which were captured in the latest simulation summary [6].

This yields to following proposal.

Take into account simulation results in R4-2416437 for the additional PRS configurations to determine remaining CP measurement accuracy requirements in the draft big CR.

Conclusion

Remaining open issues for RRM performance requirements for NR carrier phase positioning (NR CPP) being one of the objectives in the Rel-18 WID on expanded and improved positioning [1], as depicted in [2] and [4] are discussed in this contribution and corresponding proposals are made.

The following proposals are made.

- RAN4 to remove following PRS configurations for RSCPD and DL RSCP measurement accuracy:   i) configurations with PRS repetition=4 and ii) configurations with SCS=60kHz in FR2 (40 GHz).

- RAN4 to adopt the following margins for RSCPD and DL RSCP in the big CR to TS 38.133:

Table 10.1.43.2-5: Margin for DL RSCPD measurement accuracy in FR1 for reference and target TRP belonging to Wide Area BS class [TS 38.104].

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 52 | ≥ 48 | N/A | [21] |
| N/A | N/A | ≥ 64 | [10] |

Table 10.1.43.2-6: Margin for DL RSCPD measurement accuracy in FR1 for reference and target TRP belonging to Medium Range or Local Area BS class [TS 38.104].

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 52 | ≥ 48 | N/A | [41] |
| N/A | N/A | ≥ 64 | [21] |

Table 10.1.43.2-7: Margin for DL RSCPD measurement accuracy in FR2 for reference and target TRP belonging to Wide Area BS class [TS 38.104].

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| N/A | ≥ 64 | [51] |

Table 10.1.43.2-8: Margin for DL RSCPD measurement accuracy in FR2 for reference and target TRP belonging to Medium Range or Local Area BS class [TS 38.104].

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| N/A | ≥ 64 | [103] |

Table 10.1.44.2-5: Margin for DL RSCP measurement accuracy in FR1 for reference and target TRP belonging to Wide Area BS class [TS 38.104].

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 52 | ≥ 48 | N/A | [21] |
| N/A | N/A | ≥ 64 | [10] |

Table 10.1.44.2-6: Margin for DL RSCP measurement accuracy in FR1 for reference and target TRP belonging to Medium Range or Local Area BS class [TS 38.104].

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 52 | ≥ 48 | N/A | [41] |
| N/A | N/A | ≥ 64 | [21] |

Table 10.1.44.2-7: Margin for DL RSCP measurement accuracy in FR2 for reference and target TRP belonging to Wide Area BS class [TS 38.104].

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| N/A | ≥ 64 | [51] |

Table 10.1.44.2-8: Margin for DL RSCP measurement accuracy in FR2 for reference and target TRP belonging to Medium Range or Local Area BS class [TS 38.104].

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| N/A | ≥ 64 | [103] |

It is noted Proposals 1 and 2 are implemented in the accompanying draft CR [8].

- Take into account simulation results in R4-2416437 for the additional PRS configurations to determine remaining CP measurement accuracy requirements in the big CR.

- References

- RP-233382, “Revised WID on Expanded and Improved NR positioning”, 3GPP TSG RAN#102

- R4-2416870, “WF on RRM requirements for NR_pos_enh2_Part2”, CATT

- R4-2415605, “Topic summary for [112bis][203] NR_pos_enh2_Part2”, Moderator (CATT)

- R4-2416892, “Draft big CR to 38.133 for RRM performance part for expanded and improved NR positioning”, Ericsson

- R4-2416437, “Simulation results for DL RSCPD and DL RSCP”, Nokia

- R4-2416906, “Updated summary of simulation results for CPP accuracy requirements”, CATT

- R4-2416505, “On the Need for Additional Scenarios for CPP”, Lenovo

- R4-2419773,”Draft CR 38.133 Introduction of margins for DL RSCPD and DL RSCP measurement accuracy”, Nokia