3GPP TSG-RAN WG4 Meeting#113                             R4-2419818

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.11.2

Source: 	ZTE Corporation, Sanechips, Nokia

Title: 	TP to TR38.908: clause 3 and 5

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. This TP is to provide some inputs for clause 3 and also some updates for 5 to align with core spec.

<Start of TP>

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain BS antenna array, there may be more than one beam.

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

3.2	Symbols

For the purposes of the present document, the following symbols apply:

	 elevation angle above horizon (defined between 0° and 90°)

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EIRP	Effective Isotropic Radiated Power

EEIRP	Expected EIRP

FSS	Fixed Satellite Service

NR	New Radio

RF	Radio Frequency

5	RF requirement

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions. This requirement shall be applied to BS operating in band n104 to protect FSS (Earth-to-space) satellite receiver.

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the limits specified in table 5-1.

The expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed:

‒ over azimuth angles from −180º to +180º with reference to BS horizontal boresight and over the specified range of angles θHLi ≤ θ < θHHi , above the horizon as defined in Table 5-1, and

‒ over the supported beam directions.

Note: The beamforming directions over time means directions are measured sequentially in the test.

Table 5-1: EEIRP limits as function of elevation above horizon

| Bin number | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<30 | 16 |
| 6 | 30≤<60 | 15 |
| 7 | 60≤<90 | 15 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |  |

The definitions of elevation angle θHLi and θHHi are illustrated in Figure 5-1.

Figure 5-1: Definitions of θHLi and θHHi angles.

<End of TP>