3GPP TSG-RAN WG4 Meeting #113												 	   		R4-2420089

Orlando, US, 18th – 22nd November 2024

Agenda item:			7.4.4

Source:	Intel Corporation

Title:	WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2

Document for:	Approval

# Topic #1: RRM requirements

## Agreements summary for topic #1

Issue 1-1-1: Intra-band CA RRM requirements

Agreement:

- According to the WID, intra-band CA is not in the WI scope.

Issue 1-1-2: NW indication on PBCH BW in MO configuration:

Agreement:

- No need to introduce network indication on PRB number for PBCH transmission.

Issue 1-1-3: NW indication on PBCH BW in HO command with direct SCell activation:

Agreement:

- No need to introduce new indication.

Issue 1-1-5: PSCell change delay requirements:

Agreement: No spec impact for this issue given the red text under option 1 is already in the spec.

Issue 1-1-6: Handover with PSCell delay requirements:

Agreement:

- Since the handover with PSCell delay requirements refer to the PSCell addition delay requirements, there is no need to update the specification in Rel-19.

Issue 1-1-7: Measurement requirements for deactivated SCC:

Agreement:

- Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

Table 9.2.5.1-x: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Issue 1-1-8: Conditional handover delay requirements to PCell with 12PRB SSB:

Agreement: Not specify CHO requirement for target Pcell with 12 PRB SSB as part of Rel-19 WI.