3GPP TSG-RAN WG4 Meeting #113	R4-2420092

# Orlando, US, 18th – 22nd November, 2024

Title: 	WF on RRM requirements for NR_ATG_enh

Agenda Item:	7.10.5

Source: 	CMCC

Document for:	Approval

<Topic 1> RRM core requirement

<Sub-topic 1-1> General

Issue 1-1-1: UE antenna type for inter-band CA

FFS

Issue 1-1-2: UE Antenna Type signaling

FFS

<Sub-topic 1-2> Timing

Issue 1-2-1: MRTD for ATG inter-band CA

FFS

- Option 1:

  - For the MRTD except Case#4: 33µs. Further discuss for case 4.

- Option 2:

  - For the MRTD except Case#4: 33µs.

  - If case 4 is not excluded based on RF discussion: 3µs.

- Option 3:

  - For the MRTD for all cases: 33µs.

<Sub-topic 1-3> Signalling characteristics

Issue 1-3-1: Interruption requirements

Agreement:

- Introduce following requirements for R19 ATG

  - Interruptions at SCell addition/release

  - Interruptions at SCell activation/deactivation

  - Interruptions at direct SCell activation

  - Interruptions at fast SCell activation

  - Interruptions during measurements on deactivated SCC

  - Interruptions due to SCell dormancy

  - Interruptions due to UE-specific CBW change

  - Interruptions due to Active BWP switching Requirement

  - Interruptions when identifying CGI of an NR cell with autonomous gaps

- Not introduce following requirements for R19 ATG

  - Interruptions at UL carrier RRC reconfiguration

  - Interruptions at inter-frequency SFTD measurement

  - Interruptions at NR SRS carrier based switching

  - Interruptions due to PUCCH SCell activation/deactivation

  - Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

- FFS whether following requirements need to be introduced

  - Interruptions at NR SRS antenna port switching

- Further discuss the potential impact from UE antenna types

- Further discuss how to reflect the interruption requirement for R19 ATG

Issue 1-3-2: SCell Activation and Deactivation Delay

FFS

Issue 1-3-3: SCell Addition/Activation mechanism

FFS

Issue 1-3-4: Active BWP switch delay on multiple CCs

Agreement:

- Introduce following requirements and the legacy can be reused as baseline

  - Simultaneous DCI based BWP switch delay on multiple CCs

  - Simultaneous RRC based BWP switch delay on multiple CCs

  - Simultaneous Timer based BWP switch delay on multiple CCs

  - non-simultaneous Timer based BWP switch delay on multiple CCs

Issue 1-3-5: Pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation

Agreement:

- Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

<Sub-topic 1-4> Measurement procedure

Issue 1-4-1: Capabilities for Support of Event Triggering and Reporting Criteria

Agreement:

- For capabilities for Support of Event Triggering and Reporting Criteria, configured NR serving frequencies will include PCell and SCells carrier frequencies in the total number of NR reporting criteria for intra-frequency category.

Issue 1-4-2: Pre-configured measurement gap requirement

Agreement:

- SCell related triggering conditions should be considered for network-controlled and autonomous activation/deactivation mechanism of pre-configured measurement gap, and the legacy requirement can be reused.

Issue 1-4-3: SCell activation triggered measurement reporting requirement

Agreement:

- The measurement reporting requirement for SCell activation Triggered Reporting shall be introduced and the legacy requirements can be reused.

Issue 1-4-4: Deactivated SCell measurement

Agreement:

- beam sweeping factor N1 is needed.

- FFS on L3/L1 measurement sharing factor

Issue 1-4-5: Scheduling Availability/Restrictions of UE performing measurements on FR1 TDD band

Agreement:

- For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, the scheduling restriction extension to intra-band/inter-band CA is identified for R19 ATG

  - When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

  - When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Issue 1-4-6: Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1

FFS

Issue 1-4-7: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

FFS

Issue 1-4-8: Sharing factor between L1 measurement and RRM measurement

FFS

Reference

[1] R4-2418275, Topic summary for [113][217] NR_ATG_enh