3GPP TSG-RAN WG4 Meeting #113												R4-2420528

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.23.5

Source:	vivo

Title:	WF on UE RF requirements for LP-WUS

Document for:	Approval

Introduction

This is WF for Rel-19 LP-WUS UE RF.

Topic #1: General and system parameters

Sub-topic 1-1 General

Issue 1-1-3: General issue on how to introduce LP-WUS feature into core specs 38.101-x

Agreements:

  - Add a new suffix “M” as “LP-WUS/WUR”

Issue 1-1-4: views on which requirements should be updated in core spec

Agreements:

  - The part should be updated for LP-WUS/WUR in spec.

    - The additional requirements are not precluded, if needed.

| Sections or requirements | Needed or not? |
| --- | --- |
| Operation bands | FFS |
| UE channel bandwidth | FFS |
| Channel raster | FFS |
| REFSENS | YES |
| Maximum input level | YES |
| ACS | YES |
| ASCS | YES |
| In-band blocking | YES |
| Out-of-band blocking | YES |
| Narrow band blocking | YES |
| Spurious response | FFS |
| Wide band Intermodulation | FFS |
| Spurious emissions | FFS |
| Annex A Measurement channels | YES |

Sub-topic 1-2 System parameters

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

  - No new channel raster is introduced dedicated for LP-WUS/WUR.

    - FFS on whether there is need of the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost

Topic #2: REFSENS, ASCS and ACS requirements

Sub-topic 2-1 SNR simulation and values

Issue 2-1-1: Target FR1 SNR value for LP-WUS/WUR

Agreements:

  - Use worst case waveform of OOK-4 M=4 to derive SNR.

    - Some detailed aspects should be also confirmed, e.g., CRC bit (8), sequence length (8, 16), codepoint length of known part of the sequence)

Issue 2-1-3: Link level simulation assumption for FR2 LP-WUR

Agreements:

  - Further discussions and alignment on FR2 RF simulation assumption not precluded.

  - Use the following as starting point

| Attributes | Assumptions |  |  | Notes |
| --- | --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |  |
| Center frequency | 24300MHz |  |  | n258 example band |
| WUS structure | Total 8/16/32 bits |  |  | (placeholder for RAN1 decision) |
| Channel structure |  |  |  | not needed |
| Chip rate | M=1 | M=1/2/4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |  |
| Time error | up to UE implementation |  |  | Difficult to separate time and frequency errors in realistic receivers |
| Residual Frequency error | 0/10/20 ppm |  |  |  |
| SCS | 120kHz |  |  |  |
| UE Channel BW | 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2 |  |  |  |
| WUS RB | - Fixed 11RB |  |  | Based on agreement in RAN1#118-Bis |
| Position within channel | - Center for ASCS, edge for ACS [assume no ASCS impact] |  |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - Refer to 5.3.3 in TS38.101-2 |  |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |  |
| Filter | - 3th/5th Order lowpass Butterworth - Other order lowpass filter is not precluded |  |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; NOTE: decide the interference level depending on SNR |  |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |  |
| Sampling rate | TBD |  |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |  |
| Phase noise | FFS |  |  |  |
| Non-linearities | Not modelled |  |  |  |
| Power boosting | 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |  |
| Channel Model | Option 1: TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  | The procedure for using CDL channel models in demod analysis is not fully established. |

Sub-topic 2-2 NF and REFSENS requirements

Issue 2-2-1: General views on REFSENS for LP-WUS

Agreements:

  - Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values.

Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR

- Proposals

  - Proposal 1: check the following input from companies and converge the range for OOK-based LR.  (Moderator)

| FR1 OOK-based receiver | Huawei | Sony | Spread trum | Apple | vivo | QC | Samsung | LGE | CMCC | ZTE | Nokia | E// |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IM(dB) | 2.5 | 2.5 | 2.5 | >=2.5 | 2.5 | 2.5 | 3 | 2.5 | 2.5 | 2.5 | 1 | 2.5 |
| NF(dB) (assume MR=9dB) | 17 | 10 | 17 | 16 | 15 | 11.5 | 15 | 16 | 12 | 14 | 12 | 12 |
| SNR(dB) | -2 | -7~-2 | -3 |  | -2 | -1 for sequence length of 18 | -2 |  | -2.5 | -2 |  | -1.5~ -5 |

Note: the SNR value may be updated based on new simulation results. RAN1 waveform design is not finalized as of this WF.

Agreements:

  - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported.

  - The total value of NF+IM can be considered.

  - On how to handle the NF values:

    - Option 1: For NF whether average value can be adopted.

    - Option 2: two sets NFs for different LP-WUR types with different power consumption target

    - Option 3: also consider coverage

  - Capture the table in WF as background information

Issue 2-2-4: companies input on SNR, NF and IM value for FR1 OFDM-based LR

- Proposals

  - Proposal 1: check the following input from companies and converge the range for OFDM-based LR.  (Moderator)

| FR1 OFDM-based receiver | Huawei | Sony | Spread trum | Apple | vivo | QC | Samsung | LGE | CMCC | ZTE | Nokia | E// |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IM(dB) | 2.5 | 2.5 | 2.5 | >=2.5 | 2.5 |  |  | 2.5 | 2.5 | 2.5 | 1 | 2.5 |
| NF(dB)(assume MR=9dB) | 11 | 10 | 11 | 16 | 11 |  |  | 12 | 9 | 11 | 12 | 12~13 |
| SNR(dB) | -2 | -7~-2 | -3 |  | -2 or -5 |  |  |  | -4.7 | -5 |  | -3.2 |

Note: the SNR value may be updated based on new simulation results. RAN1 waveform design is not finalized as of this WF.

Agreements:

  - Further discuss next meeting

Issue 2-2-6: Baseline OFDM-based architecture for FR2 LP-WUR

Agreements:

  - FR2 LPWUR adopts zero-IF topology baseline assumption. FFS details

    - Multiple frequency conversion in RF domain is not precluded

  - For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features:

    - It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized

    - FFS on whether and how to combine two Rx chains

    - Antenna gain is that of a single element

    - Coverage target should be considered

Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-1: Required number of guard RB for ASCS

Agreements:

  - [0RB] as guard RB for ASCS.

Issue 2-3-2: Detailed Test case for ASCS

WF:

  - RAN4 further discuss the test case next meeting

Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-1: Phase noise for simulation assumption

Agreements:

  - Companies can use different phase noise assumptions, the phase noise assumption should be reported

Issue 2-4-2: General ACS requirements value and guard RB

Agreements:

  - For ACS evaluation, companies are encouraged to provide the simulation results by assuming

    - Baseline: the same interference experienced by LP-WUR and main radio.

    - Optional: the different interference level for LP-WUR and main radio

Issue 2-4-4: Test parameters for LP-WUR ACS case

WF:

  - RAN4 further discuss detailed test parameters table and how to modify spec.

Topic #3: Other RF requirements

Sub-topic 3-1 IBB, OBB and Intermodulation requirements for UE RF

Issue 3-1-1: IBB, OBB requirements

- Proposals

  - Option 1: the interference levels for IBB could be relaxed compared to the values defined for MR (at least for the LP-WUR type which targets ultra-low power consumption). OBB could be kept unchanged at least for LP-WUR type which targets moderate power consumption, FFS for ultra-low power consumption LP-WUR.

  - Option 2: In-band blocking requirement for legacy UE could be reused. the OBB requirement for legacy UE in TS 38.101-1 could be reused.

WF:

  - Further discuss IBB and OBB requirements

Issue 3-1-3: detailed table for test parameters of IBB and OBB test case

- Proposals

  - Proposal 1: further discuss detailed test parameters table and how to modify spec, after concluding IBB and OBB requirements value and guard RBs.

WF:

  - FFS

Sub-topic 3-2 Other Rx requirements

Issue 3-2-1: spurious response requirements

- Proposals

  - Option 1: spurious response should be relaxed at least for the LP-WUR type which targets ultra-low power consumption.

  - Option 2: Reuse the spurious response requirements from TS 38.101-1 for LP-WUR.

WF:

  - FFS whether spurious response can be relaxed

Issue 3-2-2: Maximum input level requirements

Agreements:

  - For maximum input power level, further discuss

    - Option 1: use legacy -25dBm

    - Option 2: scaled to -26.5dBm.

    - Option 3: same as MR, FFS test signal

Topic #4: Testability issues

Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing

Agreements:

  - The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested.

Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance

Agreements:

  - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point:

    - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance.

    - FFS if the MR should only be woken up when data transfer the system simulator is needed.

    - Other options are not precluded.