3GPP TSG-RAN WG4 Meeting #112 bis	R4-2416910

Hefei, China , 14th  – 18th  Octorber , 2024

Title: 	Simulation assumption for evaluate L1- SRS-RSRP measurement requirements

Source: 	Ericsson

Agenda item:	6.20.3

Document for:	Approval

# Introduction

This paper provides the link level simulation assumption for L1-SRS-RSRP measurement for Rel-19 NR evolution of NR duplex operation.

# Discussion on simulation assumption

The intention of the simulation is to investigate the measurement performance of the L1-SRS-RSRP for NR evolution of NR duplex operation.

## L1- SRS-RSRP measurement

Table 2-1 Simulation assumptions for L1-SRS-RSRP measurement

| Parameters |  | Value |  |
| --- | --- | --- | --- |
|  |  | UE0 [victim UE] | UE1 [aggressor UE] |
| Carrier frequency |  | 4GHz for FR1 30GHz for FR2 |  |
| Subcarrier spacing |  | 15kHz, 30kHz, 60kHz for FR1 60kHz, 120kHz for FR2 |  |
| SRS configuration | Number of port | N/A | 1 |
|  | TransmissionComb |  | 2 |
|  | combOffset |  | 0 |
|  | Cyclic shift |  | 0 |
|  | StartPosition (SRS symbol start position) |  | 0 |
|  | Number of symbols |  | 1, [2,3] Note 1 |
|  | Number of RBs |  | [24],48 |
|  | Frequency domain position and shift |  | 0 |
|  | Periodicity [slot] |  | 5 |
| Residual timing error (time offset between measurement timing at UE0 and arrival timing of SRR from UE 1) Note 2 |  | - Case 1: 4.67us for FR1, 3.67us for FR2 - Case 2: 1.67us for FR1, 0.67us for FR2 |  |
| CP Length |  | Normal |  |
| Number of measurement samples |  | 1, [2,3] | N/A |
| Measurement bandwidth [RB] |  | [24],48 | N/A |
| Measurement interval Note 3 |  | 20ms | N/A |
| Number of UE receive antennas |  | 2 Rx | N/A |
| Propagation conditions |  | AWGN, [TDL-A10, TDL-C100] for FR1, AWGN, [TDL-A75, TDL-C300] for FR2 |  |
| SRS SNR |  | 1, [2,3] dB | N/A |
| Note 1: The number within [] are optional for 1st simulation results Note 2: The residual timing error case 1 is the Rel-16 with cell phase error, case 2 is remove the cell phase error. Note 3: The measurement interval only applicable for multiple number of samples being configured Note 4: No frequency hopping is assumed. Note 5: Allocated RBs are within active BWP for measurement UE |  |  |  |

## Performance metric for L1-SRS-RSRP

The following performance metrics can be considered:

- The CDF curves (including the 95% and 5% CDF points) for delta L1-SRS-RSRP.

Delta L1-SRS-RSRP is defined as follows:

- Delta L1-SRS-RSRP = Measured L1-SRS-RSRP – Ideal L1-SRS-RSRP.

Ideal L1-SRS-RSRP is defined as the true value assuming perfect channel estimation using the same sampling rate and sampling occasions as used in the measured L1-SRS-RSRP evaluation.

# Reference

- R4-1907392, Simulation assumption for CLI measurement requirements, LG Electronics