3GPP TSG RAN WG4 Meeting #112bis		R4-2417119

Hefei, China, 14th – 18th October 2024

Title:	LS on Rel-19 NR channel BW less than 5MHz for FR1

Response to:

Release:	Rel-19

Work Item:	NR_FR1_lessthan_5MHz_BW_Ph2

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Zhang, Meng

meng.zhang@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 has had extensive discussions on the UE RF requirements for Rel-19 less than 5MHz UE configured with CA and DC band combinations. RAN4 reached consensus on the associated UE capability signalling aspects as provided below:

- UE needs to be able to indicate for each carrier in the CA/DC band combination whether it supports less than 5 MHz channel bandwidth operation for DL/UL - To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination UE needs to indicate UE capabilities defined for NR < 5MHz channel bandwidth in Rel-18 and the existing UE capability applicability restriction to single carrier needs to be removed. - UE also indicates BCS including support of less than 5MHz channel bandwidth - For BCS signalling the following framework is used: - Traditional BCSs (i.e., not BCS4 or BCS5): UE is expected to support 3MHz channel bandwidth in the band in the band combination if it indicates a corresponding BCS, which includes 3MHz channel bandwidth in RAN4 specifications. No signalling changes are expected by RAN4. - BCS4: if UE indicates support for BCS4 including a band that it also supports 3 MHz channel bandwidth for, then it shall support 3 MHz for CA for that band in the band combination. No signalling changes are expected by RAN4. - BCS5: if UE indicates support for BCS5 and minimum channel bandwidth of 3MHz as a part of BCS signalling, then it shall support 3 MHz for CA for that band in the combination. The existing BCS5 signalling framework needs to be modified to allow indication of 3 MHz as minimum channel bandwidth. - It should be possible for UE to indicate support of CA/DC with less than 5MHz channel bandwidth starting from Rel-18 (i.e., allow early implementation from Rel-18) - RAN4 has not identified a need for new signaling element. The signalling details are up to RAN2.

The specific UE feature groups for NR less than 5 MHz channel bandwidth, which have single carrier restriction are listed below:

- support12PRB-CORESET0-r18

- support3MHz-ChannelBW-Asymmetric-r18

- support3MHz-ChannelBW-Symmetric-r18

- support5MHz-ChannelBW-20PRB-CORESET0-r18

- support12PRB-CORESET0-GSCN-41637-r18

# 2	Actions

To RAN2

ACTION: 	RAN4 kindly asks RAN2 to take into consideration RAN4 agreements and introduce necessary UE capability signalling update to enable NR CA/DC operation with less than 5 MHz channel bandwidth.

# 3	Dates of next TSG RAN WG4 meetings

3GPP RAN WG4 #113		18th – 22nd November 2024		Orlando, USA

3GPP RAN WG4 #114		17th – 21st February 2025			Athens, Greece